3GPP TSG-WG SA2 Meeting #153E	S2-2208429

# E-meeting, October 10 - 17, 2022	(revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0745 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Non-3GPP access for SNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The following was concluded on KI#2 (Support of Non-3GPP access for SNPN) in TR 23.700-08: - Access to SNPN services via Untrusted non-3GPP access network includes support for Credential Holder, UE Onboarding and Emergency services. Specifically: - When UE registers to SNPN with credentials owned by the SNPN, UE uses the same N3IWF selection procedure as specified for access to stand-alone non-public network services via PLMN in clause 6.3.6.2a of TS 23.501 [3]. - UE initiates N3IWF selection for emergency services when the UE detects a user request for emergency session and determines that Untrusted non-3GPP access is to be used for the emergency access. The UE with SNPN subscription performs the following: - If the UE determines that it is located in the same country as the subscribed SNPN, the UE uses the configured N3IWF FQDN for N3IWF selection. - Otherwise, the UE follows the N3IWF selection procedure for Emergency services for UE not equipped with UICC, as defined in clause 6.3.6.4.2 of TS 23.501 [3]. - When UE registering to SNPN over Untrusted N3GPP access for UE Onboarding, UE may select a N3IWF in the SNPN which supports UE Onboarding by using an SNPN ID in the pre-configured ON-SNPN selection information to construct N3IWF FQDN. NOTE 1: The format of FQDN will be specified by CT WG4. - If the PVS is reachable from the local Untrusted non-3GPP access network (e.g. via the Internet) using the local IP connectivity, UE may connect directly (i.e. without connected to a N3WIF) with a PVS to obtain the SNPN credentials. - Access to SNPN services via Trusted non-3GPP access network is to be specified according to the principles described in Solution #3, including support for credential Holder, Emergency services and UE onboarding. Specifically: - ANQP is to be extended to support advertising of: - SNPN IDs and GINs corresponding to SNPNs with which 5G connectivity is supported and related parameters as described in clause 6.3.3.1. - Support for Emergency services. - Onboarding enabled indication. NOTE 2: The work on additional parameters in ANQP is to be kept internal to 3GPP. - When accessing SNPN services via Trusted or Untrusted non-3GPP access the UE needs to be able to construct a prioritized list of WLAN access networks by using enhanced WLAN Selection Policy (WLANSP) rules from ANDSP (currently supported only for PLMN in TS 23.402 [9]) or based on local configuration. - N3IWF and TNGF needs to be able to include the "selected NID" in the [NGAP] INITIAL UE MESSAGE, which is up to RAN3 to define. - Access to SNPN services via wireline access network and use of Credential Holder is to be supported by defining a new GCI including a "NID" - The NSWO procedure is to be extended to support UE authentication using SNPN credentials (applies both to SIM-based and non-SIM based credentials). It is expected that the impact is limited to the use of a SUCI format whose "realm" part enables routing of SWa requests from the WLAN AN to the NSWO in the SNPN's 5GC, which is already supported. NOTE 3: Additional conclusions for wireline access related to UE behind 5G-RG depends on the progress on the FS_5WWC_Ph2 study. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding support of Non-3GPP access for SNPN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of Non-3GPP access for SNPN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.1.2; 6.6.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 6.6	UE policy information

### 6.6.1	Access Network Discovery & Selection Policy Information

#### 6.6.1.1	General

The Access Network Discovery & Selection policy is an optional policy that may be provided to UE by the network.

In this release of the specification, the Access Network Discovery & Selection policy shall contain only rules that aid the UE in selecting a WLAN access network. Rules for selecting other types of non-3GPP access networks are not specified.

The WLAN access network selected by the UE with the use of Access Network Discovery & Selection policy may be used for direct traffic offload (i.e. sending traffic to the WLAN outside of a PDU Session) and for registering to 5GC using the non-3GPP access network selection information.

If the UE supports non-3GPP access to 5GC, it shall support ANDSP.

The procedure for WLAN access network selection is defined in clause 6.6.1.3, the procedure for N3IWF selection is defined in clause 6.3.6.1 of TS 23.501 [2].

The Access Network Discovery & Selection policy shall contain one or more WLAN Selection Policy (WLANSP) rules defined in clause 4.8.2.1.6 of TS 23.402 [9].

The Access Network Discovery & Selection policy may contain information to select ePDG or N3IWF by the UE as specified in TS 23.501 [2]

Table 6.6.1-1: Access Network Discovery & Selection Policy

| Information name | Description | Category | PCF permitted to modify in a UE context | Scope |
| --- | --- | --- | --- | --- |
| WLANSP rules | 1 or more WLANSP rules as specified in 4.8.2.1.6 of TS 23.402 [9] | Mandatory | Yes | UE context |
| ePDG identifier configuration | The UE uses this information to select ePDG as defined in clause 6.3.6.1 of TS 23.501 [2] | Optional | Yes | UE context |
| N3IWF identifier configuration | The UE uses this information to select N3IWF as defined in clause 6.3.6.1 of TS 23.501 [2] | Optional | Yes | UE context |
| Non-3GPP access node (N3AN) selection information | The UE uses this information to select ePDG or N3IWF as defined in clause 6.3.6.1 of TS 23.501 [2] | Optional | Yes | UE context |

#### 6.6.1.2	UE selecting a WLANSP rule

The UE, when not operating in SNPN access mode, may be provisioned with multiple valid WLANSP rules (by the HPLMN and by the VPLMN when the UE is roaming). A WLANSP rule is valid if it meets the validity conditions included in the WLANSP rule (if provided).

When the UE is in the home the UE uses the valid WLANSP rules from the home PLMN to select an available WLAN. When the UE is roaming and the UE has valid rules from both HPLMN and VPLMN the UE gives priority to the valid WLANSP rules from the VPLMN.

The SNPN-enabled UE may be provisioned with multiple valid WLANSP rules by the subscribed SNPN or by the Credential Holder to be used when operating in SNPN access mode. A WLANSP rule is valid if it meets the validity conditions included in the WLANSP rule (if provided).

#### 6.6.1.3	UE procedure for selecting a WLAN access based on WLANSP rules

The UE shall apply the procedure in this clause when the UE is provisioned with WLANSP rules and:

a)	when the UE initiates untrusted non-3GPP access to 5GC and attempts to select a WLAN access network; or

b)	when the UE initiates trusted non-3GPP access to 5GC by executing the Access Network Selection Procedure specified in clause 6.3.12.2 of TS 23.501 [2] and attempts to select a WLAN access network.

The procedure in this clause applies to both UE operating in SNPN access mode and UE not operating in SNPN access mode, except where stated otherwise.

When the UE has valid 3GPP subscription credentials (i.e. a valid USIM or valid SNPN credentials) and WLANSP rules, the UE shall perform WLAN selection based on these rules, the applicable User Preferences On Non-3GPP Access Selection and the corresponding procedures specified in this document. User Preferences On Non-3GPP Access Selection take precedence over the WLANSP rules.

The UE determines the most preferred WLAN access network using WLANSP rules when a WLAN access network cannot be selected based on User Preferences On Non-3GPP Access Selection (e.g. when there are no User Preferences On Non-3GPP Access Selection or when there is no user-preferred WLAN access network available).

The UE constructs a prioritized list of the available WLANs by discovering the available WLANs and comparing their attributes / capabilities against the groups of selection criteria in the valid WLANSP rule(s). When there are multiple valid WLANSP rules the UE evaluates the valid WLANSP rules in priority order. The UE evaluates first if an available WLAN access meets the criteria of the highest priority valid WLANSP rule. The UE then evaluates if an available WLAN access meets the selection criteria of the next priority valid WLANSP rule.

Within a valid WLANSP rule, the WLAN(s) that match the group of selection criteria with the highest priority are considered as the most preferred WLANs, the WLAN(s) that match the group of selection criteria with the second highest priority are considered as the second most preferred WLANs, etc.

When a group of selection criteria includes the HomeNetwork attribute and is set, then the UE (a) shall create a list of available WLANs that directly interwork with the home operator (as specified in clause 4.8.2.1.6 of TS 23.402 [9]) and (b) shall apply the group of selection criteria to all the WLANs in this list. Otherwise, when the HomeNetwork attribute is not set or is not present, the UE shall apply the group of selection criteria to all available WLANs. The UE may need to perform ANQP procedures (as specified in the HS2.0 Rel-2 specification [ref]) or other procedures in order to discover the attributes / capabilities of the available WLANs.

When the UE, that is not operating in SNPN access mode, is roaming the UE may have valid WLANSP rules from both the VPLMN and the HPLMN. In such a case the UE gives priority to the valid WLANSP rules from the VPLMN. The UE constructs a prioritised list of the available WLANs when the available WLAN accesses meet the selection criteria of the valid rules from the VPLMN and the valid rules from the HPLMN. The prioritised WLAN accesses based on the WLANSP rules from the HPLMN will have lower priority from the prioritised list of WLAN access based on the WLANSP rules of the VPLMN.

# * * * * End of changes * * * *