SA WG2 Meeting #S2-153E	S2-2208453r02

10 – 17 October 2022, Electronic	(revision of S2-22006359r06 was r05)

Source: 	MediaTek Inc.

Title: 	KI#2 KI#6: MT-LR and MO-LR signalling for peer-to-peer SL Positioning and Ranging

Document for: 	Approval

Agenda item: 	9.20

Work Item / Release:	FS_Ranging_SL / Rel-18

Abstract of the contribution: This contribution is a resubmission of S2-2206359r06 (itself update of r05) submitted to Friday CC#5 at SA2#152e. Though r05 was requested (twice) to be handled in CC#5 during the email discussion, it was omitted in CC#5 and while requested once again to be handled in CC#5, it was unfortunately not treated due to lack of time.

# Introduction

This contribution is a resubmission of S2-2206359r06 (itself update of r05) submitted to Friday CC#5 at SA2#152e with only the following amendments done (vs r06):

-	It only contains the pCR part

-	Terminology has been updated according to the latest 23.700-86 (v1.0.0) – such changes are in blue highlights (except in the figures that remain B&W)

It should be noted that:

-	r05 had been made available on time wrt revision deadline, and requested (twice) to be handled in CC#5 during the email discussion

-	after the revision deadline, offline discussions were had finding an acceptable compromise in the form of r06 with simple updates as shown in green highlights in r06. r06 was uploaded to the CC folder well on time.

-	unfortunately, r05 was not handled in CC#5 as it did not appear on the list of CC#5 docs and though requested once more during the call, CC#5 did run out of time.

We do acknowledge this could be seen as a new solution, however, this paper is not something that has not been seen or discussed before, so we hope it could be treated esp. in view of the above and earlier discussions.

We also would like to note some of the aspects raised by this paper have also been raised in the pre-SA2#153e CC – it is important that a sound signaling framework be defined that will help integrating SL Positioning and Ranging with LCS.

# pCR 23.700-86

The proposals in §2 are incorporated in the following pCR.

**** FIRST CHANGE ****

## 6.0	Mapping of solutions to key issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | X |  |  |  |  |  |  | X |
| 2 | X |  |  |  |  |  |  |  |
| 3 |  |  |  | X |  |  |  |  |
| 4 |  |  | X | X |  |  |  |  |
| 5 |  |  | X | X |  |  |  |  |
| 6 |  |  |  |  | X |  |  |  |
| 7 |  |  |  |  | X |  |  |  |
| 8 |  |  |  |  | X |  |  |  |
| 9 |  |  |  |  |  | X |  |  |
| 10 |  |  |  |  |  | X |  |  |
| 11 |  |  |  |  |  | X |  |  |
| 12 |  |  |  |  |  |  | X |  |
| 13 |  |  |  |  |  |  | X |  |
| 14 | X | X | X | X | X | X | X | X |
| X |  | X |  |  |  | X |  |  |

**** NEXT CHANGE (all new text) ****

## 6.X	Solution #x: MT-LR and MO-LR signalling for UE-only Operation of SL Positioning and Ranging

Editor's note:	This clause describes a solution addressing one or more key issues identified in clause 5. The structure of the clauses can be adjusted. The list of key issues which this solution attempts to resolve should be clearly indicated.

### 6.X.1	General

This solution addresses KI#2 (Ranging service operation procedure with the assistance of another UE), KI#6 (Ranging and sidelink positioning service exposure to a UE).

Solution #26 introduces, among others, the notion of a Location Server that can be hosted in a UE, as not all UEs may be able to position themselves and a network Ranging/SL Positioning capable LMF may simply not be reachable or available.

UE-only Operation further implies:

-	LCS today assumes an LCS Client may operate in a UE; as specified in TS 23.273, an LCS Client is an "entity that interacts with GMLC for the purpose of obtaining location information for one or more UEs. The LCS Client may reside in the UE." In UE-Only Operation this model needs to be adapted, i.e. for operation in a third-party UE. The term "SL Positioning Client UE" is used hereafter.

-	GMLC, UDM and AMF are not available, while noting LCS Clients traditionally access LCS services from a GMLC using the Le reference point. These NFs provide important functionality that need to be transposed into UE-only scenarios.

Additionally, one or multiple SL Reference UEs need to be involved to position a target UE using PC5.

This solution proposes to introduce a signalling framework to enable PC5-MT-LR and PC5-MO-LR LCS procedures supporting sidelink positioning and ranging, given the above.

### 6.X.2	Functional descriptions

In a network scenario, it is the GMLC (with assistance from the UDM) that authorizes LCS Clients/AF and ensures the privacy of a Target UE is respected. In UE-only scenario, without any network entity to rely on, it should be considered whether a UE could host Vetting functionality. A key question is then whether a UE has authority to verify a) that SL Positioning Client UE is authorized and b) the Target UE privacy with respect to the -SL Positioning Client UE obtaining the location of the Target UE.

This solution proposes that no distinct GMLC/UDM UE be introduced. Instead, it proposes that the necessary Vetting functionality be hosted either in the Target UE (Case I.) or the SL Positioning Server UE (Case II.) as follows:

I.	Target UE (with Vetting functionality): the Target UE interacts directly with SL Positioning Client UEs. The Target UE itself vets any incoming location requests addressed to it and only responds to requests it successfully vets, such that "illicit" requests do not cause undue signalling, given that:

-	In network-based scenarios, an LCS Client is verified to be authorized, or not, to retrieve the UE location, as a function of the Subscriber LCS Privacy Profile (SLPP) that is stored as part of the subscription data in the UDM, and LCS Client Data (e.g. location request type, client identity). The SLPP is subscriber-specific and can be updated by the UE, e.g. depending on user input; and

-	In UE-only scenario, it can therefore be expected that the Target UE itself holds its own Subscriber SL Positioning Privacy Profile, and using this information and SL Positioning Client UE data (such as location request type e.g. positioning/ranging, accuracy, client identity, SL Reference UE ID(s), etc.) is able to decide whether or not to authorize the Client to retrieve its location.

Furthermore, without an AMF, the Target UE will need taking on the "role" of the AMF. In particular, the Target UE itself will select and interact with the SL Positioning Server UE (if the Target UE determines it needs assistance from a SL Positioning Server UE for positioning of the Target UE).

This solution proposes that the selection of the SL Positioning Server UE by the Target UE is performed using direct discovery. Both models A and B discovery can be used as follows (see TS 23.303 for definitions of models A and B):

-	Model A: a SL Positioning Server UE ("announcing UE") announces itself at least as a SL Positioning Server UE. Target UEs ("monitoring UE") monitor such announcements.

-	Model B: a Target UE ("discoverer UE") issues requests including information it wishes to discover a SL Positioning Server UE ("discoveree UE").

Once a suitable SL Positioning Server UE is identified by the Target UE, direct communication between these UEs can thereafter take place to complete the positioning/ranging procedure.

II.	SL Positioning Server UE (with Vetting functionality): the SL Positioning Server UE interacts with SL Positioning Client UEs i.e. it acts as a "proxy" between these SL Positioning Client UEs and Target UEs. Unlike Case I., the Target UEs would not interact directly with SL Positioning Client UEs in this case. However like Case I., it can be expected here as well that the Target UE itself holds its own Subscriber LCS Privacy profile information which can be queried/used when necessary by the SL Positioning Server UE when interacting with SL Positioning Client UEs. The SL Positioning Server UE may be able to store this information.

A SL Positioning Client UE selects a SL Positioning Server UE (with Vetting functionality) in order for the SL Positioning Client UE to be able to issue Location Service Request.

The selection of the SL Positioning Server UE by the SL Positioning Client UE is performed using direct discovery. Both models A and B discovery can be used as follows (see TS 23.303), noting Model B discovery may be more adequate:

-	Model A: a SL Positioning Server UE ("announcing UE") announces itself at least as a SL Positioning Server UE (with Vetting functionality). Client UEs ("monitoring UE") monitor such announcements.

-	Model B: a SL Positioning Client UE ("discoverer UE") issues requests including information it wishes to discover a SL Positioning Server UE (with Vetting functionality) ("discoveree UE").

The SL Positioning Client UE needs to select a SL Positioning Server UE which is able to discover the Target UE/SL Reference UE.

Once a suitable SL Positioning Server UE is identified by the SL Positioning Client UE, direct communication between these UEs can thereafter take place to perform the necessary positioning/ranging procedure.

A SL Positioning Server UE (with Vetting functionality) uses Subscriber SL Positioning Privacy Profile information from a Target UE, and SL Positioning Client UE data (such as location request type e.g. positioning/ranging, accuracy, identity, etc.) to determine whether a SL Positioning Client UE is authorized and whether this SL Positioning Client UE is authorized to retrieve the location information of the Target UE.

Resulting from the above, a signalling framework for PC5-MO-LR and PC5-MT-LR procedures in UE-only scenario are proposed in clause 6.X.3 below for both Cases I and II described above. No NI-LR procedure is deemed applicable in UE-only scenario.

All signalling transactions depicted in clause 6.X.3 between Target UE, SL Positioning Server UE, SL Reference UE operate using the RSPP protocol (see e.g. Solutions #4, #26).

Editor’s Note: 	Future coordination with RAN2 is expected to ensure alignment of the procedure details over the RSPP protocol.

Editor’s Note:	The protocol used for signalling transactions with the SL Positioning Client UE (i.e. Service Request/Response, Location Info/Info Ack) is FFS.

Editor’s Note: 	It is FFS if the MT-LR and MO-LR model described hereafter would be further optimized for UE-only operation.

Editor’s Note: It is FFS which part of the "GMLC/UDM-like" functionalities need to be standardized for SL Positioning Server UE

Editor’s Note: Discovery Models A and B are referred to in clause 6.X.3. It is FFS how Model A, Model B discovery over PC5 will be used to discover UE “roles” (e.g. Target UE, SL Positioning Server UE). Whether another channel (e.g. communication channel) is needed for discovery is FFS and should be discussed in RAN groups.

Editor’s Note: Details of discovery will be addressed at a future meeting in alignment with KI#3 solutions

Editor’s Note: Any Authentication/Authorization will be aligned with SA3. Whether it refers to CN-related information is FFS

### 6.X.3	Procedures

#### 6.X.3.1 Case I: Target UE holds Vetting functionality

##### 6.X.3.1.1	PC5-MT-LR procedure

Figure 6.X.3.1.1-1: PC5-MT-LR Procedure

- An application in a Third Party UE needs to retrieve the location information of a Target UE. It initiates discovery to discover the Target UE. The  SL Positioning Client UE issues a Service Request message over PC5 to a Target UE in order to retrieve the location of the Target UE. The Service Request message contains necessary information allowing the Target UE to identify the SL Positioning Client UE data incl. the identity, the type of location request (e.g. positioning/ranging), the accuracy of the location, etc. If the type of location request requires SL positioning or ranging, one or multiple SL Reference UE may also be identified in the Service Request.

NOTE X1: The service request may be transmitted from the SL Positioning Client UE to the Target UE through the SL Reference UE, for the case, e.g. the SL Positioning Client UE can only discover the SL reference UE.

2.	Upon reception of a Service Request message, the Target UE verifies that the originating SL Positioning Client UE is authorized. The Target UE uses its own stored Subscriber SL Positioning  Privacy Profile information and the identified SL Positioning Client UE data in Step 1. to determine whether or not to authorize the SL Positioning Client UE and its location request. If another UE was identified in Step 1, the Target UE verifies that this other UE is in proximity (e.g. by using the discovery procedure (Model A) with this other UE) and that this other UE authorizes the positioning/ranging (see Editor’s Note). If the Target UE determines that the SL Positioning Client and its location request are not authorized, and/or that a SL Reference UE included in the location request is not authorized for the positioning/ranging, the Target UE does not respond to the Service Request and the procedure stops, otherwise the Target UE proceeds with Step 3.

Editor’s Note: 	The signalling transaction for the Target UE to verify that a SL Reference UE is authorized for the positioning/ranging is FFS.

3.	If the Target UE determines it requires support from a SL Positioning Server UE, the Target UE selects a SL Positioning Server UE. The selection of the SL Positioning Server UE by the Target UE is performed using direct discovery. Both models A and B discovery can be used as follows (see TS 23.303 for definitions of models A and B):

-	Model A: a SL Positioning Server UE ("announcing UE") announces itself at least as a SL Positioning Server UE. Target UEs ("monitoring UE") monitor such announcements.

-	Model B: a Target UE ("discoverer UE") issues requests including information it wishes to discover a SL Positioning Server UE ("discoveree UE").

For the selection of the SL Positioning Server UE, the Target UE needs to verify that the candidate SL Positioning Server UE is allowed to be involved based on the Subscriber SL Positioning Privacy Profile information.

Once a SL Positioning Server UE is selected, direct communication is established between the Target UE and this SL Positioning Server UE allowing the procedure to proceed (Step 4).

If the Target UE determines instead it does not need support from a SL Positioning Server UE it proceeds to Step 5.

4.	The Target UE issues a Determine Location Request message to the SL Positioning Server UE, incl. necessary information as required by the SL Positioning Client UE in Step 1. If SL positioning or ranging was requested in Step 1 that includes one or more SL Reference UEs, the SL Reference UE(s) are also identified in the Determine Location Request message.

NOTE X2: Step 4 may be skipped if the Target UE does not determine to involve SL Positioning Server UE.

5.	The UE Positioning (/ranging) is performed by the Target UE or with support of the SL Positioning Server UE in order to determine the Target UE location, depending on the outcome of Step 3.

NOTE X3:	SL Reference UE(s) (e.g. the other UE) may also be invoked at this point to assist the Target UE or SL Positioning Server UE in determining the Target UE location.

If support from a SL Positioning Server UE was requested in Step 3, Step 6 follows, else Step 7.

6.	The SL Positioning Server UE reports the calculated Target UE location information in a Determine Location Response message to the Target UE.

NOTE X4: Step 6 may be skipped if the Target UE does not determine to involve SL Positioning Server UE.

7.	The Target UE reports its location information to the SL Positioning Client UE in a Service Response message.

##### 6.X.3.1.2	PC5-MO-LR procedure

Figure 6.X.3.1.2-1: PC5-MO-LR procedure

1.	This step is triggered in the Target UE e.g. by an application running in the UE, to retrieve its position (absolute or relative) and, possibly, to share it with a SL Positioning Client UE (e.g. at regular intervals). If the location operation requires positioning or ranging vs. another UE, the Target UE verifies that this other UE is in proximity (e.g. by using the discovery procedure (Model A) with this other UE) and that the other UE authorizes the location operation (see Editor’s Note). If the Target UE determines it requires support from a SL Positioning Server UE, the Target UE selects a SL Positioning Server UE. The selection of the SL Positioning Server UE by the Target UE is performed using direct discovery. Both models A and B discovery can be used as follows (see TS 23.303 for definitions of models A and B):

-	Model A: a SL Positioning Server UE ("announcing UE") announces itself at least as a SL Positioning Server UE. Target UEs ("monitoring UE") monitor such announcements.

-	Model B: a Target UE ("discoverer UE") issues requests including information it wishes to discover a SL Positioning Server UE ("discoveree UE").

For the selection of the SL Positioning Server UE, the Target UE needs to verify that the candidate SL Positioning Server UE is allowed to be involved based on the Subscriber SL Positioning Privacy Profile information.

Once a SL Positioning Server UE is selected, direct communication is established between the Target UE and this SL Positioning Server UE allowing the procedure to proceed (Step 2).

If the Target UE determines instead it does not need support from a SL Positioning Server UE it proceeds to Step 3, and a direct communication is not established between the Target UE and the SL Positioning Server UE.

Editor’s Note: 	The signalling transaction for the Target UE to verify that the other UE authorizes the positioning/ranging is FFS.

- The Target UE issues a Determine Location Request message to the SL Positioning Server UE, incl. necessary information as required by the intended location operation.

NOTE Y1: Step 2 may be skipped if the Target UE does not determine to involve SL Positioning Server UE.

3.	The UE Positioning (/ranging) is performed by the Target UE or with support of the SL Positioning Server UE in order to determine the Target UE location, depending on the outcome of Step 1.

NOTE Y2:	SL Reference UE(s) (e.g. the other UE) may also be invoked at this point to assist the Target UE or SL Positioning Server UE in determining the Target UE location.

If support from a SL Positioning Server UE was requested in Step 1, Step 4 follows, else Step 5.

4.	The SL Positioning Server UE reports the calculated Target UE location information in a Determine Location Response message to the Target UE.

NOTE Y3: Step 4 may be skipped if the Target UE does not determine to involve SL Positioning Server UE.

5.	If necessary (i.e. if reporting to a SL Positioning Client UE is necessary), the Target UE reports the calculated Target UE location information to an authorized SL Positioning Client UE.

6.	The SL Positioning Client UE acknowledges receipt of the Target UE location information from the Target UE.

#### 6.X.3.2 Case II: SL Positioning Server UE holds Vetting functionality

##### 6.X.3.2.1	PC5-MT-LR procedure

Figure 6.X.3.2.1-1: PC5-MT-LR procedure

1.	An  SL Positioning Client UE needs to retrieve location information of a Target UE. It initiates discovery to discover the Target UE. The SL Positioning Client UE selects a SL Positioning Server UE (with Vetting functionality). The selection of the SL Positioning Server UE by the SL Positioning Client UE is performed using direct discovery. Both models A and B discovery can be used as follows (see TS 23.303 for definitions of models A and B):

-	Model A: a SL Positioning Server UE ("announcing UE") announces itself at least as a SL Positioning Server UE (with Vetting functionality). SL Positioning Client UEs ("monitoring UE") monitor such announcements.

-	Model B: a Third Party UE ("discoverer UE") issues requests including information it wishes to discover a SL Positioning Server UE (with Vetting functionality) ("discoveree UE").

The SL Positioning Client UE needs to select a SL Positioning Server UE which is able to discover the Target UE/SL Reference UE.

Once a suitable SL Positioning Server UE is identified by the SL Positioning Client UE, direct communication between these UEs can thereafter take place to complete the procedure.

2.	The  SL Positioning Client UE issues a Service Request message over PC5 to the SL Positioning Server UE in order to retrieve the location of the Target UE. The Service Request message contains necessary information allowing the SL Positioning Server UE to identify the SL Positioning Client UE data incl. the identity, the type of location request (e.g. positioning/ranging), the accuracy of the location, etc. as well as the Target UE. If the type of location request requires SL positioning or ranging, one or more SL Reference UEs may also be identified in the Service Request.

If the SL Positioning Server UE holds Subscriber LCS Privacy Profile information of the Target UE it proceeds to Step 5, otherwise Step 3.

NOTE:	If a SL Reference UE is identified in the Service Request, the SL Positioning Server UE proceeds the same way with verifying the SLPP information of the SL Reference UE.

3.	After discovering the Target UE and establishing direct communication with the Target UE, the SL Positioning Server UE issues an Subscriber SL Positioning Privacy Profile Information Request message to the Target UE to retrieve its Subscriber SL Positioning Privacy Profile Information.

4.	The Target UE upon receiving the Subscriber SL Positioning Privacy Profile Information Request message from the SL Positioning Server UE responds with an Subscriber SL Positioning Privacy Profile Information Response message including the Target UE’s Subscriber SL Positioning Privacy Profile information.

5.	The SL Positioning Server UE verifies that the SL Positioning Client UE is authorized. The SL Positioning Server UE uses the Subscriber SL Positioning Privacy Profile information of the Target UE (and, of the  SL Reference UE(s) as applicable) and the identified SL Positioning Client UE data in Step 2 to determine whether or not to authorize the SL Positioning Client UE and its location request. If the SL Positioning Client UE (and associated location request) is not authorized the SL Positioning Server UE may reject the Service Request from the SL Positioning Client UE. If the SL Positioning Client UE is authorized, the SL Positioning Server UE proceeds with Step 6.

6.	The SL Positioning Server UE forwards the authorized Service Request to the Target UE. If the Target UE determines it needs support from the SL Positioning Server UE to determine its location, it proceeds to Step 7, otherwise Step 8.

7.	The Target UE issues a Determine Location Request message to the SL Positioning Server UE, incl. necessary information as required by the Client in Step 1. If SL positioning or ranging was requested in Step 1 (resp. Step 6) that includes one or more SL Reference UEs, then th SL Reference UE(s) are also identified in the Determine Location Request message.

NOTE Z1: Step 7 may be skipped if the Target UE does not determine to involve SL Positioning Server UE.

8.	The UE Positioning (/ranging) is performed by the Target UE or with support of the SL Positioning Server UE in order to determine the Target UE location, depending on the outcome of Step 6.

NOTE Z2:	SL Reference UE(s) may also be invoked at this point to assist the Target UE or SL Positioning Server UE in determining the Target UE location.

If support from a SL Positioning Server UE was requested in Step 6, Step 9 follows, else Step 10.

9.	The SL Positioning Server UE reports the calculated Target UE location information in a Determine Location Response message to the Target UE.

NOTE Z3: Step 9 may be skipped if the Target UE does not determine to involve SL Positioning Server UE.

10.	The Target UE reports its location information in a Service Response message to the SL Positioning Server UE which in turn forwards it to the SL Positioning Client UE.

##### 6.X.3.2.2	PC5-MO-LR procedure

Figure 6.X.3.2.2-1: PC5-MO-LR procedure

1.	This step is triggered in the Target UE e.g. by an application running in the UE, to retrieve its position (absolute or relative) and, possibly, to share it with a SL Positioning Client UE (e.g. at regular intervals). If the location operation requires SL positioning or ranging with a SL Reference UE, the Target UE verifies that this SL Reference UE is in proximity (e.g. by using the discovery procedure (Model A) with this other UE). If the Target UE determines it requires support from a SL Positioning Server UE, the Target UE selects a SL Positioning Server UE. The selection of the SL Positioning Server UE by the Target UE is performed using direct discovery. Both models A and B discovery can be used as follows (see TS 23.303 for definitions of models A and B):

-	Model A: a SL Positioning Server UE ("announcing UE") announces itself at least as a SL Positioning Server UE. Target UEs ("monitoring UE") monitor such announcements.

-	Model B: a Target UE ("discoverer UE") issues requests including information it wishes to discover a SL Positioning Server UE ("discoveree UE").

For the selection of the SL Positioning Server UE, the Target UE needs to verify that the candidate SL Positioning Server UE is allowed to be involved based on the Subscriber SL Positioning Privacy Profile information.

Once a SL Positioning Server UE is selected, direct communication is established between the Target UE and this SL Positioning Server UE allowing the procedure to proceed (Step 2).

If the Target UE determines instead it does not need support from a SL Positioning Server UE it proceeds to Step 3.

2.	The Target UE issues a Determine Location Request message to the SL Positioning Server UE, incl. necessary information as required by the intended location operation. If the type of location request requires SL positioning or ranging with one or more SL Reference UEs, the SL Reference UE(s) are also identified in the Determine Location Request message. The SL Positioning Server UE verifies that the location operation with the other UE is authorized (following if necessary similar steps as Steps 3, 4 and 5 in clause 6.X.3.2.1 i.e. Subscriber SL Positioning Privacy Info Req/Resp with, and Vetting of the SL Reference UE).

NOTE A1: Step 2 may be skipped if the Target UE does not determine to involve SL Positioning Server UE.

3.	The UE Positioning (/ranging) is performed by the Target UE or with support of the SL Positioning Server UE in order to determine the Target UE location, depending on the outcome of Step 1.

NOTE:	SL Reference UE(s) may also be invoked at this point to assist the Target UE or SL Positioning Server UE in determining the Target UE location.

If support from a SL Positioning Server UE was requested in Step 1, Step 4 follows, else Step 5.

4.	The SL Positioning Server UE reports the calculated Target UE location information in a Determine Location Response message to the Target UE.

NOTE A2: Step 2 may be skipped if the Target UE does not determine to involve SL Positioning Server UE.

5.	If necessary (i.e. if reporting to a SL Positioning Client UE is necessary), the Target UE reports the calculated Target UE location information to the SL Positioning Server UE, with necessary information allowing identification of the SL Positioning Client UE. The Target UE includes its SLPP information, which, together with the Client information allows the SL Positioning Server UE to determine whether the SL Positioning Client UE is authorized. If so, the SL Positioning Server UE forwards the location information to the authorized SL Positioning Client UE.

6.	The SL Positioning Client UE acknowledges receipt of the Target UE location information from the SL Positioning Server UE which in turn forwards it to the Target UE.

### 6.X.4	Impacts on services, entities, and interfaces

UE: Support of PC5-MO-LR and PC5-MT-LR procedures for Case I and/or Case II incl.

-	Support of SL Positioning Server UE selection by Target UE or SL Positioning Client UE

-	Support of SL Positioning Client UE authorization incl. support storage of Subscriber LCS Privacy Profile information and identification of SL Positioning Client UE data by Target UE or SL Positioning Server UE

-	(SL Positioning Client UE) Support of location requests/responses pertaining to SL positioning/ranging

**** END OF CHANGES ****