3GPP TSG-SA WG2#153E	S2-2208490r01

Elbonia, 10 - 17 October 2022	(was S2-22xxxxx)

Source:	CATT, Nokia, Nokia Shanghai Bell

Title:	KI#5: Remove EN in KI#5 Conclusions

Document for:	Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2/Rel-18

Abstract of the contribution: This contribution proposes to remove the EN about Rel-18 ATSSS dependency in KI#5 Conclusions.

# Introduction

The support of redundant traffic steering was studied and concluded in Rel-18 FS_ATSSS_Ph3 study item, and it can be applied to MA PDU Session for multi-path transmission over direct Uu and Layer-3 U2N Relay with N3IWF.

So the EN in KI#5 conclusions can be removed and a NOTE can be added to clarify the dependency.

# Proposal

It is proposed to capture the following texts into TR 23.700-33.

>>>>First Change<<<<

## 8.5	Key Issue #5: Support of multi-path transmission for UE-to-Network Relay

For Key Issue #5 "Support of multi-path transmission for UE-to-Network Relay", the followings are taken as conclusions:

-	Policy authorization for multi-path transmission via direct Uu path and Layer 3 U2N Relay UE without N3IWF is needed, detailed URSP enhancement design and potential Registration procedure enhancement (e.g. in Sol #25) will be determined in normative phase.

NOTE 1:	Whether and how to authorize the multi-path transmission based on ProSe policy will be determined in normative phase.

Editor's note: Whether policy authorization for multi-path transmission via direct Uu path and Layer 3 U2N Relay UE with N3IWF is needed is FFS.

-	Multi-path transmission via direct Uu path and Layer 3 U2N Relay UE with N3IWF can rely on MA-PDU Session (introduced in clause 5.32 of TS 23.501 [7]) as described in Sol #28 and Sol #41, no system enhancement is required; in normative phase, how to use the MA PDU Session to support multi-path transmission via direct Uu path and Layer 3 U2N Relay UE with N3IWF will be described for information (e.g. as described in Sol #28 and Sol #41).

NOTE 2:	The conclusions of support of redundant traffic steering in Rel-18 FS_ATSSS_Ph3 study item can be applied to MA PDU Session for multi-path transmission over direct Uu and Layer-3 UE-to-Network Relay with N3IWF.

-	For multi-path transmission via direct Uu path and Layer 2 U2N Relay UE:

-	Multi-path transmission via direct Uu path and Layer 2 U2N Relay will be defined by RAN WGs and alignment work can be made by SA2 based on the progress and outcome in RAN WGs.

>>>>End Changes<<<<