SA WG2 Meeting #153e	S2-2208533r05

10 – 17 October 2022, e-Meeting	(revision of S2-220xxxx)

Source:	Qualcomm Incorporated

Title:	KI#1: Conclusions

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_Ph3 / Rel-18

Abstract of the contribution: Proposes conclusions for KI #1.

# 1. Introduction

Conclusions for KI #1 in TR 23.700-71 for KI #1 are provided here.

# 2. Text Proposal

The following text is proposed to be applied to TR 23.700-71.

*** First Change (all new) ***

# 8.1	Key Issue #1: Architectural Enhancement to support User Plane positioning

To support User Plane positioning, the conclusion includes the following alternatives which to be deployed depends on operator decision:

-	Alternative #1: lightweight protocol based solution

-	The UE uses the URSP which includes user plane positioning related PDU session parameters (e.g., DNN and S-NSSAI) to establish the PDU session for user plane positioning.

-	It is LMF to decide whether to use user plane or control plane positioning mode when receiving positioning requests from AMF.

-	If LMF decides to utilize user plane positioning, the LMF sends its UP positioning address and security related information to UE to trigger the UP connection if it is not available.

-	LMF and UE may maintain the established user plane connection between UE and LMF for subsequent LCS session.

-	The user plane connections can be supported using protocols decided by stage 3 and SA3.

-	LMF connects with UPF via N6 interface.

-	As per both TS 38.305 from RAN and TS 23.271 for GERAN/UTRAN/E-UTRAN, the continuous User Plane work aligned with both RAN and 2G/3G/4G should use OMA SUPL compliant solutions.

The UP connection between UE and LMF supports LPP message transfer and Supplementary service.

-	Alternative #2: OMA SUPL compliant solution

-	As per both TS 38.305 from RAN and TS 23.271 for GERAN/UTRAN/E-UTRAN, the continuous User Plane work aligned with both RAN and 2G/3G/4G should use OMA SUPL compliant solutions.

-    It is LMF to decide whether to use user plane or control plane positioning mode when receiving positioning requests from AMF.

-	A user plane connection may be used between a UE and AF or LCS Client as defined in Solution #19 for a periodic or triggered MT-LR for the case where UE based location is used and where the UE determines location estimates. The AF or LCS Client address and security related information is then provided to the UE in the MT-LR request. The UE establishes the user plane connetion to the AF or LCS Client and uses it to send event reports to the AF or LCS Client.

*** End of Changes ***