3GPP TSG-SA2 Meeting #153E	S2-2208545

# October 10 – October 14, 2022										         (revision of S2-2206034)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3682 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE-Slice-MBR exemption for priority services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | UE-Slice-MBR need to be exempted for priority services |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Subscribed UE-Slice-MBR value of the UE that has priority subscriptions (i.e., MPS or MCX) shall be set to ensure that QoS Flows of priority users are not restricted or rejected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | QoS Flows of MPS users or MCX users can be rejected due to UE-Slice-MBR |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * 1st Change * * *

5.15.13	Support of data rate limitation per Network Slice for a UE

A UE subscription information may include an optional Slice Maximum Bit Rate for the UE (Subscribed UE-Slice-MBR) for an S-NSSAI, which applies for 3GPP access type only. The Subscribed UE-Slice-MBR includes a UL and a DL value. If a Subscribed UE-Slice-MBR is associated to an S-NSSAI in the subscription information, it is provided by the AMF to the RAN when the AMF provides the Allowed NSSAI for the UE to the RAN as UE-Slice-MBR QoS parameter. The UE-Slice-MBR QoS parameter is defined in clause 5.7.2.6. If the Subscribed UE-Slice-MBR for a UE changes, the AMF updates UE-Slice-MBR in the RAN accordingly.

NOTE: In order to prevent that a QoS flow of  a UE that has priority subscription (e.g., MPS priority, MCX priority) is rejected due to the enforcement of UE-Slice-MBR, it is recommended not to set  Subscribed UE-Slice-MBR for the S-NSSAIs in the UE Subscription. In case the priority subscription (e.g., MPS priority, MCX priority) has updated to be valid, it is recommended that the Subscrbibed UE-Slice-MBR for the S-NSSAIs is set to a value that ensures that QoS Flows of the UE are not restricted or rejected.

In roaming case, the UE-Slice-MBR is provided for the S-NSSAI of the VPLMN which maps to the S-NSSAI of the HPLMN and the AMF may first interact with the PCF for authorization of the Subscribed UE-Slice-MBR. If the AMF interacts with the PCF, the PCF may provide the Authorized UE-Slice-MBR that is used as UE-Slice-MBR by the AMF as described in clause 6.1.2.1 of TS 23.503 [45].

NOTE 1:	For a roaming UE, the S-NSSAI of the VPLMN provided with UE-Slice-MBR should map to only one S-NSSAI of the HPLMN, to ensure the accuracy of performing data rate limitation for the corresponding slice of HPLMN.

The enforcement of the UE-Slice-MBR value, if present in the UE context in the RAN for an S-NSSAI, is described in clause 5.7.1.10.

NOTE 2:	The PCF for the PDU Session may in addition be configured to monitor the data rate per Network Slice for a UE and to strengthen or relax the traffic restrictions for individual PDU Sessions or PCC rules accordingly, as described in TS 23.503 [45] clause 6.2.1.9.

* * *End of Changes * * *