3GPP SA WG2 Meeting #153E	S2-2208550

e-meeting, October 10 – 17, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#6 Conclusion updates

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: This paper proposes a conclusion update for KI#6 (to resolve open items) in TR 23.700-60.

# Discussion

Proposes a simple way forward to resolve the EN related to Round trip latency by leveraging already existing mechanism for monitoring QoS Flows.

# Proposal

It is proposed to add the following solution to TR 23.700-60.

*** Start of changes ***

# 8	Conclusions

## 8.2	Conclusions for Key issue#6: Uplink-downlink transmission coordination to meet Round-Trip latency requirements

The following aspects are concluded as principles for the normative work:

-	The RT latency can be preconfigured in PCF. Alternatively, the RT latency requirement can be provided by the AF. The RT latency requirement from the AF can be a requested 5GS delay (as defined in clause 6.1.3.22 of TS 23.503 [4]) together with a requested RT latency indication. The requested RT latency indication indicates the service data flow needs to meet that the RT delay overhead doesn't exceed the doubling of the 5GS delay. The AF provides the RT latency requirement to the PCF during AF session with required QoS procedure.

-	Based on RT latency requirement, the 5GS shall split the RT latency into an UL PDB and a DL PDB. The UL PDB and DL PDB can be unequal, but their sum shall not exceed the RT latency.The RT latency can be tracked by monitoring the UL and DL QoS Flows over a period of time (i.e., leveraging QoS monitoring method as defined in clause 5.33.3.2 of TS 23.501[2]). The alternative QoS Parameter Sets for UL/DL QoS Flows may be also derived based on the RT latency requirements and further added in the two PCC rules by PCF. Then SMF generates alternative QoS Profiles for both the UL and DL QoS Flows and sends them to RAN.

-	If the PCF split the RT latency into an UL PDB and a DL PDB, the PCF shall generate two PCC rules for the media stream, one for the UL and one for the DL direction and assigns the 5QIs according to the PDBs respectively. The two PCC rules shall also have associated QoS monitoring policy to enable RT latency tracking. This enables the UL/DL PDB to be tracked independently.

-	Different 5QIs shall apply to different QoS Flows as defined in current QoS model.

NOTE 1: As the DL and UL are separate QoS flows, the PER for UL QoS flow and PER for DL QoS flow are guaranteed respectively as legacy.