3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208565r03

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#1: Update Conclusion for EACI delivery

Document for:	Approval

Agenda Item:	9.11

Work Item / Release:	FS_EDGE_Ph2 / Rel-18

Abstract: Update the conclusion for KI#1 on ECS address configuration information delivery.

# 1. Introduction

This contribution proposes that for the HR case with HPLMN does not have the knowledge of EACI in VPLMN, the AF deployed in VPLMN provides EACI in VPLMN to the V-SMF via V-NEF, similar as the mechanism of providing EAS deployment information to the V-SMF. With this, there is no need to introduce V-PCF just for EACI delivery.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-48.

# * * * * First change * * * *

### 8.1.3	Conclusion for ECS Address Configuration Information delivery

1. ECS Address Configuration Information (EACI) provision by the AF to 5GC:

-	For LBO situation, the AF in visited PLMN provides the EACI by parameter provisioning procedure to UDM. This principle follows the design in Rel-17.

-	For HR situation, there are two categories:

1.	HPLMN has the knowledge of ECS address in VPLMN

In this case, it is assumed that an AF deployed in HPLMN provides the EACI to HPLMN (i.e. UDM) as per Rel-17.

NOTE: SA2 is not going to define how an AF deployed in HPLMN may know the EACI that should apply in a given VPLMN. A potential solution is that an AF deployed in VPLMN sends the EACI to an AF deployed in HPLMN (this procedure might be in scope of SA6).

For identification of the EACI to be used for UEs in different VPLMNs, all of the EACI should be retrieved per VPLMN ID. In the UDM and UDR, the subscription data of EACI should be stored as PLMN ID.

2.	HPLMN does not have the knowledge of EACI in VPLMN

In this case, the AF deployed in VPLMN provides the EACI in VPLMN to the V-SMF via the V-NEF. During the HR PDU Session establishment, the V-SMF sends the VPLMN EACI obtained from V-AF to H-SMF.

2. Provision of the EACI to the UE:

-	The V-SMF (for the LBO case) or the H-SMF via the V-SMF (for the HR case) provides the UE with the EACI as indicated in step 11 in figure 4.3.2.2.1-1 of TS 23.502 [9].

-	For HR situation, the V-SMF does not modify, but just delivers the EACI provided by the H-SMF.

For identification of the EACI to be used for UEs in different VPLMNs, all of the EACI should be retrieved per VPLMN ID. In the UDM and UDR, the subscription data of EACI should be stored as PLMN ID.

# * * * * End of changes * * * *