3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208579r03

Elbonia, October 10 – 14, 2022

Source:	China Mobile

Title:	KI#5: conclusion for support of edge relocation based on GSMA OPG impacts and improvements

Document for:	Approval

Agenda Item: 	9.11

Work Item / Release:	FS_EDGE_Ph2

Abstract: update conclusion for KI #5.

# 1. Discussion

# 2. Text Proposal

It is proposed to capture the following aspects in TR 23.700-48.

# * * * * First change * * * *

## 8.5	Interim conclusion for KI#5: GSMA OPG impacts and improvements for EHE operated by separate party

# Edge Relocation

For edge relocation in GSMA OPG scenario, the EAS reselection methods should be the same as the initial selection.

Edge relocation when EHEs are operated by different service providers and within the same hosting PLMN’s scenario is supported with below principles.

- SMF initiates AF/EAS relocation and selection of the target EHE e.g. due to UE mobility.

- SMF determines AF in the target EHE domain corresponding to the UE target location based on service area or EHE domain supported by AF

- Notification of User plane management event procedure is enhanced to support AF/EAS change (Sol #23)

EAS relocation between different PLMN’s EHE depend on GSMA’s feedback.

# * * * * End of changes * * * *