3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208608r01

Elbonia, October 10th – 17th, 2022	(revision of S2-2208608)

Source:	Huawei, HiSilicon

Title:	KI#2, Sol#9 Update Supporting AR Communication

Document for:	Approval

Agenda Item:	9.17.1

Work Item / Release:	FS_NG_RTC / Rel-18

Abstract: This paper proposes to update solution #9 AR communication in FS_NG_RTC TR.

- Introduction

This paper updates solutions 9 of TR 23.700-87.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-87.

# * * * * First change * * * *

6.9	Solution #9: Supporting AR communication

6.9.1	Description

According to the definition of TR 26.998 [14], different AR devices have different media processing capabilities. IMS needs to support media transmission and media rendering related to AR communication. This solution defines how IMS can be enhanced to support AR communication.

6.9.1.1	Assumption

This solution has the following assumptions:

-	Due to the flexibility (constant iteration, frequent upgrade required) and diversity (various industries, different experiences) of AR applications, built-in or pre-installed AR applications in AR devices will lead to software version fragmentation and difficult integration of different terminals. Therefore, this solution only considers the unified use of IMS data channel downloading mode for AR applications.

-	For the security of national laws and regulations, 3D models used for AR communication should be provided to the IMS when the AR application connects to IMS and saved in the persistent database. IMS needs to be notified when the 3D model is changed by AR application.

6.9.1.2	Definitions

The following definitions apply to this solution:

3D Model: 3D Model include 3D virtual object model and 3D virtual human avatar model.

AR metadata: Data used for AR media rendering, including user pose information and camera information and so on.

6.9.1.3	Architecture

Figure 6.9.1.3-1: Enhanced IMS architecture to support AR communication

NOTE 1:	The DCMF could be an enhanced MRF that support DC, the DC related architecture and interfaces will be defined in the solution to KI#1.

NOTE 2:	The support for network rendering of AR communication is optional for both DCMF and enhanced MRF.

AR communication architecture is enhanced based on data channel architecture, including the following network functions:

1.	Add a new network function AR Application Server, responsible for AR service control related to AR communication, including AR session media controlling, AR media rendering negotiation and AR service handling.

NOTE 3:	AR Application Server can be deployed in operator's network or belongs to third party.

2.	Enhance DCMF to support AR media processing, including the following functions:

-	AR Rendering Logic: controls the application-based rendering logic of AR communication.

-	AR Media Processing Function: including Vision Engine, 3D Rendering Engine. Vision Engine and 3D Rendering Engine will establish spatial map, and render the scenes, virtual human models and 3D object models according to the field of view, posture, position, etc. which are transmitted from UE using data channel.

The following reference points are used for the IMS AR communication architecture. It is suggested that SBI is applied to these reference points.

-	DC4: Service based reference point between the AR Application Server and the DCSF for AR service handling and AR session media control.

NOTE 4:	The DC4 interface will be defined in solution #20.

The following media interface are used for the IMS AR communication architecture.

-	MDC2: Reference point of data channel media between the AR Application Server and DCMF for AR media rendering negotiation.

-	Mb (RTP&DC): The media interface between UE and IMS-AGW, also between IMS-AGW and DCMF, including RTP channel used for audio/video media stream transmission and data channel used for data media stream transmission.

6.9.1.4 Interface and Operations

DC1 and DC2 interface related to AR service should be defined in this solution. DC1 and DC2 related to others will be defined in solution #20.

6.9.1.4.1 Ndcmf_RTPMediaResourceManagement(RTPMRM)

6.9.1.4.1.1 General

Service description: This service enables the IMS AS to create, update and delete IMS RTP media related resources.

The following table illustrates the DCMF Services and Service Operations:

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ndcmf_RTPMediaResourceManagement (RTPMRM) | Create | Request/Response | IMS AS |
|  | Update | Request/Response | IMS AS |
|  | Delete | Request/Response | IMS AS |

Table 6.9.1.4-1: NF services provided by the DCMF

6.9.1.4.1.2 Ndcmf_RTPMRM_Create service operation

Service operation name: Ndcmf_RTPMRM_Create

Description: The IMS AS requests the DCMF to allocate RTP media related resource.

Inputs, Required: None.

Inputs, Conditional: the remote end point SDP information.

Inputs, Optional: None.

Outputs, Required: result indication, connection ID, termination, SDP information of the DCMF.

Outputs, Optional: None.

6.9.1.4.1.3 Ndcmf_RTPMRM_Update service operation

Service operation name: Ndcmf_RTPMRM_Update

Description: The IMS AS requests the DCMF to update RTP media related resource.

Inputs, Required: connection ID, termination, the remote end point SDP information.

Inputs, Conditional: None.

Inputs, Optional: None.

Outputs, Required: result indication.

Outputs, Optional: None.

6.9.1.4.1.4 Ndcmf_RTPMRM_Delete service operation

Service operation name: Ndcmf_RTPMRM_Delete

Description: The IMS AS requests the DCMF to delete RTP media related resource.

Inputs, Required: connection ID, termination.

Inputs, Optional: None.

Outputs, Required: result indication.

Outputs, Optional: None.

6.9.1.4.2 Nimsas_RTPMediaControl (RTPMC) service

6.9.1.4.2.1 General

Service description: This service enables the DCSF to indicate the IMS AS to execute operations on RTP media streams.

6.9.1.4.2.2 Nimsas_RTPMC_Request service operation

Service operation name: Nimsas_RTPMC_Request

Description: The DCSF requests the IMS AS to deal with RTP media connection.

Inputs, Required: Call-ID of the SIP session, operation type.

The operation type includes:

-  Anchor caller audio/video media.

-  Anchor callee audio/video media.

Inputs, Conditional: None.

Inputs, Optional: None.

Outputs, Required: result indication.

Outputs, Optional: None.

6.9.1.4.3 Nimsas_ER_Notify service operation

A new event type(RTP media control request) will be added to Nimsas_ER_Notify which is defined in solution #20.

6.9.2	Procedures

According to AR communication scenarios defined by TR 26.998 [14] and TR 26.928 [15], media processing capabilities are different for different AR devices (STAR, EDGAR, and WLAR). The AR communication call flows can be summarized as follows:

1.	Terminal rendering process: When the media processing capability of AR device meets the requirements of AR communication (such as AR mark and simple AR effect) according to its status such as power status, signal status, computing power status, internal storage status, etc, the AR device can independently implement AR media rendering by itself. The either side of IMS provides AR application to the calling and the called users at the same time. The terminal performs local rendering based on the media obtained locally or sent by the peer.

2.	Network rendering process: When the media processing capability of AR device cannot meet AR communication requirements (such as complex scene or virtual human rendering requirements) according to its status, the AR device can decide to request IMS for AR media rendering. The either side of IMS provides AR application to the calling and the called users at the same time, IMS then performs AR media rendering based on AR media received from the calling or the called users, finally IMS sends rendered AR media using normal audio/video streams through RTP channel to the calling or called user.

6.9.2.1	Terminal Rendering Process

Figure 6.9.2.1-1: Terminal rendering process

Key steps:

1.	The UE-A initiates an AR communication session and establishes audio and video session connections with the UE-B.

2.	The bootstrap and application data channel connections are established for the UE-A and UE-B according to AR application requirements.

Scenario 1: The UE performs AR media rendering based on the AR media get from local application.

3.	The UE-A gets AR media (such as 3D model) from AR application locally and performs AR media rendering itself.

4.	The UE-A sends the rendered AR audio/video media to the peer through the RTP channel.

Scenario 2: The UE performs AR media rendering based on the AR media get from remote application.

5.	The UE-B sends AR media through application data channel to UE-A.

6.	The UE-A performs AR media rendering itself based on the AR media received, and displays the rendered audio/video media on its screen.

6.9.2.2	Network Rendering Process

Figure 6.9.2.2-1: Network rendering process

Key steps:

1.	The UE-A initiates an AR communication session and establishes audio and video session connections with the UE-B. Then the bootstrap data channels are established for the UE-A and UE-B.

AR Media Rendering Negotiation Procedure:

2.	The UE-A decide to request network media rendering based on its status such as power status, signal status, computing power status, internal storage status, etc.

3.	The UE-A initiate the application data channel between UE-A and the DCMF.

4.	The AR AS stores the media point information for DCMF.

5. The UE-A sends a Media Rendering Negotiation Request to IMS, finally the request is transferred to AR AS.

Editor note:	It is FFS how a UE invoke the network rendering function. SA4 has been asked to include procedures to support media rendering negotiation.

6.	The AR AS decides whether to provide media rendering function based on the request from UE in step 5 and its available resource.

7.	The AR AS sends a Media Rendering Negotiation Response to indicate the media rendering negotiation is successful.

UE-A can request to change the network rendering content if the status changed in the UE-A, such as low power, low computing power etc.

AR Session Media re-Negotiation Procedure For Network Rendering:

8.The AR AS sends a RTP Media Control request to the DCSF, then the DCSF sends to IMS AS using Nimsas_RTPMC_Request to anchor UE-A’s and UE-B’s audio/video stream to DCMF.

9-10. The IMS AS sends a reINVITE message to UE-A, to get UE-A’s audio/video media SDP information.

11. After receiving 200 from the UE-A, the IMS AS sends a RTP media create request using Ndcmf_RTPMRM_Create to the DCMF, carrying UE-A’s audio/video media SDP information, to connect UE-A’s audio/video RTP stream to DCMF.

12.	The IMS AS sends an ACK message to the UE-A carrying DCMF’s SDP information.

13-14.The IMS AS sends a reINVITE message to UE-B, to get UE-B’s audio/video media SDP information.

15. After receiving 200 from the UE-B, the IMS AS sends a RTP media create request using Ndcmf_RTPMRM_Create to the DCMF, carrying UE-B’s audio/video media SDP information, to connect UE-B’s audio/video RTP stream to DCMF.

16.	The IMS AS sends an ACK message to the UE-B carrying DCMF’s SDP information.

17.	The IMS AS sends a notify message to DCSF using Nimsas_ER_Notify, finally to AR AS to indicate the RTP media control response.

Networking Rendering Procedure:

18.	The UE-A sends AR media through application data channel to DCMF for AR media rendering.

19.	The DCMF performs AR media rendering.

20.	The UE-A performs part of the AR media rendering itself.

21-22.	The UE-A sends the rendered audio/video media over RTP to DCMF through IMS-AGW. If UE-A does not perform the rendering, the UE-A sends the original audio/video media over RTP to DCMF.

23-25.	The DCMF combines the AR media rendered by DCMF and the rendered AR media received from UE-A, and sends the complete audio/video media to UE-B. If UE-A does not perform the rendering itself, step 20 can be omitted, and the DCMF directly sends the rendered media to the UE-B.

6.9.3	Impacts on existing nodes and functionality

IMS AS:

-	Support RTP media resource management.

DCMF:

-  Support anchoring audio/video RTP media stream.

-	Support AR media rendering for AR communication based on the received AR metadata and audio/video RTP media stream.

AR AS:

-	Support AR media rendering negotiation between UE and IMS.

-	Support AR application service handling.

UE:

-	Support data channel capability.

-	Decides whether to use the network rendering method and change the split ratio of the rendering tasks based on its status.

# * * * * End of changes * * * *