3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208660r05

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	China Mobile

Title:	KI#3 Conclusion

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_Ph2 / Rel-18

Abstract: KI#3 is the KI studying the on-demand MBS session issue. Currently there are three solutions for this KI. It is proposed to clarify the way forward of KI#3.

- Introduction

Currently there are three solutions documented in the TR 23.700-47. Namely, solution #10, #11 and #30.

Solution #30 was the one submitted in the last meeting, and the general idea of sol#30 is to (Quote from TR 23.700-47, clause 6.30.1):

- "Avoid the need for an AF to establish the multicast session and fills related gaps that are left to implementation in Rel-17. It allows to have preconfigured policies for such on-demand multicast sessions. It allows to determine whether to establish a multicast session towards the radio based on the number/frequency/location of join requests".

And the proposed solution #30 is not for the on demand multicast MBS session management by AF, it is mainly for specifying the on demand multicast MBS session triggered by the UE, and the related steps were considered as implementation-specific in rel-17. On the other hand, it may not be aiming at resolving KI#3 specifically. The scope of KI#3 is mentioned in clause 5.3.1:

- "Whether and how to enhance the Release-17 MBS procedures to enable the on-demand multicast MBS session management. If needed, what information can be exposed by the 5GC to the AF or be provided by the AF, to enable on demand multicast MBS session management by AF"

The other solutions (#10 and #11) are for the purpose of letting AF provide the list of UEs to 5GC and later 5GC process the residual UE join procedure.

On-demand MBS for AF (Sol#10 and #11): converge these two solutions (and different alternatives in solution #11) into a single one, and address the concerns, including: 1) clarifying the steps and changes on current mechanism; 2) clarifying the benefits. By using the on-demand MBS from AF, one UL NAS signalling can be saved per UE, and when considering the number of the joined UE (especially for the public safety use case, the number could be high), considerable gain can be achieved. At the same time, Solution#11 mandates dynamic PCC deployment, assumes the availability of UE’s private IP address, and add impact to AF, PCF, SMF and UE.

Enabling temporary multicast group is the responsibility of the application layer, for instance in case of public safety groups it is a matter for SA6 to consider:

For public safety in EPS, SA6 has specified group management in TS 23.280 which allows group creation by administrators as well as user regrouping by authorized user/dispatcher (cf. clauses 10.2, 10.15)

For public safety in 5GS, similar functionality as in EPS is addressed in the ongoing study TR 23.700-76 (cf. Ad hoc group communication in clauses 7.1 and 7.2).

For other cases/scenarios, SA6 has also specified group management in TS 23.434.

After the temporary group is formed on the application layer, if multicast MBS Session is to be used to deliver the MBS service, the AF creates the MBS session and perform service announcement, and the UE then request to join the MBS Session in 5GC as specified in Rel-17.

Therefore, it is our view that the use case in KI#3 can already be achieved in Rel-17.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-47.

# * * * * Next change * * * *

8	Conclusions

8.3	Key Issue #3: On demand multicast MBS session

The following conclusions is proposed for KI#3:

-	Solution #11 is selected as the baseline for On-demand MBS for AF with the assumption that PCC deployment is available, and AF interacts with PCF via unicast slices and DNNs.

Temporary multicast group can be created by the application layer.

NOTE: For example, as per SA6 specification and study (e.g., TS 23.280, TR 23.700-76), creation of a temporary group on the application layer is already possible.

However, for external IP multicast sessions the application layer may not have central knowledge of receivers and arbitrary clients can join. Rel-17 already allows the 5GC to create MBS multicast sessions without request from the AF when a UE requests to join an external IP multicast session, but leaves details of SMF and MB-SMF behaviour open to the implementation. The open aspects will be closed in Rel-18 based on solution #30.

# * * * * End of changes * * * *