SA WG2 Meeting #153e	S2-2208684r01

# October 10th – 14th, 2022; Elbonia               	  			  		(revision of S2-2208684+8320)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3578 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction to N3IWF selection to support the slices needed by the UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell [Huawei, HiSilicon, Qualcomm, China Telecom]? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC_Ph2 |  |  |  |  |  | Date: |  |  | 2022-09-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Conclusions of FS_5WWC_Ph2 documented in clause 8 of TR 23.700-17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update to 4.12.2.2 Registration procedure for untrusted non-3GPP access to support the UE Selection of a N3IWF that supports the S-NSSAIs needed by the UE as enabled based on ANDSP configuration and N3IWF selection based on this information defined in 23.501. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature agreed as part of Conclusions of FS_5WWC_Ph2 documented in clause 8 of TR 23.700-17 cannot be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.12.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | 23.501 CR ffs |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.12.2.2	Registration procedure for untrusted non-3GPP access

The signalling flow in Figure 4.12.2.2-1 does not show all the details of a registration procedure via untrusted non-3GPP access. It shows primarily the steps executed between the UE and N3IWF. All the details of a registration procedure, including interactions with PCF, UDM, etc. are specified in clause 4.2.2.2.2.

Figure 4.12.2.2-1: Registration via untrusted non-3GPP access

1.	The UE connects to an untrusted non-3GPP Access Network with any appropriate authentication procedure and it is assigned an IP address. For example, a non-3GPP authentication method can be used, e.g. no authentication (in the case of a free WLAN), EAP with pre-shared key, username/password, etc. When the UE decides to attach to 5GC network, the UE selects an N3IWF in a 5G PLMN, as described in clause 6.3.6 of TS 23.501 [2].

NOTE 0:	The UE Selection of a N3IWF that supports the S-NSSAIs needed by the UE is enabled based on ANDSP configuration defined in 23.501 [2]. The N3IWF selection based on this information is documented in 23. 501 [2].

2.	The UE proceeds with the establishment of an IPsec Security Association (SA) with the selected N3IWF by initiating an IKE initial exchange according to RFC 7296 [3]. After step 2, all subsequent IKE messages are encrypted and integrity protected by using the IKE SA established in this step.

3.	The UE shall initiate an IKE_AUTH exchange by sending an IKE_AUTH request message. The AUTH payload is not included in the IKE_AUTH request message, which indicates that the IKE_AUTH exchange shall use EAP signalling (in this case EAP-5G signalling). If the UE supports MOBIKE, it shall include a Notify payload in the IKE_AUTH request, as specified in RFC 4555 [40], indicating that MOBIKE is supported. In addition, as specified in TS 33.501 [15], if the UE is provisioned with the N3IWF root certificate, it shall include the CERTREQ payload within the IKE_AUTH request message to request the N3IWF's certificate.

4.	The N3IWF responds with an IKE_AUTH response message, which includes an EAP-Request/5G-Start packet. The EAP-Request/5G-Start packet informs the UE to initiate an EAP-5G session, i.e. to start sending NAS messages encapsulated within EAP-5G packets. If the N3IWF has received a CERTREQ payload from the UE, the N3IWF shall include the CERT payload in the IKE_AUTH response message containing the N3IWF's certificate. How the UE uses the N3IWF's certificate is specified in TS 33.501 [15].

5.	The UE shall send an IKE_AUTH request, which includes an EAP-Response/5G-NAS packet that contains the Access Network parameters (AN parameters) and a Registration Request message. The AN parameters contain information that is used by the N3IWF for selecting an AMF in the 5G core network. This information includes e.g. the GUAMI, the Selected PLMN ID (or PLMN ID and NID, see clause 5.30 of TS 23.501 [2]), the Requested NSSAI and the Establishment cause. The Establishment cause provides the reason for requesting a signalling connection with 5GC. Whether and how the UE includes the Requested NSSAI as part of the AN parameters is dependent on the value of the Access Stratum Connection Establishment NSSAI Inclusion Mode parameter, as specified in clause 5.15.9 of TS 23.501 [2]. The registration request may contain an indication that the UE supports N3IWF selection based on the slices the UE wishes to use over non Trusted access.

NOTE 1:	The N3IWF does not send an EAP-Identity request because the UE includes its identity in the first IKE_AUTH. This is in line with RFC 7296 [3] clause 3.16.

6.	The N3IWF shall select an AMF based on the received AN parameters and local policy, as specified in clause 6.3.5 of TS 23.501 [2]. The N3IWF shall then forward the Registration Request received from the UE to the selected AMF within an N2 message. This message contains N2 parameters that include the Selected PLMN ID and the Establishment cause.

7.	The selected AMF may decide to request the SUCI by sending a NAS Identity Request message to UE. This NAS message and all subsequent NAS messages are sent to UE encapsulated within EAP/5G-NAS packets.

8.	The AMF may decide to authenticate the UE by invoking an AUSF. In this case, the AMF shall select an AUSF as specified in clause 6.3.4 of TS 23.501 [2] based on SUPI or SUCI.

The AUSF executes the authentication of the UE as specified in TS 33.501 [15]. The AUSF selects a UDM as described in clause 6.3.8 of TS 23.501 [2] and gets the authentication data from UDM. The authentication packets are encapsulated within NAS authentication messages and the NAS authentication messages are encapsulated within EAP/5G-NAS packets. After the successful authentication:

-	In step 8h, the AUSF shall send the anchor key (SEAF key) to AMF which is used by AMF to derive NAS security keys and a security key for N3IWF (N3IWF key). The UE also derives the anchor key (SEAF key) and from that key it derives the NAS security keys and the security key for N3IWF (N3IWF key). The N3IWF key is used by the UE and N3IWF for establishing the IPsec Security Association (in step 11).

-	In step 8h, the AUSF shall also include the SUPI, if in step 8a the AMF provided to AUSF a SUCI.

NOTE 2:	EAP-AKA' or 5G-AKA are allowed for the authentication of UE via non-3GPP access, as specified in TS 33.501 [15]. Figure 4.12.2.2-1 only shows authentication flow using EAP-AKA'. Authentication methods other than EAP-AKA' or 5G-AKA are also allowed for UE accessing SNPN services via a PLMN, as specified in TS 33.501 [15], Annex I.

9a.	The AMF shall send a NAS Security Mode Command to UE in order to activate NAS security. If an EAP-AKA' authentication was successfully executed in step 8, the AMF shall encapsulate the EAP-Success received from AUSF within the NAS Security Mode Command message.

9b.	The N3IWF shall forward the NAS Security Mode Command message to UE within an EAP/5G-NAS packet.

9c.	The UE completes the EAP-AKA' authentication (if initiated in step 8), creates a NAS security context and an N3IWF key and sends the NAS Security Mode Complete message within an EAP/5G-NAS packet.

9d.	The N3IWF relays the NAS Security Mode Complete message to the AMF.

10a.	Upon receiving NAS Security Mode Complete, the AMF shall send an NGAP Initial Context Setup Request message that includes the N3IWF key.

10b.	This triggers the N3IWF to send an EAP-Success to UE, which completes the EAP-5G session. No further EAP-5G packets are exchanged.

11.	The IPsec SA is established between the UE and N3IWF by using the common N3IWF key that was created in the UE in step 9c and received by the N3IWF in step 10a. This IPsec SA is referred to as the "signalling IPsec SA". After the establishment of the signalling IPsec SA, the N3IWF notifies the AMF that the UE context (including AN security) was created by sending a NGAP Initial Context Setup Response. The signalling IPsec SA shall be configured to operate in tunnel mode and the N3IWF shall assign to UE an "inner" IP address. If the N3IWF has received an indication that the UE supports MOBIKE (see step 3), then the N3IWF shall include a Notify payload in the IKE_AUTH response message sent in step 11a, indicating that MOBIKE shall be supported, as specified in RFC 4555 [40].

All subsequent NAS messages exchanged between the UE and N3IWF shall be sent via the signalling IPsec SA and shall be carried over TCP/IP. The UE shall send NAS messages within TCP/IP packets with source address the "inner" IP address of the UE and destination address the NAS_IP_ADDRESS that is received in step 11a. The N3IWF shall send NAS messages within TCP/IP packets with source address the NAS_IP_ADDRESS and destination address the "inner" IP address of the UE. The TCP connection used for reliable NAS transport between the UE and N3IWF shall be initiated by the UE right after the signalling IPsec SA is established in step 11a. The UE shall send the TCP connection request to the NAS_IP_ADDRESS and to the TCP port number specified in TS 24.502 [41].

12.	The AMF determines the subset of the requested NSSAI that is allowed by the subscribed S-NSSAI(s); the AMF may detect that the N3IWF used by the UE is not compatible with this subset and then proceed with steps 15 to 19. Otherwise (the N3IWF supports the the subset of the requested NSSAI that is allowed by the subscribed S-NSSAI(s) the AMF proceeds with step 13 and 14 and steps 15-19 are skipped

NOTE 3: The AMF considers the subscribed S-NSSAI(s) before determining to trigger the UE PCF. This is to avoid triggering the UE PCF to update the UE policies in relation with Requested but non subscribed S-NSSAI, avoiding the potential DoS related with repeatedly trying to update policies on the UE because the UE is repeatedly asking to access to S-NSSAI(s) that are not allowed by its subscription

13.	The AMF sends the NAS Registration Accept message in a N2 message sent to the N3IWF. The N2 Message includes the Allowed NSSAI for the access type for the UE. The Allowed NSSAI is a subset of the slices supported by the selected N3IWF

14.	The N3IWF forwards the NAS Registration Accept message to UE via the established signalling IPsec SA. If the NAS Registration Accept message is received by the N3IWF before the IPsec SA is established, the N3IWF shall store it and forward it to the UE only after the establishment of the signalling IPsec SA.

Steps 15 to 19.correspond to the case where the AMF has detect that the N3IWF used by the UE is not compatible with the subset of the requested NSSAI that is allowed by the subscribed S-NSSAI(s);

15. If the UE Registration Request contains an indication that the UE supports N3IWF selection based on the slices the UE wishes to use over non-Trusted access, the AMF triggers the UE PCF to update the N3IWF selection related policies on the UE (actually ANDSP), providing this indication to the PCF, and asks to be notified when this policy update is completed. For this purpose, the AMF may use Npcf_UEPolicyControl_Create or Npcf_UEPolicyControl_Update indicating a willingness to be notified when the policy update is completed

Editor's note:	Whether there is a need for the UE to indicate support of extended ANDSP to the AMF so that AMF can establish the UE Policy Association only for supporting UEs is FFS.

- The PCF updates the UE policy

17. The PCF notifies via Npcf_UEPolicyControl_UpdateNotify that the UE policy update has completed

- The AMF sends via the N3IWF a UE Registration Reject indicating that the UE selected N3IWF was not appropriate for the requested slices that the UE is allowed to access to. The AMF optionaly may provide target N3IWF information (e.g., FQDN and/or IP address) to the UE within the Registration Reject message

NOTE 4:	The AMF may determine a target N3IWF that supports the subset of the requested NSSAI that is allowed by the subscribed S-NSSAI(s) based on the list of supported TAs and the corresponding list of supported slices for each TA obtained in N2 interface management procedures as specified in TS 38.413 [10].

- if supported by the UE, The UE may connect to a target N3IWF using target N3IWF information received in the Registration Reject, otherwise the UE may perform N3IWF selection again using the updated N3IWF selection information received in step16. The UE uses the target N3IWF information in the Registration Reject only for the N3IWF selection directly following the rejected registration

The AMF provides the Access Type set to " Non-3GPP access" to the UDM when it registers with the UDM and the RAT type determined as specified in TS 23.501 clause 5.3.2.3.

NOTE 5:	The Access Type and the RAT type are set to "Untrusted Non-3GPP access" even when the UE accesses SNPN services via PLMN over 3GPP access.

NEXT CHANGE

NEXT CHANGE

NEXT CHANGE (4)

NEXT CHANGE (5)

END OF CHANGES