SA WG2 Meeting #153E e-meeting	S2-2208732

Elbonia, 10th – 17th October 2022	(revision of S2-220xxxx)

Source:	Samsung, Nokia, Nokia Shanghai-Bell, Ericsson

Title:	Conclusion update on KI #2(MOCN network sharing)

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_ph2 / Rel-18

Abstract: This contribution proposes some principles for conclusion on KI #2.

# 1.	Introduction

MOCN RAN’s sharing scenario update and MBS service PLMN’s sharing scenario update should be considered for normative works considering actual service scenario in the field.

# 2.	Proposal

It is proposed to capture the following changes into TR 23.700-47.

*******Start of 1st change*******

## 8.2	Key Issue #2:	MOCN network sharing

### 8.2.1	Interim Conclusions

Editor's note:	RAN WGs will determine the feasibility of radio resource utilization optimization.

For conclusions, the following aspects will be considered:

-	For solutions where the broadcast MBS sessions for different PLMNs are established towards a NG-RAN node, the NG-RAN node shall be able to identify the same MBS service and avoid multiple deliveries over radio.

-	A solution compatible with Rel-17 UEs is preferred.

The following interim conclusions will be taken into account:

-	It should be possible not to establish all the shared delivery tunnels to the same NG RAN from different PLMNs for the same MBS service.

-	The solution should support scenario where all RAN nodes are shared by PLMNs and the scenario where only part of the RAN nodes are shared by PLMNs.

-	The solution may support the addition and removal of PLMNs from a shared broadcast MBS service.

-	NEF should be able to validate whether an AF is allowed to create an MBS session for MOCN network sharing

-	 For the scenario that different AFs interact with different PLMNs for the same MBS service with MOCN network sharing, it needs to be evaluated in the normative phase whether and how those AFs need to coordinate possible MBS service information applicable in all PLMNs. and need to provide the same data for the MBS session.

*******End of changes*******