SA WG2 Meeting #153E eMeeting	S2-2208777r04

Elbonia, Oct. 10 – 14, 2022 	(revision of S2-20xxxxx)

Source:	NTT DOCOMO, Oppo, Huawei, Oracle, Toyota, Samsung

Title:	TR 23.700-80: KI#5 – Conclusion

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys / Rel-18

Abstract of the contribution: This contribution proposes a KI#5 conclusion.

# Background

Based on the evaluation in clause 7.5, the following conclusions are proposed for KI#5.

# Proposals

***** Start of Change *****

## 8.5	Key Issue #5: 5GC Enhancements to enable Application AI/ML Traffic Transport

First Step: AI/ML data transfer window negotiation mechanism: It is agreed to consider solution #10 as a basis for KI#5, with the following principles:

When the AF requests the network to provide QoS with individual QoS parameters, one or more Requested Alternative QoS Parameter Set(s) in a prioritized order. Each Requested Alternative QoS Parameter Set is comprised of the following individual parameters: Requested 5GS Delay and Requested Guaranteed Flow Bitrate.

-	Principle 1: A new Application Data Transfer (ADT) policy is adopted to support application data transfer. The ADT policy includes a list of recommended time window(s) with predicted QoS parameters (e.g., PDB, PER, bit rate) per UE per time window for the Application data transfer to start. In addition, if the AF provided one or more Requested Alternative QoS Parameter Set(s) then an indication of the Requested Alternative QoS Parameter Set for the provided recommended time window. .

-	Principle 2: The AF includes QoS requirements such as the Requested Traffic Rate, the Requested Packet Loss Rate and the Requested Packet Delay Budget and possibly Alternative QoS Profile(s) with a combination of QoS parameters PDB, PLR and bitrate with minimum requirements to which the application data traffic transmission is able to adapt , in the ADT policy negotiation request.

NOTE 1: The Alternative QoS Profile also includes the GFBR, however this is part of the AF request for ADT negotiation as it is not possible to derive a GFBR from the Network Performance Analytics and the Data Network Performance Analytics.

NOTE 2: In the QoS requirements, AF includes the expected minimum QoS requirement from the network.

-	Principle 3: The PCF uses the extended Network Performance Analytics of NWDAF to derive the candidate time window(s) that may fulfil the requirements requested from AF. The PCF generates the list of ADT policies and feedback it to the AF. The QoS parameters in the ADT policy are determined by PCF based on the list of individual QoS parameters and the requested Alternative QoS Profile (s)  from the AF.

-	Principle 4: The Network Performance Analytics support the following  analytics outputs:

- 	Overall Average and Maximum Packet Delay for UL/DL per UE per time window;

-	Overall Average and Maximum Packet Loss Rate for UL/DL per UE per time window;

-	Overall Average, Minimum and Maximum Traffic rate for UL/DL per UE per time window.

-	Principle 5: The AF selects one of the ADT policies provided by PCF which may most fit for its requirement, and indicate it to the PCF.

-	Principle 6: It is agreed to support data transfer time window negotiation between the AF and the 5GS

  - The Application traffic could be non GBR type or GBR type. The AF can provide Alternative QoS profile(s) for GBR flows only.

Second step - Application Data Transmission:

-

When the Application AI/ML begins e.g., to transport intermediate data, local training data, inference results, or model performance data, through 5GS, the AF can leverage the either existing service Nnef_AFsessionWithQoS_Create request message as described in clause 4.15.6.6 of TS 23.502 [4], including the specific QoS parameters for the AF session or new AF service which will be further determined during the normative phase.

Charging for Application AIML traffic transport:

-	Based on operator policy, charging of different Application AI/ML traffic needs to be perform, however, nodo not expect any normative work on this aspect in the SA2 specifications.

NOTE 2: An alignment can be required (if any) in the normative based on the outcome of SA WG5.

AI/ML transport routing configuration information:

-	The existing AF Influence of traffic routing mechanism is used to support the routing configuration AI/ML transport.

-	Transport routing configuration parameters include AI/ML AF address and A/ML DNS server address.

***** End of Change *****