SA WG2 Meeting #S2-153e(e-meeting)	S2-2208810r01

10 - 17 October, 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	CATT

Title:	KI#11: Add evaluation and conclusion for key issue#11

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_Ph3

Abstract of the contribution: this pCR is proposed to add evaluation and conclusion for key issue#11.

# 1. Discussion

This pCR is proposed to add evaluation and conclusion for key issue#11.

- KI#11: Enhance the Triggered Location for UE power saving purpose

There are two solutions related to key issue#11, i.e. solution#25 and solution#35.

In solution#25, event report allowed area can be applied to all event types and is introduced to limit the area to send the event report. The UE can adjust the area based on the power status, e.g. if the power status of UE is low, the UE can set the area to a small area. Only when the UE is in the area, the UE needs to detect whether the event happens. So the event report allowed area is used to reduce UE power consumption by reducing the number of event report and the number of UE positioning procedures which may be triggered by the event report.

In solution#35, the area type event report is enhanced, i.e. event report is needed only when the UE locates in the exact target area. To make the decision, UE or LMF triggers UE positioning procedure to obtain UE location and compares the UE location with the target area. Because the decision depends on the UE location, it is very likely that multiple UE positioning procedures are needed before the event happens. The UE positioning procedure consumes UE power, so this solution leads to higher UE power consumption especially when more than one UE positioning procedures are needed before the event happens.

# 2. Proposal

Based on the discussion in clause 1, it is proposed to add the evaluation and conclusion for key issue#11.

* First change *

## 7.11	Key Issue #11: Enhance the Triggered Location for UE power saving purpose

There are two solutions related to key issue#11, i.e. solution#25 and solution#35.

In solution#25, event report allowed area can be applied to all event types and is introduced to limit the area to send the event report. The UE can adjust the area based on the power status, e.g. if the power status of UE is low, the UE can set the area to a small area. Only when the UE is in the area, the UE needs to detect whether the event happens. So the event report allowed area is used to reduce UE power consumption by reducing the number of event report and the number of UE positioning procedures which may be triggered by the event report. Solution #25 also contains two ambiguous aspects concerning whether a UE should occasionally send event reports when outside an allowed area to show an HGMLC and LCS Client that event reporting is still active in the UE and on whether an allowed area can be used in an opposite sense where a UE only reports when outside a certain area (e.g. where a UE spends most time in the area but UE location is only needed when the UE is outside the area).

In solution#35, the area type event report is enhanced, i.e. event report is needed only when the UE locates in the exact target area. To make the decision, UE or LMF triggers UE positioning procedure to obtain UE location and compares the UE location with the target area. Because the decision depends on the UE location, it is very likely that multiple UE positioning procedures are needed before the event happens. The UE positioning procedure consumes UE power, so this solution leads to higher UE power consumption especially when more than one UE positioning procedures are needed before the event happens.

* Second change *

## 8.x	Key Issue #11: Enhance the Triggered Location for UE power saving purpose

For key issue #11, it is concluded that solution#25 (Event Report in an Allowed Area) is used as the baseline for normative work with the two ambiguous aspects identified in clause 7.11 also being resolved.

* End of changes *