3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208883r02

Elbonia, Oct 10th – 17th, 2022	(revision of S2-220xxxx)

Source:	China Mobile

Title:	KI#7 Update to Conclusion

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract of the contribution: This contribution proposes a conclusion update for KI#7.

1. Background

This paper proposes conclusion update on the key aspects of the Enhancements on QoS Sustainability analytics discussed in solutions adopted in the TR for Key Issue 7. To determine the UEs which locate in fine granularity, NWDAF should first determine the UEs in the TA/cell from AMF, then get the geographical location via LCS service request to GMLC, at last identify the UEs located in finer granularity area by comparing the UEs' locations to the finer granularity area, so GMLC should also be involved.

2. Text Proposal

It is proposed to adopt the following text within the TR 23.700-81.

# * * * * First change * * * *

8	Conclusions

8.7	Key Issue #7: Enhancements on QoS Sustainability analytics

In addition to the RAN UE Throughput and the QoS flow Retainability, additional input data should be standardized for the QoS Sustainability analytics as proposed in Solution #18 and Solution #20, such as average UL or DL packet delay, average GTP metric (UL/DL packet delay, UL/DL capacity or UL/DL available capacity), as well as support for additional filter information related to the UE Context or Subscription in the request or subscription from the service consumer.

For NWDAF to derive QoS sustainability analytics in a finer granularity area smaller than cell, Alt#2 of Sol#19 and Sol#50 are selected as baseline for the normative work. The selected proposals are as follows:

-	Based on the requested finer granularity area from AF, NWDAF derives the UE ID list within the finer granularity area based on the UE Location information from GMLC/LMF/AMF, e.g., NWDAF identifies the UEs inside of the finer granularity area by comparing the UEs' geographical locations retrieved from GMLC/LMF with the finer granularity area provided from AF.

-	NWDAF collects the input data for the UE ID list within the finer granularity area such as QNC/AQP and QoS profile from SMF/PCF, or MDT data from OAM, or QoS flow information from UPF. In addition, the input data may also include QoS flow Bit Rate/QoS flow Packet Delay from UPF and additional input from UDM in Sol#50.

-	NWDAF derives QoS sustainability statistics or predictions for the finer granularity area by averaging these input data for the UE list.

-	In order to limit impact on signalling, input data from SMF/UPF/PCF shall be collected only in case of QoS metrics are not fulfilled

Note:  	The additional input data required for finer granularity is only collected after the AF has requested/subscribed to the analytics. The UEs in the area of interest/UE path to be analysed by the NWDAF may not be the ones which initiate the analytics request. Therefore there is no guarantee that relevant input data for the generation of the analytics can be collected immediately after AF subscription.  AF may need to request/subscribe to the analytics certain time (e.g. days, weeks, or months) before analytics are needed.  Alternatively, NWDAF can use other mechanisms (e.g. non-standard) to collect input data prior to application request/subscription.

# * * * * End of changes * * * *