3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208911

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#1: Conclusion update

Document for:	Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2 / Rel-18

Abstract: Update of KI#1 conclusion.

# 1. Introduction/Discussion

This paper proposes the update of KI#1 conclusion with the following proposals,

Proposal 3: For service authorization and policy/parameter provisioning for UE-to-UE Relay operation, it proposes that Relay Service Code(s) and UE-to-UE Relay Layer indicator(s) per ProSe service are included in the policy/parameter.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-33.

# * * * * First change * * * *

## 8.1	Key Issue #1: Support of UE-to-UE Relay

For Key Issue #1 (Support of UE-to-UE Relay), the followings are taken as conclusions:

The following conclusions are common for both Layer-3 UE-to-UE Relay and Layer-2 UE-to-UE Relay:

-	For UE-to-UE Relay discovery, both Model A and Model B discovery are supported.

Editor's Note: It is FFS the exact information to be included in discovery messages.

Editor's Note: It is FFS if discovery integrated into PC5 unicast link establishment procedure is supported.

-	For UE-to-UE Relay selection, the Source UE performs the UE-to-UE Relay selection for both Model A and Model B discovery.

-	For service authorization and policy/parameter provisioning for UE-to-UE Relay operation, the PCF based service authorization and provisioning as defined in TS 23.304 [3] is used as basis for normative work.

-	The policy/parameters per ProSe service includes Relay Service Code(s) and UE-to-UE Relay Layer indicator(s); a UE-to-UE Relay Layer Indicator per RSC that indicates whether the RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-UE Relay service.

Editor's Note: It is FFS if Target UE can also perform the UE-to-UE Relay selection for Model B discovery.

-	For UE-to-UE Relay reselection, the negotiated UE-to-UE Relay reselection between Source UE and Target UE and the UE-to-UE Relay selection procedure can be used under different conditions.

NOTE 1:	UE-to-UE Relay selection/reselection requires cooperation with RAN WGs during normative work.

-	IP, Ethernet and Unstructured traffic types are supported.

NOTE 2: 	Ethernet and Unstructured traffic types can be encapsulated in IP traffic type if supported by source and target UE.

NOTE 3: 	Conclusion on Service Authorization information related to UE-to-UE Relay operation sent by AMF to NG-RAN can be finalized based on feedback from RAN WGs.

The following conclusions are specific for Layer-3 UE-to-UE Relay:

- 	For UE-to-UE Relay Per-hop links setup (i.e. PC5 link establishment between Source UE and UE-to-UE Relay, as well as between UE-to-UE Relay and Target UE), Source UE initiates the PC5 link setup with UE-to-UE Relay, and UE-to-UE Relay initiates the PC5 link setup with the target UE.

- 	For QoS control of Layer-3 UE-to-UE Relay, the UE-to-UE Relay receives E2E QoS from Source UE and determines the per-hop QoS parameters to satisfy the E2E QoS.

The following conclusions are specific for Layer-2 UE-to-UE Relay:

-	Per-hop links (i.e. PC5 link between Source UE and UE-to-UE Relay, as well as between UE-to-UE Relay and Target UE) needs to be established before E2E PC5 link establishment is performed.

NOTE 4:	How the E2E PC5-S messages are forwarded by the UE-to-UE Relay is to be determined by RAN WGs.

NOTE 5:	For Layer-2 UE-to-UE Relay, RAN WGs will define how the E2E QoS will be handled and split over the PC5 links.

# * * * * End of changes * * * *