SA WG2 Meeting #S2-153E	S2-2208949

10 - 14 October 2022, Electronic, Elbonia	(revision of S2-220xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0141 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the shared delivery tunnel management for common gNB UP case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Nokia, Nokia Shanghai-Bell, Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN3 send the LS (S2-2208148) on the shared NG-U Termination issue among gNBs in case of shared delivery establishment for multicast MBS session. In the scenario, all the NG-RAN CP share the same UP (i.e. same DL TEID) for one multicast MBS sesson. It is unclear how the MB-SMF control the MB-UPF to handle the shared tunnle in the sceanrio. RAN3 has provides 3 options: - MB-SMF based alt; - MB-UPF based alt; - NG-RAN based alt; The discussion paper S2-2208948 analyzes the scenarios and solution options and propose the MB-SMF based solution which does not impact the signalling/interface. In addition, stage 3 specs state that the NG RAN Node ID is sent to the MB-SMF to enable a subsequent restoration of NG RAN nodes. This is not reflected in stage 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | During the shared delivery tunnel establishmentt, the MB-SMF store the RAN id and DL GTP tunnel info. The MB-SMF check whether the tunnel has been established before it config the MB-UPF. During the shared delivery tunnel release, the MB-SMF remove the stored the RAN id and DL GTP tunnel info. The MB-SMF check whether the tunnel is still used by other shared delivery tunnel before it config the MB-UPF to stop. Stage 3 allignment: NG RAN ID is sent from AMF to MB-SMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how the MB-SMF control the MB-UPF to handle the shared tunnle in the shared NG-U Termination. Contradictions with stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9.1, 7.2.1.4, 7.2.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.9.1	MBS Session Context

The MBS Session Context contains all information describing a particular MBS session in the 5GS and is created in each node involved in the delivery of the MBS data.

The content of the Multicast MBS Session Context is described in Table 6.9.1-1.

Table 6.9.1-1: Multicast MBS Session Context

| Parameter | Description | NG-RAN | AMF | SMF | MB-SMF |
| --- | --- | --- | --- | --- | --- |
| State | State of MBS session ('Active' or 'Inactive' or 'Configured') | X (note 2) |  | X (note 2) | X |
| SSM (source specific IP multicast address) | IP multicast address identifying the MBS session. |  |  | X (note 1) | X (note 1) |
| TMGI | Temporary Mobile Group Identity allocated to the MBS Session. | X | X | x | X |
| Area Session Identifier | Used for MBS session with location dependent content. When present, the Area Session Identifier together with the TMGI uniquely identify the MBS Session in a specific MBS service area. | X (note 1) | X | X (note 1) | X (note 1) |
| MB-SMF | The MB-SMF that handles the MBS session. |  | X | X |  |
| QoS information | QoS information of the MBS session. | X |  | X | X |
| MBS Service Area | Area over which the MBS session data is distributed (i.e. Cell ID list or TAI list). | X (note 1) |  | X (note 1) | X (note 1) |
| NG-RAN Node ID(s) | NG-RAN nodes which are involved in the Multicast MBS session |  | X |  | X (note 1, note 4) |
| AMF | The AMF(s) which are selected for the MBS session | X |  |  | X |
| IP multicast and source address for data distribution | IP addresses identifying the SSM user plane transport for shared delivery between MB-UPF and NG-RAN and for individual delivery between MB-UPF and UPF when the IP multicast transport is used. | X (note 1) |  | X (note 1) | X (note 1) |
| SMF | The SMF(s) that manages the associated PDU session. |  |  |  | X |
| UE ID | ID identifying the UE that successfully join the Multicast MBS session. For NG-RAN it is NGAP UE ID and for SMF it is SUPI. | X (note 3) |  | X (note 3) |  |
| IP address for distribution | The IP addresses and TEID of NG-RAN used for the user plane between NG-RAN and MB-UPF and between MB-UPF and UPF when Point to Point tunnel is used. | X (note 1) |  | X (note 1) | X (note 1, note 4) |
| TEID for data distribution | The tunnel ID used for receiving the multicast data for shared delivery by NG-RAN and for individual delivery by UPF | X |  | X | X (note 1, note 4) |
| PCF | The MB-PCF that provides policy control for the MBS session. |  |  |  | X (note 1) |
| NOTE 1: It is an optional parameter. NOTE 2: The value 'Configured' is not applicable for NG-RAN and SMF. NOTE 3: the UE ID is available within the UE Context which contains the MBS information. NOTE 4: The Parameter needs to be stored in deployments with shared NG-U termination(s) if unicast transport is used. |  |  |  |  |  |

In Broadcast MBMS mode, an MBS Session Context is created in the NG-RAN, AMF, MB-SMF and MBSF as a result of the MBS Session Start procedure.

The content of the Broadcast MBS Session Context is described in Table 6.9.1-2.

Table 6.9.1-2: Broadcast MBS Session context

| Parameter | Description | NG-RAN | AMF | MB-SMF |
| --- | --- | --- | --- | --- |
| TMGI | Temporary Mobile Group Identity allocated to the MBS Session. | X | X | X |
| Area Session Identifier | Used for MBS session with location dependent content. When present, the Area Session Identifier together with the TMGI uniquely identify the MBS Session in a specific MBS service area. | X (note 1) | X (note 1) | X (note 1) |
| AMF | The AMF(s) which are selected for the MBS session | X |  | X |
| MB-SMF | The MB-SMF that handles the MBS session. |  | X |  |
| QoS information | QoS information for the MBS Session, including the QoS parameters of QoS flows. | X |  | X |
| MBS Service Area | Area over which the MBS session data is distributed (i.e. Cell ID list or TAI list). | X | X | X |
| NG-RAN Node ID(s) | NG-RAN nodes which are selected for the Broadcast MBS session |  | X |  |
| IP multicast address for data distribution | IP addresses identifying the user plane transport used for shared delivery between MB-UPF and NG-RAN when the IP multicast transport is used. | X (note 1) |  | X (note 1) |
| NG-RAN IP Address for data distribution | The IP address of NG-RAN used for the user plane between NG-RAN and MB-UPF when Point to Point tunnel is used. | X (note 1) |  | X (note 1) |
| TEID for data distribution | The tunnel ID used for receiving the broadcast data for shared delivery by NG-RAN | X |  | X |
| PCF | The PCF that provides policy control for the MBS session. |  |  | X (note 1) |
| MBS FSA ID | MBS Frequency Selection Area (FSA) ID is used for broadcast MBS sessions to guide the frequency selection of the UE. | X |  | X |
| NOTE 1: It is an optional parameter. |  |  |  |  |

Next CHANGE

#### 7.2.1.4	Establishment of shared delivery toward RAN node

In the following cases, the shared tunnel for shared delivery is established between the NG-RAN and MB-UPF:

-	The first UE is included in the context of the MBS session in the NG-RAN.

NOTE 1:	When the multicast MBS session is inactive, if there is at least one UE joining the multicast MBS session is in RRC-CONNECTED state in the NG-RAN, the shared delivery is not released.

NOTE 2:	Share delivery establishment procedures are used when MBS supporting NG-RAN node(s) get involved in the multicast MBS session regardless of the state of the multicast MBS session.

-	Handover to the target NG-RAN when the shared delivery tunnel is not established in the target RAN node for this multicast MBS session.

Figure 7.2.1.4-1: Establishment of shared delivery toward NG-RAN node

1.	A NG-RAN node decides to establish shared delivery for a multicast MBS session when it serves at least one UE within the multicast MBS session. For location dependent services, the NG-RAN node needs to establish shared delivery for the location dependent contents of a multicast MBS session if it serves at least one UE assigned to an MBS Session ID and Area Session ID.

2.	The NG-RAN sends an N2 MBS Session request message (MBS Session ID, [Area Session ID], N2 SM information ([unicast DL tunnel Info])) towards the AMF.

If the NG-RAN node is configured to use unicast transport for the shared delivery, it allocates a GTP tunnel endpoint and provides the unicast DL tunnel Info in the request, which includes the GTP tunnel endpoint and NG-RAN node address. For location dependent MBS services, the NG-RAN node also provides the Area Session ID.

3.	The AMF selects the MB-SMF serving the multicast MBS session, e.g. using the NRF discovery service or locally stored information. It invokes Nmbsmf_MBSSession_ContextUpdate request (MBS Session ID, [Area Session ID], N2 SM information, NG-RAN Node ID) to the MB-SMF.

The AMF stores the information of the NG-RAN nodes (e.g. NG-RAN node ID) for the subsequent signaling related to the multicast MBS Session.

4.	[Conditional] When the MB-SMF receives the unicast DL tunnel Info, it checks the stored DL GTP tunnel endpoint to avoid establishing an already existing tunnel.

If the  shared delivery tunnel is not established, i.e. the DL tunnel endpoint has not been stored before, it configures the MB-UPF to send multicast data for the multicast MBS session (or location dependent content of the multicast MBS session if an Area Session ID was received) towards that GTP tunnel endpoint via unicast transport.

If the shared NG-U termination is established, the MB-SMF only stores the received NG-RAN Node ID and DL GTP tunnel endpoint.

5.	The MB-SMF stores the information of the AMF (e.g. AMF ID) in the MBS Multicast MBS session context (or location dependent part of the Multicast MBS Session Context if an Area Session ID was received) to enable subsequent signalling towards that AMF.

6.	The MB-SMF sends Nmbsmf_MBSSession_ContextUpdate response (MBS Session ID, [Area Session ID], N2 SM information ([TMGI], multicast QoS flow information, session state (Active/Inactive), [multicast DL tunnel Info], [MBS service areas])) to the AMF. If the MB-SMF did not receive unicast DL tunnel Info in step 3, it provides the multicast DL tunnel info that includes transport multicast address (e.g. a LL SSM) and a GTP tunnel endpoint for multicast transport of the shared delivery.

7.	The AMF sends an N2 MBS Session response message (MBS Session ID, [Area Session ID], N2 SM information) to the NG-RAN node. If the NG-RAN node receives the multicast DL tunnel Info of the shared delivery, it uses the transport multicast address included in the multicast DL tunnel info to join the multicast transport distribution.

Next CHANGE

#### 7.2.2.4	Release of shared delivery toward RAN node

In the following case, the shared delivery tunnel may be released between NG-RAN and MB-UPF:

-	The last UE is excluded from the context of the multicast MBS session in the NG-RAN node;

-	Handover to the target NG-RAN when the UE is the last UE for this multicast MBS session in the source NG-RAN node during handover preparation phase known by the source NG-RAN node;

-	Handover to the target E-UTRAN when the UE is the last UE for this multicast MBS session in the source NG-RAN node;

-	MBS session deletion.

NOTE:	When the multicast MBS session is inactive, the shared delivery is not released if there is at least one UE is in RRC-CONNECTED state for this multicast MBS session.

Figure 7.2.2.4-1: Release of shared delivery toward RAN node

1.	A RAN node decides to release shared delivery for a multicast MBS session, e.g. because it no longer serves at least one UE within the multicast MBS session. For location dependent services, the NG-RAN node may release shared delivery for the location dependent contents of a multicast MBS session if it no longer serves at least one UE assigned to an MBS Session ID and Area Session ID.

2.	The NG-RAN node sends N2 MBS Session release request (MBS Session ID, [Area Session ID], [N2 SM information ([GTP tunnel info], release indication)])]) to the AMF. For location dependent services, the NG-RAN node also provides the Area Session ID. The RAN node includes the unicast DL tunnel info if unicast transport is used for the shared delivery. If the NG-RAN node was configured to use multicast transport for the 5GC Shared MBS traffic delivery, the NG-RAN node does not include the N2 SM information in the message.

3.	[Conditional] If the N2 SM information is received or it is the last RAN node controlled by the AMF serving the multicast MBS session, the AMF invokes Nmbsmf_MBSSession_ContextUpdate request (MBS Session ID [Area Session ID], [leave indication], [N2 SM information], NG-RAN Node ID) to the MB-SMF corresponding to the MB-SMF ID stored in the AMF for the MBS Session ID. If it is the last RAN node controlled by the AMF serving the multicast MBS session identified by the MBS Session ID or both the MBS Session ID and Area Session ID (if exists), the leave indication is included.

4.	[Conditional] If unicast transport was used towards the NG-RAN node, the MB-SMF determines whether the context update is for tunnel release or create based on the release indication in the N2 container. If the MB-SMF determines the context update is for tunnel release, the MB-SMF removes the received NG-RAN Node ID from the stored NG-RAN Node ID and DL GTP tunnel endpoint pair(s) for the MBS session ID, and checks whether other NG-RAN Node ID(s) use the related DL GTP tunnel.

If the MB-SMF finds the related DL GTP tunnel is not in used, the MB-SMF sends N4mb Session Modification to the MB-UPF to release the N3mb tunnel used for the multicast MBS session (or location dependent content of the multicast MBS session if an Area Session ID was received) towards that NG-RAN node using the received GTP tunnel info.

5.	[Conditional] The MB-SMF responses to the AMF with MBS Session ID and Area Session ID if received. If leave indication is received, the MB-SMF also removes the information of the AMF from the context of the multicast MBS session.

6.	The AMF removes the information of the RAN node from the context of the multicast MBS session (or location dependent part of the multicast MBS session if an Area Session ID was received).

7.	The AMF sends an N2 MBS Session release response (MBS Session ID, [Area Session ID]) to the RAN node. The NG-RAN node deletes the GTP tunnel info (if unicast transport is used for the shared delivery) or sends IGMP/MLD leave message to leave the multicast distribution tree (if multicast transport is used for the shared delivery). The NG-RAN node releases local resources for the multicast MBS session.

END OF CHANGES