SA WG2 Meeting #153E (e-meeting)	S2-2209000r12

October 10 – 14, 2022, Elbonia	(revision of S2-220xxxx)

Source:	NEC, LGE, Lenovo, Nokia,

Title:	KI#2: Conclusions

Document for:	Approval

Agenda Item:	9.14 Study on Network Slicing Phase 3

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract of the contribution: This document captures conclusions for this key issue#2 based on the inputs on basic procedure from various companies.

# Conclusion document for KI#2

## 1. Key Issue #2 Background

## 5.2 Key Issue #2: Support of providing VPLMN network slice information to a roaming UE ### 5.2.1 Description As an outcome of SA WG1 EASNS (Enhanced Access to and Support of Network Slice) work, clause 6.1.2.1 of TS 22.261 [4] captures the following service requirement for a roaming UE. For a roaming UE activating a service/application requiring a network slice not offered by the serving network but available in the area from other network(s), the HPLMN shall be able to provide the UE with prioritization information of the VPLMNs with which the UE may register for the network slice. This key issue aims at addressing the following aspects for a roaming UE requiring a network slice not offered by higher priority VPLMN(s) but available from other network(s): - Study how and when the HPLMN provides the UE with information about slice availability per VPLMN and prioritization information of the VPLMNs with which the UE may register for the network slice. The study includes the content of the information. - Study how and when to use the information received by the UE from the HPLMN to influence automatic PLMN selection. NOTE 1: For details on PLMN selection aspects and impacts on PLMN selection of solutions of this key issue, coordination with CT1 is needed. NOTE 2: Impacts to manual selection is not in scope of the key issue. This key issue only considers the network selection procedure for the 3GPP access type. NOTE 3: Coordination with the Study Item FS_5WWC_Ph2 is required regarding the VPLMN selection procedure when non-3GPP access type is used.

Solutions; Key Issues. KI#1; KI#2; KI#3; KI#4; KI#5; KI#6. Solution #6: Extended SoR VPLMN Slice Information transfer to UEs; X. Solution #7: Enabling awareness of Network Slice availability in VPLMNs; X. Solution #16: UE assisted slice based VPLMN prioritization for Extended SoR; X. Solution #17: Slice based VPLMN Selection Policy; X. Solution #18: Sending rejected NSSAI to the UDM to assist the UDM to steer the UE to the PLMN supporting rejected NSSAI; X. Solution #19: configuring the UE with network slice aware preferred PLMNs lists; X. Solution #20: VPLMN Selection following existing SoR information; X.

Note: This various aspects of the slice based SoR procedure has been discussed among the companies and the discussion is captured in the S2-2209001

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-41

# * * * * First change * * * *

## 8.X	Conclusions for KI#2

## The following principles are concluded for KI#2.

1.	A slice based SoR mechanism to deliver enhanced slice-aware SoR information will reuse the current SoR mechanism defined in TS 23.122 [7] for SoR information delivery. The encoding of the enhanced slice-aware SoR information is in the CT1 remit.

2.	The SoR container (which is used also to carry the enhanced slice-aware SoR information) from the UDM to the UE is security protected.

NOTE 1: SA3 may further define any upgrade of security protection mechanism of the SoR container, if it was needed.

3.	UDM requires knowing the support of the enhanced SoR information by theUE to deliver the enhanced slice-aware SoR information to the UE.

NOTE 2:	Whether the UE provides additional assistance information n) in addition to the UE capabilityand which kind of additional assistance information need to be discussed in CT1. Any UE additional assistance information is transparently forwarded by UDM to SoR-AF during the triggering procedure by UDM. The SoR-AF should not attempt to fetch any assistance information if not provided by the UE.

4.	Only a UE supporting slice based SoR feature can receive the enhanced slice-aware SoR information via UDM, the enhanced slice aware information include preferred PLMNs for specific S-NSSAIs in the UE subscription (a prefreed PLMN list may be also be a single PLMN that is known by HPLMN to support the S-NSSAI, or a list of PLMNs in preference order that differs from the order of the basic SoR information that is also provided). when more than one S-NSSAI has slice aware information and all these S-NSSAIs are needed by the Ue, a weighted approach to preferred PLMN selection is proposed.

5.	The UE will perform the PLMN selection based on the received enhanced slice-aware SoR information.

6.	As for the current SoR information, It shall be possible for the HPLMN to update the enhanced slice-aware SoR information  when it is required by HPLMN, e.g., change in the UE subscription or other HPLMN trigger.

7.	it is assumed that  the SoR AFcan already today take into account Subscribed S-NSSAIs of the UE. the SoR AF can get already today Subscribed S-NSSAIs using existing UDM services. This can also be used to generate enhanced slice-aware SoR information