SA WG2 Meeting #152e	S2-2209087r02

# 17 - 26 August 2022, Electronic meeting         		            (revision of S2-2206759)

Source: 	Lenovo, Motorola Mobility, Apple, Broadcom, Tencent, Nokia, Nokia Shanghai Bell, Ericsson, ZTE, CableLabs, LG Electronics, Tejas Networks, Meta USA, Google, Charter Communications

Title: 	Updates to solution #2.2: MPQUIC steering functionality using UDP proxying over HTTP

Document for: 	Approval

Agenda Item: 	9.3

Work Item / Release:	FS_ATSSS_Ph3 / Rel-18

Abstract of the contribution:

# Discussion

The following changes and updates are made to Solution 2.2:

-	The references of the solution are updates since several Internet drafts have been published as standards-track RFCs:

-	draft-ietf-masque-connect-udp: "UDP Proxying Support for HTTP"  Published as RFC 9298

-	draft-ietf-quic-http: "Hypertext Transfer Protocol Version 3 (HTTP/3)"  Published as RFC 9114

-	draft-ietf-masque-h3-datagram: "Using Datagrams with HTTP"  Published as RFC 9297

-	draft-ietf-httpbis-h3-websockets: "Bootstrapping WebSockets with HTTP/3"  Published as RFC 9220

-	The clause 6.11.4 "User-plane overhead" is added.

-	The clause 6.11.5 "Co-existence with MPTCP and ATSSS-LL" is added.

-	The clause 6.11.6 "Handling of out-of-order delivery" is added.

-	The clause 6.11.7 "Handling of duplicated packets" is added.

-	The clause 6.11.8 "Impacts on Existing Nodes and Functionality" is completed.

In addition, the following clarifications are made:

a.	The MPQUIC solution supports 3 transport modes, 2 of them supporting packet re-ordering (stream and datagram mode 1). As clarified in clause 6.11.6, the Datagram mode 1 appends sequence numbers to UDP data packets (see details in clause 6.11.3, step 5) and requires the definition of a new Context Id. This Context Id, as well as the following aspects of Datagram mode 1 will be considered in stage-3.

-	How to ensure re-ordering on the receiver side is implemented and activated, when needed.

-	How the same reordering in both downlink and uplink can be provided.

-	Which sequencing and reordering schemes can be supported and whether such schemes will be specified or will be left to UE/UPF implementation.

b.	The Datagram mode 2 is required (although it does not support re-ordering) because the MPQUIC steering functionality (as any other ATSSS steering functionality) must support all steering modes and not only steering modes that require packet reordering / deduplication. In addition, the Datagram mode 2 features benefits compared to ATSSS-LL, e.g., (a) it supports congestion control, (b) it supports RTT/PLR measurements without the need to implement the PMF protocol, (c) it is implemented in high-layers so it can interact with applications, etc.

Start of Changes

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TR 23.700-93: "Study on access traffic steering, switch and splitting support in the 5G System (5GS) architecture; Phase 2".

[6]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport".

[7]	IETF RFC 9001: "Using TLS to Secure QUIC".

[8]	IETF RFC 9002: "QUIC Loss Detection and Congestion Control".

[9]	IETF RFC 9221: "An Unreliable Datagram Extension to QUIC".

[10]	draft-ietf-quic-multipath: "Multipath Extension for QUIC".

Editor's note: The above document cannot be formally referenced until it is published as an RFC.

[…]

[27]	IETF RFC 9298: "Proxying UDP in HTTP".

[28]	IETF RFC 9114: "Hypertext Transfer Protocol Version 3 (HTTP/3)".

[29]	IETF RFC 9297: "HTTP Datagrams and the Capsule Protocol".

[30]	IETF RFC 9220: "Bootstrapping WebSockets with HTTP/3".

[31]	draft-ietf-masque-connect-ip: "IP Proxying Support for HTTP".

Editor's note: The above document cannot be formally referenced until it is published as an RFC.

[32]	IETF RFC 5225: "RObust Header Compression Version 2 (ROHCv2): Profiles for RTP, UDP, IP, ESP and UDP-Lite".

[33]	IETF RFC 6846: "RObust Header Compression (ROHC): A Profile for TCP/IP (ROHC-TCP)".

[34]	3GPP TS 38.323: "Packet Data Convergence Protocol (PDCP) specification".

[35]	IETF RFC 4555: "IKEv2 Mobility and Multihoming Protocol (MOBIKE)".

Next Change

## 6.11	Solution #2.2: MPQUIC steering functionality using UDP proxying over HTTP

### 6.11.1	Introduction

The solution in clause 6.11 specifies a new ATSSS steering functionality, called Multipath QUIC (MPQUIC) steering functionality, and addresses the objective of KI#2 for a QUIC-based steering functionality. It borrows several aspects from solutions studied in TR 23.700-93 [5], mainly from Solution #6 "MPQUIC-LL Steering Functionality" and from Solution #14 "Proxy based solution using MP-QUIC".

The solution is primarily based on RFC 9298 "proxying UDP in HTTP" [27], which specifies how UDP traffic can be transferred between a client (UE) and a proxy (UPF) using the HTTP/3 protocol [28]. The HTTP/3 protocol operates on top of the QUIC protocol [6], which supports simultaneous communication over multiple paths, as defined in draft-ietf-quic-multipath [10].

The solution supports the following transport modes for transmitting a UDP flow between UE and UPF (see further details in clause 6.11.3, step 5):

-	Datagram mode 2: This transport mode is the mode already supported in RFC 9298 [27]. It encapsulates UDP packets within QUIC Datagram frames and provides unreliable transport with no sequence numbering and no packet reordering / deduplication. It can be applied for UDP flows where the application is robust against out-of-order delivery and jitter (e.g., because it can perform itself appropriate packet reordering / deduplication).

-	Datagram mode 1: This transport mode is an extension of the mode supported in RFC 9298 [27]. It encapsulates UDP packets within QUIC Datagram frames and provides unreliable transport but with sequence numbering and with packet reordering / deduplication. It can be applied for UDP flows where the application does own reliability mechanism (i.e. re-transmissions) or  it is unknown whether the application  does own reliability mechanism (re-transmissions). The datagram mode 1 requires the definition of a new Context ID (see RFC 9298), which can be done in stage-3. Also, additional details (e.g., the algorithms for packet re-ordering) may also be considered in stage-3.

-	Stream mode: This transport mode is readily supported by the QUIC protocol. It encapsulates UDP packets within QUIC Stream frames and provides reliable transport with sequence numbering and with packet reordering / deduplication. It can be applied for UDP flows where it is known that the application does not perform retransmissions.

The PCF selects which of the above transport modes shall be applied for a UDP flow (SDF). The selected transport mode is provided to UE and UPF within the ATSSS rules and N4/MAR rules respectively.

NOTE 1:	When the Datagram mode 2 is used with steering modes that use both accesses simultaneously, e.g. the Load-Balancing steering mode, traffic scheduling with per-packet splitting can result to excessive out-of-order delivery.

NOTE 2:	The Stream mode provides strict reliability and in-order delivery with re-transmissions and therefore can lead to melt down phenomena [see http://sites.inka.de/~W1011/devel/tcp-tcp.html] when reliable traffic (e.g. QUIC) is carried, or counteracts application decisions when UDP is selected to avoid reliability and/or in-order delivery. Therefore, it should be avoided for applications which perform their own reliability mechanisms.

The Datagram mode 1 and the Stream mode can be used for supporting per-packet splitting, i.e., when the packets of a UDP flow are split across multiple accesses and may be received out-of-order. The Datagram mode 2 is a transport mode which can be applied for applications that can tolerate packet reordering or duplicated packets. The Datagram mode 2 is required (although it does not support re-ordering) because the MPQUIC steering functionality (as any other ATSSS steering functionality) must support all steering modes and not only steering modes that require packet reordering / deduplication. The Datagram mode 2 features compared to ATSSS-LL, supports congestion control, RTT/PLR measurements without the need to implement the PMF protocol, and it is implemented in high-layers so it can interact with applications. However, it also adds additional overhead and encryption.

### 6.11.2	High-level Description

The key principles of the solution are summarized below.

-	After the MA PDU Session establishment, the UE creates one or more multipath QUIC connections with the UPF. Each multipath QUIC connection is associated with a QoS flow, i.e., it carries the traffic mapped to a QoS flow.

-	The UE operates as a connect-udp client and the UPF operates as a connect-udp proxy, both defined in RFC 9298 [27]. Therefore, the UE supports an HTTP/3 client and the UPF supports an HTTP/3 proxy, both of them operating over QUIC.

-	When the UE wants to transmit the first uplink packet of a new UDP flow, the UE:

-	Selects which QUIC connection will be used for the uplink traffic of the UDP flow based on the QoS flow associated with the UDP flow;

-	Creates a new bidirectional QUIC stream on the selected QUIC connection;

-	Configures the QUIC stream to apply a steering mode (i.e., the steering mode that should be used for the uplink traffic of the UDP flow based on the ATSSS rules);

-	Sends to UPF, via the QUIC stream, an extended HTTP CONNECT request [27] containing 'path' information that identifies a destination address and port for the UDP flow (i.e., identifies the remote host where the UDP flow should be forwarded to); and

-	Forwards to UPF the uplink packets of the UDP flow using multipath QUIC transport.

-	When the UPF wants to transmit a downlink packet of a UDP flow, the UPF:

-	Selects which QUIC connection will be used for the downlink traffic of the UDP flow based on the QoS flow associated with the UDP flow (this QUIC connection is the same as the one selected by UE for the UDP flow, assuming the QoS flow in UL and DL directions is the same);

-	Selects a bidirectional QUIC stream on the selected QUIC connection (this QUIC stream is the same as the one created by the UE for the UDP flow);

-	Configures the QUIC stream to apply a steering mode (i.e., the steering mode that should be used for the downlink traffic of the UDP flow based on the N4 rules); and

-	Forwards to UE the downlink packets of the UDP flow using multipath QUIC transport.

The following figure illustrates how the traffic of a UDP flow is transferred between the UE and UPF using multipath QUIC transport. In this figure, it is assumed that the uplink traffic and the downlink traffic of the UDP flow are mapped to the same QoS flow (QFI-1). Therefore, both the uplink traffic and the downlink traffic of the UDP flow use the same multipath QUIC connection.

Figure 6.11.2-1: Using multipath QUIC transport for a UDP flow

The bidirectional QUIC stream is established by the UE to enable transmission of uplink data packets (blue) and downlink data packets (red) of the UDP flow. The UE configures this stream to send uplink traffic with a steering mode determined based on the ATSSS rules in the UE (uplink steering mode). The UPF configures this stream to send downlink traffic with a steering mode determined based on the N4 rules in the UPF (downlink steering mode). The data packets of the UDP flow shown in Figure 6.11.2-1 are transmitted in datagram mode (mode 1 or mode 2), i.e., they are encapsulated in HTTP datagrams and in QUIC DATAGRAM frames, each one carrying header information (see the Quarter Stream ID defined in RFC 9297 [29]) that associates it with the established bidirectional stream. As discussed below, the data packets of the UDP flow may be transmitted in stream mode (instead of datagram mode), i.e., transmitted directly over the bidirectional stream. In this case, the data packets of the UDP flow are encapsulated in DATAGRAM capsules (see [29]) and in QUIC STREAM frames.

Figure 6.11.2-2 and Figure 6.11.2-3 illustrate the components of the MPQUIC steering functionality used to support data transmission in the uplink and downlink direction respectively. The MPQUIC steering functionality is composed of three components:

1)	QoS flow selection & Steering mode selection: This component in the UE initiates the establishment of one or more QUIC connections, after the establishment of the MA PDU Session and, for each uplink UDP flow, it selects a QoS flow (based on the QoS rules), and a steering mode and a transport mode (based on the ATSSS rules). This component in the UPF selects, for each downlink UDP flow, a QoS flow (based on the N4 rules), a steering mode and a transport mode (based on the N4 rules).

In the UE, this component is only used in the uplink direction, while, in the UPF, this component is only used in the downlink direction.

2)	HTTP/3 layer: Supports the HTTP/3 protocol defined in RFC 9114 [28] and the extensions defined in:

-	RFC 9298 [27] for supporting UDP proxying over HTTP;

-	RFC 9297 [29] for supporting HTTP datagrams; and

-	RFC 9220 [30] for supporting Extended CONNECT.

The HTTP/3 layer selects a QUIC connection to be used for each UDP flow and allocates a new QUIC stream on this connection that is associated with the UDP flow. It also configures this QUIC stream to apply a specific steering mode.

In the UE, the HTTP/3 layer implements an HTTP/3 client, while, in the UPF, it implements an HTTP/3 proxy.

3)	QUIC layer: Supports the QUIC protocol as defined in the applicable IETF specifications (RFC 9000 [6], RFC 9001 [7], RFC 9002 [8]) and the extensions defined in:

-	RFC 9221 [9] for supporting unreliable datagram transport with QUIC; and

-	draft-ietf-quic-multipath [10] for supporting QUIC connections using multiple paths simultaneously.

Figure 6.11.2-2: Components of MPQUIC steering functionality used for UL data transmission

Figure 6.11.2-3: Components of MPQUIC steering functionality used for DL data transmission

The protocol stack of the solution is depicted in Figure 6.11.2-4 below.

Figure 6.11.2-4: UP protocol stack of the solution

### 6.11.3	Procedures

Figure 6.11.3-1 below depicts the key steps of the procedure that enables data traffic to be exchanged between the UE and UPF using the MPQUIC steering functionality. For simplicity, the components of the MPQUIC steering functionality at the UPF are not depicted.

Figure 6.11.3-1: Procedure for enabling data traffic using the MPQUIC steering functionality

1.	The UE establishes a MA PDU Session with the 5G core (5GC) network. During the MA PDU Session establishment:

-	In the PDU Establishment Request message, the UE indicates that it supports the MPQUIC steering functionality. This indication can be used by the network (a) to select a UPF that supports the MPQUIC steering functionality and (b) to decide whether the ATSSS/N4 rules for the MA PDU Session may use the MPQUIC steering functionality.

-	The UE receives MPQUIC proxy information, i.e., one IP address of UPF, one UDP port number and the proxy type (e.g., "connect-udp"). This information is used by the UE for establishing QUIC connections with the UPF, which is also referred to as "MPQUIC proxy".

-	The UE receives one IP address/prefix for the MA PDU Session and two additional IP addresses/prefixes, called "link-specific multipath QUIC" addresses; one associated with 3GPP access and another associated with non-3GPP access. These two addresses can be used by the UE to create two paths in a multipath QUIC connection.

-	The UE receives QoS rules and ATSSS rules to be applied for the MA PDU Session, for QoS enforcement and traffic steering enforcement respectively. Similar rules (N4 rules) are received by UPF.

2.	After the MA PDU Session is established and the UE identifies that one or more ATSSS rules require traffic steering using the MPQUIC steering functionality, the UE determines the number of multipath QUIC connections to be established with the UPF (MPQUIC proxy). For example, the UE determines to establish as many multipath QUIC connections, as the number of QoS flows of the MA PDU Session, i.e., one multipath QUIC connection per QoS flow. This way, every QUIC packet can carry only traffic (e.g., one or more UDP payloads) that belongs to the same QoS flow, which facilitates QoS handling in the access layer.

The QoS rules provided to UE include downlink QoS information and the UE applies the downlink QoS information to establish QUIC connections for the QoS flows used for downlink traffic only.

3.	The UE establishes the number of multipath QUIC connection with the UPF (MPQUIC proxy) determined in the previous step. This results into several multipath QUIC connections between the UE and UPF, each one composed of multiple paths, e.g., one path over 3GPP access and another path over non-3GPP access.

NOTE 1:	Data transmitted over a multipath QUIC connection must be encrypted according to RFC 9001 [7]. However, encryption might not be necessary when the multipath QUIC connection is established between UE and UPF, because the underlying 5G security mechanisms can be applied.

Editor's note: Whether and how encryption in the QUIC layer can be omitted is FFS and need to be studied by SA WG3. It could be studied, for example, whether RFC 9150 "TLS 1.3 Authentication and Integrity-Only Cipher Suites” can be applied to omit the data encryption. The impact due to encryption regarding overhead and performance is FFS in SA3.

During a QUIC connection establishment, the UE and UPF negotiate QUIC transport parameters and indicate (a) support of QUIC Datagram frames and (b) support of multipath. They indicate support of QUIC Datagram frames by providing the "max_datagram_frame_size" transport parameter with a non-zero value (see RFC 9221 [9]) and they indicate support of multipath by providing the "enable_multipath" transport parameter (see draft-ietf-quic-multipath [10]).

After a QUIC connection establishment, the HTTP/3 client and the HTTP/3 proxy negotiate HTTP settings and indicate support of HTTP Datagrams (see RFC 9297 [29]) and support of Extended CONNECT (see RFC 9220 [30]).

The QoS flow associated with a QUIC connection is also negotiated between the UE and UPF. This is done by using a new QUIC transport parameter (defined by 3GPP) when the QUIC connection is established.

NOTE 2: The new QUIC transport parameter needs to be registered in IANA (by stage 3).

4.	An app in the UE generates a new data packet (also referred to as "UDP payload") that should be sent via the MA PDU Session. This data packet initiates a new UDP flow, i.e., a sequence of data packets using the same 5-tuple. In the example shown in Figure 6.11.3-1, the data packet should be sent to IP address 144.23.1.47 and to UDP port 556.

5.	For the new UDP flow (and for each new UDP flow):

-	The UE selects a QoS flow (QFI) over which the UDP flow should be transmitted. This is selected by using the received QoS rules.

-	The UE selects a steering mode that should be applied for the UDP flow. This is selected by using the received ATSSS rules.

-	The UE selects a transport mode that should be applied for the UDP flow. This is selected by using the received ATSSS rules, i.e., each ATSSS rule which indicates that the MPQUIC steering functionality should be applied for the matching traffic, indicates also the transport mode that should be applied for this traffic.

As specified in clause 6.11.1, three transport modes are supported (Datagram mode 1, Datagram mode 2 and Stream mode) and the transport mode that should be applied for a UDP flow is selected by PCF and is provided to UE and UPF within the ATSSS rules and the N4/MAR rules respectively.

-	The Datagram mode 2 is already specified in RFC 9298 [27] and does not require further specification in 3GPP. In this mode, the HTTP/3 proxy/client encapsulates each UDP data within an HTTP Datagram that contains a Context Id=0. Then, the HTTP Datagram is sent to the QUIC layer for multipath transmission. No sequence numbers are included in the HTTP Datagrams (see [27]).

-	The Datagram mode 1 is similar to the Datagram mode 2 but it also provides a sequence number for each UDP data. In this mode, the HTTP/3 proxy/client encapsulates each UDP data within an HTTP Datagram that contains a Context Id and a sequence number. Then, the HTTP Datagram is sent to the QUIC layer for multipath transmission. The value of this Context Id will be defined in stage-3 and indicates that a sequence number is also included. The sequence numbers are used by the receiving endpoint to re-order the UDP data and remove duplicated UDP data. Details of how packet reordering is supported (including the mechanisms for reordering) can be specified in stage-3.

The Datagram mode 1 and Datagram mode 2 use two different (and pre-defined in 3GPP) Context IDs, e.g., Context ID=0 and Context ID=1, respectively. With Context ID=0, the HTTP Datagram Payload contains the UDP data, whereas, with Context ID=1, the HTTP Datagram Payload contains a sequence number followed by the UDP data. The format of QUIC DATAGRAMs used in both datagram modes is shown below. As specified in clause 6.11.6, the Datagram mode 1 supports reordering in the HTTP/3 layer (as described above).

The following figure shows how UDP data is encapsulated in a QUIC datagram using the Datagram mode 1 and the Datagram mode 2. It is assumed that the value of Context Id used in Datagram mode 1 is set to 1.

| Datagram mode 1 |  |
| --- | --- |
| Datagram mode 2 |  |

6.	The UE selects a multipath QUIC connection to be used for the new UDP flow (e.g., based on the selected QFI) and the UE allocates a new bidirectional QUIC stream (e.g., stream 40) in this multipath QUIC connection. This new stream is associated with the new UDP flow. The UE configures the new stream to transmit uplink data traffic using the selected steering mode for this UDP flow.

7-9. The UE sends the data packet using the allocated new stream on the selected QUIC connection.

When the datagram transport mode is selected (either mode 1 or mode 2), the UE encapsulates the data packet within an HTTP DATAGRAM frame [27], which is transferred inside a QUIC DATAGRAM frame [9]. The header of the HTTP DATAGRAM indicates that this datagram is associated with stream 40 (i.e., the Quarter Stream ID is set to 10).

When the stream transport mode is selected, the UE encapsulates the data packet within an HTTP DATAGRAM frame [29] that is further encapsulated in a DATAGRAM capsule (see clause 3.5 in [29]), which is transferred inside a QUIC STREAM frame [6].

In the example procedure shown in Figure 6.11.3-1, the datagram transport mode (either mode 1 or mode 2) is selected. The UE sends an HTTP CONNECT request via the allocated stream 40, which indicates to UPF (MPQUIC proxy) that the UE wishes to create an HTTP tunnel to send UDP traffic to remote host 144.23.1.47 and port 556. The :protocol pseudo-header is set to "connect-udp" (as defined in [27]) to indicate that the protocol to be spoken on the tunnel is the capsule protocol defined in RFC 9297 [29]. Finally, the UE sends a QUIC DATAGRAM frame to UPF that encapsulates the data packet, which is forwarded to the remote host (144.23.1.47:556). Note that the QUIC DATAGRAM frame may contain a sequence number (as defined in step 5).

If the stream transport mode were selected, the HTTP DATA frame in step 8 would contain a DATAGRAM capsule, which would include the data packet in the HTTP Datagram Payload field (see [29]).

10.	The UPF (MPQUIC proxy) responds with an HTTP 200 status, indicating that the request to proxy data packets to a remote host 144.23.1.47 and destination port 556 is accepted.

11.	When a data packet is received by UPF (MPQUIC proxy) from the remote host (data packet #2), this data packet is transferred to the UE using the established context information for the UDP flow, i.e., using the selected multipath QUIC connection, the selected stream on this connection, the selected steering mode, and the selected transport mode. Such context information is stored in the UPF and in the UE and is applied for all the data packets of the UDP flow.

12.	Similarly, when another data packet is generated by the UE app (data packet #3), this data packet is transferred to UPF (MPQUIC proxy) using again all the stored context information for the UDP flow.

NOTE 2:	The context information for a UDP flow in the UE and in the UPF is created when the first data packet (i.e., Data packet #1) of this UDP flow is transferred. All subsequent data packets of the same UDP flow are transferred between the UE and the UPF using this context information.

When the UE identifies that the context information for a UDP flow is no longer needed (e.g., the app closes the associated UDP socket), the UE deletes this context information and releases the associated QUIC stream, which cause the UPF to delete the context information stored in UPF.

The following figure 6.11.3-2 illustrates an example of how the uplink traffic of various UDP flows is transferred from the UE to UPF using QUIC multipath transport, and how the UPF relays this traffic to a final destination (remote host). Note that, for each UDP flow, there is an associated QUIC connection and an associated bidirectional QUIC stream, which is configured to apply a specific steering mode for the uplink traffic. For example, the red UDP flow is associated with the multipath QUIC connection #1 and with the red Stream Y, which is configured to apply the Smallest-delay steering mode in the uplink direction.

All UDP flows shown in this figure, except the blue UDP flow, are transferred with the datagram transport mode (mode 1 or mode 2), so their data packets are transferred inside QUIC DATAGRAM frames. The blue UDP flow is transferred with the stream transport mode, so its data packets are transferred inside QUIC STREAM frames.

The downlink traffic of UDP flows is transferred between the UE and UPF in a similar way.

Figure 6.11.3-2: Example of user-plane operation using the MPQUIC steering functionality (UL direction)

#### 6.11.4	User-plane overhead

As shown in the figure 6.11.3-2above and table1 below, the user-plane overhead of the MPQUIC steering functionality is given by (a) a proxying mechanism (instead of IP-in-IP tunneling) and (b) the HTTP/3 encapsulation and (c) the QUIC protocol which can multiplex payload packets into a single QUIC packet as explained later in this section. This proxying mechanism "allows an HTTP client to create a tunnel for UDP communications through an HTTP server that acts as a proxy" (see RFC 9298 [27]).

| Transport mode | Additional Header size [Bytes] | Comment |
| --- | --- | --- |
| MP-QUIC UDP Datagram mode 1 | 46 – 67 average 55 | Datagram Length, Quarter Stream ID and Context ID are variable length encoded fields varying between 1, 2, 4, or 8 Bytes. They determine the range, which starts from 46 Byte and in an average 55 Byte. |
| MP-QUIC UDP Datagram mode 2 | 38-52 average 46 | As Datagram mode 1 but without Sequence number |
| MP-QUIC UDP Stream mode | 33-68 average 48 | As Datagram mode 1 but with more variable length encoded fields. An average value of 48 Byte overhead is assumed to be likely. |

Table 1: User-plane overhead introduced by protocol headers.

The additional header shown in table 1 is calculated from following documents:

- MP-QUIC solution #2.2

- QUIC: [6]

- MP-QUIC: [10]

- QUIC Datagram: [9]

- Connect-UDP: [27]

- HTTP/3-Datagram: [29]

According to these documents, the solution specific headers which are introduced compose high-level of:

- MP-QUIC UDP Datagram mode:

UDP + QUIC + MP extension + QUIC DATAGRAM + HTTP Datagram

- MP-QUIC UDP Stream mode:

UDP + QUIC + MP extension + STREAM Frame + HTTP Datagram

Derived from that, the size is calculated for each below:

- Datagram mode:

UDP|QUIC Short   |DATAGRAM      |HTTP Datagram    |HTTP Info

|1-RTT Packet |Type [Length] |Quarter Stream ID|Context ID Seq No.

8  |26           |1    {1,2,4,8}|{1,2,4,8}        |{1,2,4,8}  8?      = 46/67 Bytes

QUIC Short: [6]

DATAGRAM: [9]

HTTP DATAGRAM: [29]

HTTP Info Context ID: [27]

HTTP Info Seq. No.: Solution specific

Datagram Length parameter is required to support multiplexing capability

Stream ID is required for differentiating QoS flows and steering mode within one MP-QUIC connection

MP extension has no impact to the header size as it only impacts the meaning of the RFC9000 defined packet number.

Seq. No. is assumed to be 8 Byte, however it is not specified so far.

Length, Quarter Stream ID and Context ID are variable length encoded fields varying between 1, 2, 4, or 8 Bytes. In the case of Datagram mode 2, Context ID is a 1 Byte value.

- Stream mode:

In case MP-QUIC Stream mode is applied, the HTTP Datagram is capsuled in a Datagram Capsule element which itself is part of a STREAM Frame. Compared to Datagram modes, HTTP Info field is not required as ContextID and Seq. No. are not required.

UDP|QUIC Short  |STREAM Frame                    |Datagram Capsule|HTTP Datagram

|1-RTT Packet|Type Stream ID [Offset] [Length]|Type, Length    |Quarter Stream ID

8  |26           1    {1,2,4,8} {1,2,4,8}{1,2,4,8}|1    {1,2,4,8}  {1,2,4,8} = 41/76 Bytes

QUIC Short: [6]

STREAM Frame: [6]

Datagram Capsule: [29]

HTTP DATAGRAM: [29]

STREAM FRAME Length parameter is required to support multiplexing capability

STREAM FRAME Offset parameter is required to keep send order

The figure 6.11.4-1 below shows that, when the app provides two payloads in succession, the MPQUIC steering functionality can multiplex these payloads in the same QUIC packet (this is a feature supported by QUIC), thus, reducing the user-plane overhead (by 34Byte in this example). In addition, a single UDP datagram can multiplex multiple QUIC packets, which can further reduce the overhead.

NOTE: As a consequence of RFC9000 [6] the precondition for possible multiplexing of payload packets is that those payload packets fit completely inside a single QUIC packet, which is limited by UDP datagram and max. MTU size, typically 1200 Byte.

An overhead of the MPQUIC steering functionality, which is not evident from the figure below, is the HTTP CONNECT request plus the 200 OK response, sent at the beginning of every UDP flow (see steps 7 and 10 in Figure 6.11.3-1)..

Figure 6.11.4-1: Example of multiplexing QUIC datagram frames in one QUIC packet when using the MPQUIC steering functionality (UL direction)

### 6.11.5	Co-existence with MPTCP and ATSSS-LL

The MPQUIC steering functionality can co-exist with the MPTCP and the ATSSS-LL steering functionalities and can complement the capabilities supported by these functionalities. As shown in the following figure, the MPQUIC steering functionality and the MPTCP steering functionality coexist and complement each other by applying the same principles for different types of flows (UDP/IP traffic flows and TCP/IP traffic flows respectively):

-	They both support a proxy functionality in the UPF. During the MA PDU Session establishment, the UE receives proxy information for MPQUIC (IP address, UDP port and type) and, in a similar way, it receives proxy information for MPTCP (IP address, TCP port and type).

-	They are both high-layer functionalities that can interact with apps via multipath-aware APIs.

-	The MPTCP establishes a new MP-TCP connection for every TCP data flow, while the MPQUIC establishes a new QUIC stream for every UDP data flow.

-	The MPTCP applies a steering mode for every MP-TCP connection, while the MPQUIC applies a steering mode for every QUIC stream.

Since the UE indicates if it supports the MPQUIC steering functionality (in the PDU Establishment Request message), the network may configure the UE/UPF to apply MPQUIC for UDP data flows, only if it is supported by the UE. Otherwise, the network may configure the UE/UPF to apply ATSSS-LL for UDP data flows.

In case of an MA PDU Session with Ethernet type, only the ATSSS-LL is applied.

Figure 6.11.5-1: Co-existence of steering functionalities

### 6.11.6	Handling of out-of-order delivery

In simultaneous multipath transmission of a UDP flow using the MPQUIC steering functionality a percentage of received packets are out-of-order. These packets need to be reassembled in the correct order (i.e., to be re-ordered) before they are consumed. This out-of-order delivery arises when:

-	The MPQUIC steering functionality applies a steering mode that simultaneously uses both accesses, such as the Load-Balancing steering mode, for an UDP flow and (a) the transmitted packets are scheduled across the two accesses on a per-packet decision (aka "per-packet splitting") and (b) there is a difference of RTTs across the two accesses.

Nevertheless, to enable scenarios where per-packet spitting is required and out-of-order delivery cannot be avoided otherwise, the MPQUIC steering functionality is able to:

-	Apply packet re-ordering by using the existing mechanisms provided by QUIC for stream transport, when the PCF indicates that the stream mode should be applied (head-of-line blocking can happen when delay due to re-transmissions is too high).

-	Apply packet re-ordering by using mechanisms not provided by QUIC, when the PCF indicates that the Datagram mode 1 should be applied. As mentioned before, the Datagram mode 1 adds sequence numbers to the HTTP Datagrams that carry the UDP data packets (see details in clause 6.11.3, step 5) and requires the definition of a new Context Id. This Context Id, as well as the following aspects of Datagram mode 1 will be considered in stage-3.

-	How to ensure re-ordering on the receiver side is implemented and activated, when needed.

-	How the same reordering in both downlink and uplink can be provided.

-	Which sequencing and reordering schemes can be supported and whether such schemes will be specified or will left to UE/UPF implementation.

### 6.11.7	Handling of duplicated packets

In some scenarios, the multipath transmission of a UDP flow using the MPQUIC steering functionality can lead to reception of double copies of the same packet. The duplicated copies of packets need to be discarded (i.e., to be deduplicated) before they are consumed. This duplicated delivery issue can arise when:

-	The MPQUIC steering functionality applies the redundant steering mode (RSM) for an UDP flow.

The MPQUIC steering functionality can identify and remove the duplicated packets by using a transport mode that supports packet reordering / deduplication, such as the Datagram mode 1 or the Stream mode.

### 6.11.8	Impacts on Existing Nodes and Functionality

The 5GC network functions shall be able to support the following functionality in addition to the ATSSS functionality defined in Rel-17.

AMF

-	No additional functionality is needed. It is assumed that when 5GC supports ATSSS / Rel-18, then all ATSSS-capable SMFs in 5GC are able to support MPQUIC.

SMF

-	Shall be able to determine whether the UE supports the MPQUIC steering functionality and to indicate this to PCF.

-	Shall be able to create ATSSS/N4 rules that apply the MPQUIC steering functionality, based on the PCC rules received from PCF.

-	Shall be able to select a UPF that supports the MPQUIC steering functionality.

PCF

-	If it is informed from SMF that the UE supports the MPQUIC steering functionality, then it shall be able to determine which UDP flows should be steered with the MPQUIC steering functionality. For each UDP flow, it shall be able to indicate a steering mode and a transport mode.

UPF

-	Shall be able to support the MPQUIC steering functionality for steering the UDP flows indicated in the received N4 rules. For each UDP flow, it should apply the transport mode indicated in the received N4 rules.

UE

-	Shall be able to indicate in the PDU Session Establishment Request that is supports the MPQUIC steering functionality.

-	Shall be able to support the MPQUIC steering functionality for steering the UDP flows indicated by the received ATSSS rules. For each UDP flow, it should apply the transport mode indicated in the received ATSSS rules.

The MPQUIC steering functionality is largely based on existing IETF specifications (see clause 6.11.2) but does not require any changes on these specifications.

-	A new QUIC transport parameter must be defined by 3GPP and registered with IANA. This transport parameter associates a QUIC connection with a QoS flow.

End of Changes