SA WG2 Meeting #153e	S2-2209092r02

# 10 - 17 October 2022, Electronic meeting        		            (revision of S2-220xxxx)

Source: 	Lenovo, Motorola Mobility, Apple, Broadcom, Tencent

Title: 	Evaluation for KI#2

Document for: 	Approval

Agenda Item: 	9.3

Work Item / Release:	FS_ATSSS_Ph3 / Rel-18

Abstract of the contribution:

# 1.	Proposal

It is proposed to add the following changes to TR 23.700-53.

Start of Changes

## 7.1	Evaluation for KI #2: New steering functionalities for non-TCP traffic

The main classes of solutions related to KI#2 are DCCP-based (#2.1) and QUIC-based (#2.2 and #2.3). Additionally, solution #2.4 complements some of those main solutions with methods to reduce header overhead.

Both classes of solutions support a “Low-Layer” mode, where the payload of the MA-PDU session is an IP packet or an Ethernet frame. The QUIC-based class of solutions also supports “UDP proxying”, where the payload of the MA-PDU session is a UDP payload (#2.2). Solution #2.4 details an aspect of “Low-Layer” solutions, which enables bringing their user plane performance up to par with the “UDP proxying” solution.

### 7.1.1	User Plane Performance Aspect

While the main solutions differ from per-packet overhead standpoint, solution #2.4 describes how inner header compression can be used in the tunnel between UE and UPF to limit overhead in some cases (#2.1 and #2.3). The estimated per-packet overhead of all 3 solutions (#2.1, #2.2, and #2.3) becomes equivalent to each other if header compression is used with #2.1 and #2.3.

Solution #2.2 does not require header compression to achieve its low per-packet overhead. RoHC (or EHC) can be implemented with Solutions #2.1 and #2.3 to perform inner header compression in MA-PDU session endpoints (UE and UPF).

### 7.1.2	Evaluation of steering functionalities

For KI#2, three solutions have been proposed:

- Solution #2.1 (aka MP-DCCP-LL);

- Solution #2.2 (aka MPQUIC-UDP); and

- Solution #2.3 (aka MPQUIC-IP).

Solution #2.1 is based on the DCCP protocol and its multipath extension (draft-ietf-tsvwg-multipath-dccp). This solution supports the transport of UDP flows, IP flows and even Ethernet flows, over DCCP.

In contrast, Solutions #2.2 and #2.3 are both based on the QUIC protocol, its multipath extensions (draft-ietf-quic-multipath) and additional extensions defined in IETF for supporting "UDP proxying over HTTP" and "IP proxying over HTTP" respectively (see RFC 9298 and draft-ietf-masque-connect-ip). These solutions support the transport of UDP flows and IP flows respectively, encapsulated in HTTP/3.

Solutions #2.2 supports UDP traffic only.

Solutions #2.2 supports IP and UDP traffic.

Solution #2.1 support IP, UDP and Ethernet traffic

Note: Ethernet traffic support was not agreed to be in scope of the key issue but is requested functionality by BBF.

#### 7.1.2.1 Co-existence with MPTCP and ATSSS-LL

All three solutions #2.1, #2.2 and #2.3 are in principle capable to co-exist with MPTCP and ATSSS-LL and can be chosen using the solutions defined N4 and ATSSS rules.

Solutions #2.1 does not necessarily need any additional steering functionality as it can handle all kind of traffic.

#### 7.1.2.2 Evaluation of the impact of User-plane overhead

##### 7.1.2.2.1 Header Size Considerations summary

Table 1 shows header sizes introduced by MP-QUIC based solution#2.2 and solution#2.3 for carrying either UDP or IP using Datagram mode 1, Datagram mode 2 or Stream mode, and the MP-DCCP solution.

| Solution | Add. Header size [Bytes] | Comment |
| --- | --- | --- |
| MP-QUIC UDP Datagram mode 1 | 46 – 67 average 55 | - Datagram Length, Quarter Stream ID and Context ID are variable length encoded fields varying between 1, 2, 4, or 8 Bytes. |
| MP-QUIC IP Datagram mode 1 | 54 - 75 average 63 | - Datagram Length, Quarter Stream ID and Context ID are variable length encoded fields varying between 1, 2, 4, or 8 Bytes. Further gain through inner IP header compression possible |
| MP-QUIC UDP Datagram mode 2 | 38-52 average 46 | As Datagram mode 1 but without Sequence number |
| MP-QUIC IP Datagram mode 2 | 46-60 average 54 | As Datagram mode 1 but without Sequence number Further gain through inner IP header compression possible |
| MP-QUIC UDP Stream mode | 41-68 average 56 | As Datagram mode 1 but with more variable length encoded fields. |
| MP-QUIC IP Stream mode | 49-76 average 64 | As Datagram mode 1 but with more variable length encoded fields. Further gain through inner IP header compression possible |
| MP-DCCP-LL | 55, 59, 61, or 65 | Depending on short/long DCCP header and the type of delay information exchange for reordering. 55 Byte for short header and DCCP timestamp option, 65 Byte for long header and MP_RTT option. There is no reason to select the high overhead constellation. |

Table 1: MP-QUIC solutions and MP-DCCP-LL user-plane overhead introduced by protocol headers.

If MP-QUIC to provide path latency information exchange this would increase the overhead by at least 6 Bytes.

Inner header compression can reduce overhead for MP-DCCP for both IP and UDP header, for MP-QUIC IP for IP header only.

MPQUIC steering functionality can multiplex payload packets in the same QUIC packet (this is a feature supported by QUIC), thus, reducing the user-plane overhead. In addition, a single UDP datagram can multiplex multiple QUIC packets, which can further reduce the overhead.

NOTE: As a consequence of RFC9000 [6] the precondition for possible multiplexing of payload packets is that those payload packets fit completely inside a single QUIC packet, which is limited by the size of the UDP datagram and the max. MTU size, typically 1200 Byte.

##### 7.1.2.2.2 Detailed analysis on header sizes

MP-DCCP related information can be found in two documents - MP-DCCP draft [12 ] and RFC4340 (DCCP) [11].

MP-QUIC solutions requires multiple documents to investigate:

QUIC RFC9000 [6], MP-QUIC draft-ietf-quic-multipath [10], QUIC Datagram RFC 9221[9], Connect-UDP RFC 9298 [27], Connect-IP draft-ietf-masque-connect-ip [31], HTTP/3-Datagram RFC 9297 [29]

Basically, the following list gives an impression how the individual solutions are composed of. It should be noted that the multipath extension of QUIC does not leave a header footprint.

- MP-QUIC UDP Datagram mode:

UDP + QUIC + MP extension + QUIC DATAGRAM + HTTP Datagram

- MP-QUIC IP Datagram mode:

UDP + QUIC + MP extension + QUIC DATAGRAM + HTTP Datagram + UDP

- MP-QUIC UDP Stream mode:

UDP + QUIC + MP extension + STREAM Frame + HTTP Datagram

- MP-QUIC IP Stream mode:

UDP + QUIC + MP extension + STREAM Frame + HTTP Datagram + UDP

- MP-DCCP-LL:     DCCP + MP extension + IP + UDP

For the MP-QUIC solution multiple nested documents needs to be skimmed and solution specific parameters needs to be considered. Below is the extra size added by the MP-QUIC UDP solution.

UDP QUIC Short    DATAGRAM         HTTP Datagram      HTTP Info

1-RTT Packet, Type, [Length],  Quarter Stream ID, Context ID, Seq No.,

8   26            1     {1,2,4,8}  {1,2,4,8}          {1,2,4,8},  8?       = 46/67 Bytes

QUIC Short: RFC9000

DATAGRAM: RFC9221

HTTP DATAGRAM: RFC9297

HTTP Info Context ID: RFC9298

HTTP Info Seq. No.: Solution specific

Datagram Length parameter is required to support multiplexing capability

Stream ID is required for differentiating QoS flows and steering mode within one MP-QUIC connection

MP extension has no impact to the header size as it only impacts the meaning of the RFC9000 defined packet number.

Seq. No. is reasonably assumed to be 8 Byte, however it is not specified so far.

Length, Quarter Stream ID and Context ID are variable length encoded fields varying between 1, 2, 4, or 8 Bytes. In the case of Datagram mode 2, Context ID is a 1 Byte value.

In case MP-QUIC Stream mode is applied, the HTTP Datagram is capsuled in a Datagram Capsule element which itself is part of a STREAM Frame. Compared to Datagram modes, HTTP Info field is not required as ContextID and Seq. No. are not required.

UDP QUIC Short    STREAM Frame                           Datagram Capsule  HTTP Datagram

1-RTT Packet, Type, Stream ID, [Offset] , [Length] ,  Type, Length,    Quarter Stream ID

8   26            1     {1,2,4,8}  {1,2,4,8}  {1,2,4,8}   1     {1,2,4,8}  {1,2,4,8} = 41/76 Bytes

QUIC Short: RFC9000

STREAM Frame: RFC9000

Datagram Capsule: RFC9297

HTTP DATAGRAM: RFC9297

STREAM FRAME Length parameter is required to support multiplexing capability

STREAM FRAME Offset parameter is required to keep send order

For the MP-QUIC IP solution an extra 8 Bytes needs to be added to all header sizes as the UDP header of the e2e data persists.

MP-DCCP-LL encapsulates IP data, therefore compared to the MP-QUIC UDP solution the inner IP/UDP header of the e2e data counts.

Data Packet
MP_RTT for Reordering
---DCCP------- + -----MP-DCCP--- + Payload

Generic Header + MP-SEQ + MP-RTT + IP + UDP
12 or 16       + 9      + 12     + 20 + 8 = 61/65 Bytes

DCCP Timestamp for reordering

---DCCP------------------- + MP-DCCP + Payload
Generic Header + timestamp + MP-SEQ  + IP + UDP
12 or 16       + 6         + 9       + 20 + 8 = 55/59 Bytes

### 7.1.3	Evaluation of steering functionalities for UDP traffic flows

#### 7.1.3.1	General

For supporting UDP traffic flows, two alternative solutions can be used: Solution #2.1 (MP-DCCP-LL) and Solution #2.2 (MPQUIC). The following sub-clauses discuss different aspects of these solutions.

The solution #2.3 may also be used for UDP traffic flows but this solution is more suitable when IP traffic flows (other than UDP) should be supported, thus, it should be considered separately.

#### 7.1.3.2	Allocation of UPF resources

As specified in clause 6.3.5, the MP-DCCP-LL solution requires the UPF to allocate: one IP address per access and one port per MP-DCCP connection, as shown in the figure below. The UPF allocates the IP addresses and ports for each MP-DCCP connection based on information received from SMF (MP-DCCP-LL Control Information), which indicates how many MP-DCCP connections are required. The SMF derives this information based on the PCC rules received from PCF. If additional MP-DCCP connections are required during the lifetime of the MA PDU Session, the SMF should ask UPF to allocate more resources for these additional connections and then send a PDU Session Modification message to UE including the allocated UPF resources.

Fig. 7.1.2.2-1 (a)

On the contrary, the MPQUIC solution requires the allocation of one IP address and one UDP port at the UPF, which are used for all MPQUIC connections of a UE. This is possible because the QUIC protocol can multiplex many connections on the same UDP port (using the “connection ID” parameter). The UE receives the IP address and UDP port at the UPF as part of the “proxy information” in the PDU Session Establishment Accept (exactly the same as in MP-TCP). When the UE needs to establish a new MPQUIC connection to UPF, the UE applies the received “proxy information” and does not require additional resources to be allocated at the UPF.

Fig. 7.1.2.2-1 (b)

#### 7.1.3.3	Connections between UE and UPF

The MPQUIC solution requires one MPQUIC connection per QoS flow (see clause 6.11.2). Each UDP flow transferred via an MPQUIC connection is associated with a unique QUIC stream on the connection and each QUIC stream is configured to apply a steering mode. Thus, different steering modes can be applied per MPQUIC connection.

The MP-DCCP-LL requires one connection per QoS flow and per Steering Mode (see clause 6.3.2), so it needs more connections than MPQUIC. The MP-DCCP-LL connection management (adding / removing connections) is cumbersome: Every time a new UDP flow should be transmitted using a steering mode for which no MP-DCCP connection supports, a new MP-DCCP connection should be established and the SMF should send a PDU Session Modification message with the parameters (UPF address/ports) for this MP-DCCP connection.

The MP-DCCP-LL solution specifies that the UE receives "MP-DCCP Connection Setup Information", which indicates the QoS flow and the steering mode associated with each MP-DCCP connection. However, it does not specify how the UPF knows the QoS flow, and the steering mode associated with each MP-DCCP connection. In the MPQUIC solution, the QoS flow associated with an MPQUIC connection is indicated (as a new transport parameter) during the connection setup.

#### 7.1.2.4	Application visibility

The MPQUIC steering functionality is a high-layer steering functionality (like MPTCP), so multipath-aware applications in the UE can interact with it , e.g., via an enhanced sockets API (which is outside the scope of 3GPP), e.g.

An application may implement itself the QUIC protocol and use it for reliable transport (using QUIC Streams). In this case, it could inform the underlying steering functionality that it should apply unreliable transport (e.g., using QUIC Datagrams) to avoid TCP meltdown effects.

As shown in the following figure, the MP-DCCP-LL steering functionality is a low-layer functionality.

#### 7.1.3.4	User-plane overhead

As shown in the figure below, the MP-DCCP-LL steering functionality implements an IP-in-IP tunneling solution, whereas the MPQUIC steering functionality implements a UDP proxying solution over a HTTP/3 tunnel. The Table 1 in section 7.1.2.2.1 summarizes user plane overhead due to additional headers for each solution.

Additional considerations are that

both the MP-DCCP-LL and the MPQUIC send Ack packets to acknowledge successful reception.

As shown in figure below, when the app provides two payloads in succession, the MPQUIC steering functionality can multiplex these payloads in the same QUIC packet (this is a feature supported by QUIC), thus, reducing the user-plane overhead (by saving in this 34 Byte in this example) . In case of MP-DCCP-LL, however, the two app payloads will be encapsulated into two IP (inner) packets and will be transmitted with two DCCP-Data packets resulting in higher user-plane overhead (20 Byte in this example). In addition, in case of MPQUIC, a single UDP datagram can multiplex multiple QUIC packets, which can further reduce the overhead.

NOTE: As a consequence of RFC9000 [6] the precondition for possible multiplexing of payload packets is that those payload packets fit completely inside a single QUIC packet, which is limited by the size of the UDP datagram and the max. MTU size, typically 1200 Byte.

To reduce user-plane overhead, the MP-DCCP-LL solution can support header compression, e.g., with a solution similar or identical to solution #2.4. The header compression introduces additional processing.

An overhead of the MPQUIC steering functionality, which is not evident from the figure below, is the HTTP CONNECT request plus the 200 OK response, sent at the beginning of every UDP flow.

Figure 7.1.3.4-1: Example of user-plane overhead when using the MPDCCP functionality versus MPQUIC steering functionality multiplexing payload (UL direction)

#### 7.1.3.5	Impact due to encryption

As specified in RFC 9001 (Using TLS to Secure QUIC), each QUIC connection is established using TLS v1.3 handshake and all QUIC packets are encrypted, and integrity protected using the negotiated TLS keys and algorithms. However, in case of ATSSS_ph3, where QUIC is applied between the UE and UPF and where access-level security mechanisms are used to protect the user-plane traffic, the additional security mechanisms in the QUIC layer is unnecessary. The MPQUIC solution contains the following EN, so this issue is expected to be addressed by SA3.

Editor's note: Whether and how encryption in the QUIC layer can be omitted is FFS and need to be verified by SA3. The impact due to encryption regarding overhead and performance is FFS.

The double-layer security is not an issue for MP-DCCP-LL, which does not provide cryptographic security mechanisms and relies on other security methods (e.g., IPsec or 3GPP user-plane security) to protect the traffic.

MP-DCCP-LL does not add additional encryption.

#### 7.1.3.6	Packet reordering and deduplication

Both MP-DCCP-LL and MPQUIC solutions provide the means to support packet reordering and deduplication.

The MP-DCCP-LL solution mentions that "MP-DCCP protocol itself does not specify re-ordering mechanisms but provides path sequencing (DCCP packet sequencing [11]), connection sequencing (MP_SEQ option [12]), and has inherent latency (MP_RTT option[12]) or timing (DCCP timestamp option[11]) information exchange, i.e., all properties that are necessary for the packet receiving side, i.e. UE or UPF, to be able to implement re-ordering properly." Although the MP-DCCP draft [12] states that "The details of the transmission scheduling mechanism and optional reordering mechanism are up to the sender and receiver, respectively, and are outside the scope of the MP-DCCP protocol," it is understood that such mechanisms can be considered and possibly defined in stage-3.

The MPQUIC solution can readily support reordering and deduplication by leveraging the existing QUIC stream mechanisms, which are applied in the Stream transport mode. So, when the UDP traffic can be transferred in Stream transport mode, no additional mechanisms need to be specified for supporting packet reordering and deduplication.

In addition, the MPQUIC solution supports Datagram mode 1 (see clause 6.11.3), which supports unreliable transport and inserts sequence numbers in every UDP data packet. In Datagram mode 1, every UDP packet is encapsulated into a QUIC Datagram frame, which also carries a Context ID and a sequence number. The definition of this Context ID, as well as other aspects of Datagram mode 1 (e.g., reordering mechanisms), will be considered in stage-3.

#### 7.1.3.9	IETF Support

The QUIC multipath draft is planned to be published as a Standards Track RFC, while the DCCP multipath draft is planned to be published as an Experimental RFC.

As specified in RFC 2026:

"Specifications that are not on the standards track are labeled with one of three "off-track" maturity levels:  "Experimental", "Informational", or "Historic".  The documents bearing these labels are not Internet Standards in any sense."

MPDCCP has announced working group last call for November 2022.

MPQUIC draft has no associated work group mile stone available at the moment.

NOTE1: For MPDCCP a working implementation already exists.

NOTE2: TCP Converter as specified for MPTCP proxy is based on the Experimental RFC 8803.

End of Changes