3GPP SA WG2 Meeting #153E	S2-2209125r01

e-meeting, October 10 – 14, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Analysis and Conclusion KI #8: support of location service continuity in case of UE mobility

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_Ph3/Rel-18

Abstract of the contribution: This paper proposes evaluation and conclusion for KI#8 in TR 23.700-71.

Discussion

The KI#8 lists down 2 broad categories of scenarios:

Scenario-1 : LCS handling during UE mobility between EPS and 5GS (bi-direction).

Scenario-2 : LCS handling during UE mobility between NG-RAN node.

For each of these scenarios, some common aspects to be covered in the solutions are

- MTLR/MOLR

- How to handle the UE context in LMF/AMF etc.,

- Updated configuration during mobility

- Cancellation of LCS session during mobility

Proposal

It is proposed to update 23.700-71 as follows

*** Start of changes ***

7.X        Key Issue #8: support of location service continuity in case of UE mobility

Solution# (22, 30) focus on the location service continuity for UE moves between NG-RAN nodes, the solutions summary is shown in Table 7.1.1-1 based on the following solution principle evaluation:

| ID | Solution Summary | Solution Evaluation |
| --- | --- | --- |
| 22 | S-AMF shares LCS session details to T-AMF as part of HO preparation T-AMF notifies to LMF after HO completion LMF continues with the LCS Session with new configuration LMF responds with UE location | The solution is applicable to UL/DL methods + MOLR/MTLR/NILR procedures + Xn/NG Based HO - it is generic in nature Sharing of LCS information was analysed at RAN/AMF/LMF/GMLC - RAN - not possible for DL methods - LMF - LMF/eSMLC have no direct interface - GMLC - this is possible, but may take time compared to AMF - AMF - was found to be the most optimal in terms of time taken + generality of the solution - was chosen for the solution The solution ensures that the LCS Client need not be aware of the mobility For mobility in RRC-INACTIVE and RRC-IDLE the explicit solution is not proposed. For mobility in RRC-INACTIVE, we believe that the current solution defined in R17 (23.273, sec. 6.7.3/6.7.4/6.7.5) could be reused/enhanced. For mobility in RRC-IDLE, the current proposed solution (where location transfer happens from the old AMF to new AMF) could be reused. |
| 30 | As part of the HO procedure, the source RAN sends Positioning related information to Target RAN over Xn or NGAP (depending on the type of HO) Target RAN then sends the positioning information to LMF AMF/Old LMF then determine if LMF should be reselected and forward the positioning configuration to chosen LMF. AMF notifies the LMF the target cell ID/NG-RAN - based on that, the LMF configures the neighbouring RAN with the latest configuration In RRC-Idle mobility, as part of registration procedure, the new AMF checks if positioning information is present in the UE context data - if so, then sends the new AMF ID to LMF via the next periodic N1messageNotify | The solution for connected mobility proposes the sharing of Positioning information between RANs directly (through Xn or NG). The following issues are noted For DL methods, the source RAN will not know that an LCS session is on or not due to the following reasons - LCS signalling happens over LPP (invisible to RAN) - there is no UE associated NRPPa signalling for DL methods - The IE Positioning Information defined in R17:XnAP provides only SRS-Config info - for UL methods. Evidently this solution cannot be applied to DL methods. For UL methods, the sharing information over Xn is not likely to speed up the procedure, since that information can be taken into use by the target RAN only when the LMF sets up the LCS session with the target RAN In summary, we believe that this solution will not work for DL methods based LCS session and offers not much advantage for UL methods. Any solution defined should be common for UL, DL or UL+DL methods. |

Table 7.X-2: Comparison of solutions to NG-RAN node mobility

A. Solutions principle

(a) Solution#22

For location service continuity for UE moves in CM-connected with RRC-connected:

-   After handover complete, the AMF notify LMF the target cell ID, based on that information, LMF acquires the positioning information from target NG-RAN and configures the selected neighbour NG-RANs with the latest configuration.

(b) Solution#30

For location service continuity for UE moves in RRC-inactive:

-   Based on RAN WG R17 conclusion, with anchor NG-RAN relocation, target NG-RAN acquire the old LMF ID and the positioning information, target NG-RAN update the positioning configuration based on the positioning information and sends it to LMF.

-   After N2 path switch is triggered, AMF or old LMF determines whether need trigger the LMF change procedure and enable the latest configuration reach to the correct LMF.

-   After handover complete, AMF notify LMF the target cell ID and/or the target NG-RAN ID, based on that information, LMF configures the selected neighbour NG-RANs with the latest configuration. AMF/LMF need indicate the NG-RAN the new LMF ID.

For location service continuity for UE moves in CM-connected with RRC-connected:

-  Try to align with the RAN WG R17 conclusion for location service continuity for RRC-inactive UE, target NG-   RAN acquires the positioning information and source LMF ID from source NG-RAN, target NG-RAN updates the positioning configuration and sends it to LMF.

-  After N2 path switch is triggered, AMF or old LMF determines whether need trigger the LMF change procedure and enable the latest configuration reach to the correct LMF.

-   After handover complete, AMF notify LMF the target cell ID and/or the target NG-RAN ID, based on that    information, LMF configures the selected neighbour NG-RANs with the latest configuration. AMF/LMF need indicate the NG-RAN the new LMF ID.

For location service continuity for UE moves in CM-IDLE

During the registration procedure, after new AMF receive the UE context information from old AMF, if AMF finds UE has positioning information, AMF sends the new AMF instance ID to LMF.

*** Next changes ***

8.X        Key Issue #8: support of location service continuity in case of UE mobility

Normative work will take place according to principles described in this clause:

For mobility between NG-RAN nodes in connected mode

- S-AMF shares LCS session details to T-AMF as part of HO preparation

- T-AMF notifies to LMF after HO completion

- LMF continues with the LCS Session with new configuration

- LMF responds with UE location to T-AMF which responds to the LCS Client via the GMLC(s).

For mobility between NG-RAN node while the UE was RRC-INACTIVE

- The current solution defined in R17 (23.273, clause. 6.7.3/6.7.4/6.7.5) is reused

- For the case with anchor NG-RAN relocation, after path switch is triggered, if AMF receive the positioning configuration from NG-RAN and if LMF is changed:

-  The AMF/old LMF should determine the new LMF, and notify new LMF the positioning configuration

and the target cell ID.

- -  The new LMF/AMF should notify NG-RAN the new LMF.

For mobility in RRC-IDLE

-   If AMF is changed, the new AMF should notify its instance ID to LMF.

*** End of changes ***