3GPP TSG-SA WG2 Meeting #153E (e-meeting)	S2-2209128r01

# Oct 10 – 17, 2022, Elbonia

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0236 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multiple location report for MT-LR Immediate Location Request for the regulatory service |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_MLR |  |  |  |  |  |  | Date: |  | 2022-10-10 |  |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-18 |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |  |  |
| Reason for change: |  | - There are emergency regulations that PSAP could receive multiple results from LCS session. - Legacy features in TS 23.273 focuses on final response of LCS session. This CR provides solution to provide multiple responses to regulatory LCS Clients within the LCS session. - The change can also be applied to lawful interception if needed. |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | LMF in 5GC-MT-LR procedure for regulatory location service optionally can send more location estimated to GMLC by reusing Nlmf_location_EventNotify service operation within the LCS session. |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Existing periodic deferred location methods depends on the event trigger in UE and are more suitable for cLBS. When deferred location used in regulatory service, it makes UE dependency in regulation compliance. |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1a.4, 6.10 |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

### 4.1a.4	Immediate Location Request

With an immediate location request, an LCS client or AF sends or instigates a location request for a target UE (or group of target UEs) and expects to receive a response containing location information for the target UE (or group of target UEs) within a short time period which may be specified using QoS. In regulatory cases, one or more responses of the target UE’s location information can be expected. An immediate location request may be used for an NI-LR. MT-LR or MO-LR.

* * * * Next Change * * * *

### 6.10.X	Regulatory 5GC-MT-LR Procedure with Multiple Response

NOTE: When event triggers in UE are applicable in regulatory requirements, clause 6.3 can be used to generate multiple location results based on periodic or other events in UE.

This section applies when regulatory response should not rely on the event triggers in UE.

Figure 6.10.X-1 illustrates a location request for a regulatory services session, where a regulatory services client (e.g. a Public Safety Answering Point) identifies the target UE and the serving GMLC/LRF.

In following cases, GMLC/LRF don’t need to query the UDM of the target UE for the serving AMF address:

- In emergency case, AMF selects GMLC. GMLC gets AMF address from Namf_Location_EventNotify service operation.

- Lawful interception usually performs repeated tracking LCS sessions. As per Table 5.2.2.1-1 of TS 23.502, GMLC subscribes to Namf_EventExposure of AMF and can track the latest UE’s serving AMF information.

NOTE: The GMLC subscription to AMF may be handled on-demand before step 2 of Figure 6.10.X-1 or handled before-hand. For example, it could be subscribed when lawful interception session or emergency call is initiated.

This allows the GMLC to request a location from the AMF without needing to query the UDM for the serving AMF address in most regulatory cases.

Figure 6.10.X-1: Regulatory 5GC-MT-LR Procedure with multiple response

- The external emergency services client (e.g. a PSAP) sends a request to the GMLC/LRF for a location for the target UE and includes correlation information identifying the target UE. The request may include the required QoS , Supported GAD shapes, request type to accept INTERMEDIATE response, the optional reporting time window and optional QoS requirement for the INTERMEDIATE responses. The GMLC/LRF address and the correlation information would have been previously provided to the external client when the regulatory session from the UE was established.

NOTE: INTERMEDIATE reporting timing window and QoS requirement could be preconfigured or among parameters in the request of external client.

NOTE: timing window specifies the time gap between the request and first INTERMEDIATE response and time gap between multiple consecutive INTERMEDIATE responses and time gap between last INTERMEDIATE response and FINAL response. It could also specify the time variance of these gaps. The details are decided by stage 3 and should be consistent with responseTime and responseTimeEarlyFix information in TS 37.355.

NOTE: last INTERMEDIATE response is decided by LMF implementation logic. It could be EarlyFixReport with the value of moreMessagesOnTheWay defined in TS 37.355.

2.	The LRF/GMLC determines the AMF by associating the correlation information received from the external client with other information received previously from the LMF as described in clauses 6.10.1 and 6.10.3. The GMLC invokes the Namf_Location_ProvidePositioningInfo service operation towards the AMF to request the current location of the UE. The service operation includes the SUPI or the PEI and an indication of a location request from a regulatory services client and may include the required QoS , Supported GAD shapes, request type to accept INTERMEDIATE response, the LIR reference number, the optional reporting time window and optional QoS requirement for the INTERMEDIATE responses.

3.	The AMF selects an LMF based on NRF query or configuration in AMF and invokes the Nlmf_Location_DetermineLocation service operation towards the LMF to request the current current and/or INTERMEDIATE location of the UE. The service operation includes a LCS Correlation identifier, the serving cell identity and an indication of a location request from an regulatory services client and may include an indication if UE supports LPP, the required QoS and Supported GAD shapes, UE Positioning Capability if available. If any of the procedures in clause 6.11.1 or clause 6.11.2 are used the service operation includes the AMF identity.

NOTE: AMF may select LMF based on additional regulatory criteria. When INTERMEDIATE response is needed, AMF include also GMLC address, timing window and accuracy parameters to LMF.

4.	The LMF performs one or more of the positioning procedures described in clauses 6.11.1, 6.11.2 and 6.11.3. When INTERMEDIATE response is required, LMF could perform the location session with multiple transactions to generate the INTERMEDIATE result(s) before the FINAL response could be generated.

  - LMF may generate location information in the INTERMEDIATE response using the EarlyFixReport in  ProvideLocationInformation of TS 37.355.

  - LMF may also use other information to generate the location in INTERMEDIATE response, e, g. the area shown by serving/primary cell  parameter of Nlmf_Location_DetermineLocation.

4.1 Based on the reporting time window and QoS parameters, the LMF invokes an Nlmf_Location_EventNotify service operation towards GMLC and provides the INTERMEDIATE location of the UE and timestamp of the location estimate, and LIR reference number.

4.2 As per Table 5.2.2.1-1 of TS 23.502, LMF subscribes to Namf_EventExposure of AMF. For QoS requirements, LMF can use Mobility Events from AMF described in clause 4.15.4.2 of TS 23.502 to detect the UE mobility and may decide not to wait till next reporting time window but to trigger the possible INTERMEDIATE report.

NOTE: The LMF subscription to AMF may be handled on-demand before step 4.2 or handled before-hand. For example, it could be subscribed when emergency call is initiated.

4.3 As per clause 5.2.3.2 of OMA MLP 3.3 and above versions, GMLC provides the INTERMEDIATE response to regulatory client over Le interface.

5.	The LMF returns the Nlmf_Location_DetermineLocation Response towards the AMF to return the current location of the UE. The service operation includes the location estimate, its age and accuracy and may include information about the positioning method and the timestamp of the location estimate. The service operation also includes the UE Positioning Capability if the UE Positioning Capability is received in step 4 including an indication that the capabilities are non-variable and not received from AMF in step 3.

6.	The AMF returns the Namf_Location_ProvidePositioningInfo Response towards the GMLC/LRF to return the latest or best current location of the UE as the FINAL response. The service operation includes the LCS Correlation identifier, the location estimate, its age and accuracy and may include information about the positioning method and the timestamp of the location estimate.

NOTE: whether latest or best QoS location is chosen as FINAL response depends on different regulatory requirements and implementation specific.

7.	The GMLC/LRF sends the FINAL location service response to the external regulatory services client.

* * * * End of Changes * * * *