3GPP TSG-SA WG2#153E e-meeting	S2-2209154

Elbonia, 10 – 17 October 2022	(was S2-220xxxx)

Source:	Qualcomm Incorporated, Charter Communications, AT&T

Title:	KI#5: Conclusion update to address EAS discovery

Document for:	Discussion/Approval

Agenda Item:	9.11

Work Item / Release:	FS_EDGE_Ph2 / Rel-18

Abstract of the contribution: This contribution finalizes the description of Sol. 22 option 2.

1.	Discussion

This document addresses the EN in the conclusion of KI#5 related to EAS discovery

2.	Text proposal

It is proposed to agree the following changes vs. TR 23.700-48:

# >>>>BEGINNING OF CHANGES<<<<

8.5	KI#5: GSMA OPG impacts and improvements for EHE operated by separate party

The following are recommended as baseline for normative work.

EAS Deployment Provision and EAS Discovery

To enable the discovery of an EAS deployed in another PLMN (i.e., the hosting PLMN in the role of a partner Operator Platform) than the one serving the UE (i.e., the serving PLMN),

the serving PLMN is provided with the information needed to support UEs to discover and connect to the shared EHE deployed in the hosting PLMN.  In particular, the serving SMF may either store:

-	the FQDN(s), DNS Server Information, and/or DNAI(s) of the EAS Deployment Information (EDI, see TS 23.548 [x] clause 6.2.3.4-1) for each partner and use the Partner ID (e.g., PLMN ID or partner EHE ID) to identify the partner that EDI corresponds to. Such EDI information can be preconfigured in the serving SMF or provided by the AF (see clause 6.38.2.2), and the NEF from the PLMN of EHE could act as AF. NEF could be in charge of translating PLMN specific information (e.g. DNAI, others if needed) in EDI.

NOTE 1:	Whether the provision of additional parameters of the partner PLMN’s EDI (TS 23.548 [x] clause 6.2.3.4-1, e.g., DNN, S-NSSAI etc.) to the serving PLMN violate the GSMA OPG requirement of topology hiding or not will depend on feedback from GSMA OPG.

NOTE 2:	The final list of EDI parameters that can be exchanged among PLMNs will be determined in the normative phase based on feedback from GSMA OPG.

-	A list of preconfigured DNS servers per Partner ID. Such information is configured in the serving SMF and is provided to the serving EASDF (see clauses 6.22.2.2 and 6.22.3.2), to be used in case the EDI is not available.

-	The EAS discovery procedure defined in TS 23.548 [3] will be reused.

Traffic transmission between two PLMNs

-	When the traffic is transmitted between two PLMNs, the UPF in the serving PLMN should support mechanisms to facilitate low latency transmission. N6 routing information with transmission requirements could be included in EDI for routing specific edge traffic (identified by FQDN in EDI) from serving PLMN to EHE with low latency.

NOTE 3: Charing should be supported in sharing EHE scenario. And how to support charging in EHE sharing scenario should be specified with considering GSMA OPG's feedback. Some cooperation with SA5 may be needed.

# >>>>END OF CHANGES<<<<