3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2300857

# Jan 16 – 20, 2023, Elbonia

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0662 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New analytics ID for traffic flow use case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR 23.700-81 v2.0.0 has documented the conclusion for key issue#9 “Enhancement of NWDAF with finer granularity of location information”. As to the new use cases, the following conclusions are reached: “- NWDAF supports the following new Analytics IDs: - Analytics ID that supports collecting UE location when the UE is exactly in the requested area instead of the translated TA/cell list. - Analytics ID 'Relative Proximity' provides support for finer granularity of location information at NWDAF via relative proximity information of nearby UEs. The output analytics contain relative proximity and Time to Collision information. - Analytic ID that supports the traffic flow statistics use case. - Analytic ID that supports location accuracy estimate use case. NOTE 1: That new Analytics IDs to support these use cases are required (instead of enhancing Analytics IDs) will be confirmed during normative work. ” This contribution provides the analytics detail of the traffic flow use case based on the finer granularity location information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the new Analytics ID for the traffic flow use case in the TS 23.288. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not specified that the procedure for the “traffic flow” of “Enhancement of NWDAF with the finer granularity of location information” in R18 eNA study. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x(new), 6.x.1(new), 6.x.2(new), 6.x.3(new), 6.x.4(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all new texts)* * * *

6.x	Traffic Flow Analytics

6.x.1	General

The consumer of Traffic Flow analytics may request the NWDAF analytics information regarding the UE location and velocity information statistics or predictions during the analytics target period in the target area.

The service consumer is an AF.

The consumer of the analytics can indicate in the request:

-	Analytics ID = "Traffic Flow";

-	Target of Analytics Reporting: any UE;

-	Analytics Filter Information:

-	Area of Interest (AOI): restricts the scope of the traffic flow analytics to the provided area. The AOI can also be in form of shape and the coordinates of latitude and longitude (mandatory if Target of Analytics Reporting is set to "any UE", optional otherwise);

-	LCS accuracy: restricts the accuracy when retrieving UE location from LCS system. The accuracy is defined in clause 4.1b in TS 23.273 [x];

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Optionally, preferred level of accuracy of the analytics;

-	Optionally, preferred granularity of location information: "coordinate level";

-	Optionally, maximum number of objects; and

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF notifies the result of the analytics to the consumer as indicated in clause 6.x.4.

6.x.2	Input data

The NWDAF collects the UE location and velocity information within the area of interest from the sources listed in Table 6.x.2-1.

Table 6.x.2-1: Data collection by NWDAF for "Traffic Flow" analytics

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | GMLC | Time information associated with the collected information. |
| Location estimate | GMLC | The geographical location of the UE. |
| Velocity estimate | GMLC | Including speed and orientation information of the UE. |
| Indoors/outdoors indication | GMLC | Indicating indoors or outdoors. |
| Accuracy | GMLC | Including horizon accuracy and vertical accuracy. |
| Age of location | GMLC | The lifetime of location information. |

6.x.3	Output analytics

The output analytics of Traffic Flow analytics provided by NWDAF is defined in Table 6.x.3-1 and Table 6.x.3-2.

Table 6.x.3-1: Traffic Flow Statistics

| Information | Description |
| --- | --- |
| Area of interest | The location area of the user data collection |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot starts within the Analytics target period |
| > Duration | The time during the beginning and the end of UE location information collection |
| > Total number of users | Total number of users in the area of interest |
| > Average speed | Average speed of all users in the area of interest. |
| > Speed | A speed criterion. |
| >> Number of users | The number of users whose speed is faster than the speed criterion. |
| >> Ratio | Percentage of users. |
| > Direction | A direction criterion. Opposite directions in one road (e.g. heading north or south, Northwest or southeast) |
| >> Number of users | The number of users in the specific direction. |
| >> Average speed | Average speed of users in the specific direction. |
| >> Ratio | Percentage of users. |

Table 6.x.3-2: Traffic Flow Predictions

| Information | Description |
| --- | --- |
| Area of interest | The location area of the user data collection |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot starts within the Analytics target period |
| > Duration | The time during the beginning and the end of UE location information collection |
| > Total number of users | Total number of users in the area of interest |
| > Average speed | Average speed of all users in the area of interest. |
| > Speed | A speed criterion. |
| >> Number of users | The number of users whose speed is faster than the speed criterion. |
| >> Ratio | Percentage of users. |
| >>Confidence | Confidence of the prediction. |
| > Direction | A direction criterion. Opposite directions in one road (e.g. heading north or south, Northwest or southeast) |
| >> Number of users | The number of users towards the specific direction. |
| >> Average speed | Average speed of users towards the specific direction. |
| >> Ratio | Percentage of users. |
| >>Confidence | Confidence of the prediction. |

6.x.4	Procedures

Figure 6.x.4-1 depicts the procedure for traffic flow analytics provided by NWDAF.

Figure 6.x.4-1: "Traffic flow" analytics provided by NWDAF

1.	The AF consumer sends the request to NEF as defined in clause 6.1.1.2 using Nnef_EventExposure_Subscribe operation to request traffic flow analytics within a target area for any UE. If the target area is not in form of TA/cell list, the NEF converts the target area into AOI (a list of TA/cell) and both area information (i.e., target area and AOI) are delivered to NWDAF in Analytics Filter Information at step 1b. The AOI shall be used by NWDAF to acquire the list of UE IDs from AMF in step 2a and step 2b.

The NEF sends a subscribe or request to NWDAF for the traffic flow analytics using either Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription_Subscribe service (Analytics ID = Traffic Flow, Target of Analytics Reporting = any UE, Analytics Filter Information = AOI, Analytics Reporting Information= Analytics target period, Preferred granularity of location information: coordinate level).

2.	To retrieve the UE IDs (SUCI and/or GPSI) in the AOI, NWDAF subscribes to UE mobility event notifications of AMF as described in clause 5.3.4.4 of TS 23.501 [2] using event ID "UE moving in or out of Area of Interest" and Event Filters as described in Table 5.2.2.3.1-1 of TS 23.502 [3] if it is needed to retrieve the list of SUPIs (and/or GPSIs if available) in the area of interest.

3.	The NWDAF collects the UE location information using the GMLC interface including the information listed in Table 6.x.2-1. The NWDAF can initiate an immediate 5GC-MT-LR procedure as defined in TS 23.273 [X] for the list of SUPIs obtained at step 2b. If the consumer has subscribed to receive notifications at step 1 or the requirements from AF consumer need continuously data collection, the NWDAF can initiate a deferred 5GC-MT-LR procedure for the list of SUPIs obtained at step 2b.

4.	The NWDAF derives requested analytics.

5.	The NWDAF provides the analytics as listed in Table 6.x.3-1 and Table 6.x.3-2 for Traffic Flow to the consumer.

6-8.	If the consumer has subscribed to receive notifications for traffic flow analytics at step 1, after receiving event notification in step 6, the NWDAF may generate new analytics and provide them to the consumer.

# * * * * End of changes * * * *