SA WG2 Meeting #S2-154AH-e	S2-2300007

16 - 20 January, 2023, Electronic meeting

SA WG2 Meeting #S2-154	S2-2210178

14 - 18 November, 2022, Toulouse, France

SA WG2 Meeting ##153-e	S2-2208158

10-17 October 2022, Electronic Meeting

3GPP SA4#120-e		TDoc S4-221192

E-Meeting, 17 – 26 Aug, 2022

Title:	DRAFT Reply LS to 3GPP SA2 on VoLTE Roaming GBR Handling

Response to:	LS S2-2204724 on VoLTE Roaming GBR Handling from 3GPP SA2

Release:	Rel-17

Work Item:	TEI17

Source:	SA4

To:	SA2

Cc:	CT3, CT4, GSMA NRG

Contact person:	Bo Burman

bo <dot> burman <at> ericsson <dot> com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

SA4 thanks SA2 for the LS on VoLTE Roaming GBR Handling.

SA4 has discussed the issue and can confirm the SA2 understanding that a RAN GBR (in HPLMN or VPLMN) that is lower than what is needed to support the lowest voice codec mode the UE is configured to use (by the HPLMN), would result in voice packets being delayed and/or dropped. It cannot be assumed that a UE would adapt below the lowest configured voice codec mode, even in the presence of substantial packet loss and even if the specific voice codec technology supports lower modes than what is included in the current UE configuration.

Furthermore, the current TS 26.114 specification provides several optional and recommended speech adaptation procedures and possibilities to detect the need for speech adaptation. Currently, only one speech adaptation procedure is normative for the UE; adjusting speech codec mode based on received speech Codec Mode Request (CMR). However, neither sending CMR based on observed RTP voice packet losses nor adjusting speech codec mode based on (RTCP) reported voice packet losses are normative to support in the UE. This is true both for Initial Codec Mode (ICM) procedures and dynamic adaptation during an ongoing session.

Therefore, SA4 considers it of utmost importance to ensure that the used RAN GBR (in HPLMN or VPLMN) is never set lower than the lowest configured voice codec mode in the UE.

This reply LS is provided for information.

# 2	Actions

None.

To SA2

ACTION: 	SA4 asks SA2 to take the above information into account.

To CT3 / CT4

ACTION: 	SA4 asks CT3 / CT4 to take the above information into account.

To GSMA NRG

ACTION: 	SA4 asks GSMA NRG to take the above information into account.

# 3	Dates of next TSG SA WG 4 meetings

SA4#121	November 9 – 18, 2022	Europe or e-meeting

SA4#122	February 20 – March 1	Europe or e-meeting