3GPP TSG-SA2 Meeting #154-AH-e	S2-2300105

# Online, , 16th Jan 2023 - 20th Jan 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0597 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for requesting and collecting analytics from MDAF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo Mobile Com. Technology |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CR introduces interactions between the NWDAF and MDAF allowing the NWDAF to request and collect analytics from MDAF based on the conclusions for KI#10 in the TR 23.700-81. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause is added to introduce a procedure for enabling the NWDAF to request and colelct analytics from MDAF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing feature |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.2.x, 6.2.x.1, 6.2.x.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[6]	3GPP TS 22.071: "Location Services (LCS); Service description; Stage 1".

[7]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[8]	ITU-T Y.1540: "Internet protocol data communication service - IP packet transfer and availability performance parameters".

[9]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[10]	3GPP TS 23.003: "Numbering, addressing and identification".

[11]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[12]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[13]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS)".

[14]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[15]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[16]	3GPP TS 38.314: "NR; Layer 2 Measurements".

[17]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[18]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[19]	3GPP TS 23.008: "Organization of subscriber data".

[20]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".

[21]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[22]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[23]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management".

[24]	3GPP TS 28.405: "Telecommunication management; Quality of Experience (QoE) measurement collection; Control and configuration".

[25]	3GPP TS 28.406: "Telecommunication management; Quality of Experience (QoE) measurement collection; Information definition and transport".

[26]	3GPP TS 28.532: " Management and orchestration; Generic management services;".

[27]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[28]	3GPP TS 28.537: "Management and orchestration; Management capabilities".

[29]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[xx]	3GPP TS 28.104: " Management and orchestration; Generic management services;".

* * * * Second Change * * * *

### 6.2.X	Requesting and Collecting Analytics from MDAF

#### 6.2.X.1	General

The MDA functional overview and service framework in Figure 5.1-1 as defined in TS 28.104 [xx] is used by NWDAF to request analytics from an MDA Function.

The NWDAF can use MDA for generating it own analytics, e.g., Observed Service experience analytics, Redundant Transmission Experience analytics or improving the accuracy of analytics by leveraging analytics for fault management predictions/statistics to improve data collection and accuracy of analytics.

Before NWDAF requests analytics from the MDA Function, the NWDAF firstly needs to discover the MDA Function via the MnS discovery service producer as defined in clause 5 of TS 28.537 [28].

#### 6.2.X.2	Procedure for analytics collection from MDAF

Figure 6.2.X.2: Procedure for requesting and collecting analytics from MDAF

Precondition: Initially MDAF(s) or MDA MnS producers register their capabilities, i.e., MnS information or MnS profile as described in clause 5 of TS 28.537 [28] to a MnS discovery service producer. The MnS discovery service producer may contain all or partial information related to the capabilities of MDA MnS producer.

- An analytics consumer issues a request or subscription towards the selected NWDAF as described in clause 6.1.

- The recipient NWDAF determines based on the received analytics request the corresponding attributes related to the MDA request, i.e., analytics type, geographical area, etc.

- NWDAF discovers the MDA Function from the MnS discovery service producer by sending a MnS producer discovery service operation. The service operation may include the following parameters:

-	requestedMDAType: indicate a specific MDA capability such as Slice coverage analysis, Mobility performance analysis as defined in clause 7.2 of TS 28.104 [xx].

-	Area of Interest.

-	Network Slice information (i.e., NetworkSliceInfo including a DN (Distinguished Name) of the NetworkSlice managed object relating to the network slice instance associated to the S-NSSAI and NSI ID if available as defined in TS 28.541 [29]).

4.	The MnS discovery service producer feedback the address of the potential MDA Function(s).

NOTE: NWDAF needs to get the MDARequest IOC from the NRM to be able to provide the information needed in the MDARequest.

- NWDAF uses the address received and issues a request to specific MDA Function(s) to discover the capabilities related to computing, latency, etc., which rely on the current workload.

- Each MDA Function provides in a response the request capabilities.

- NWDAF selects the optimal MDA Function, which can fulfill the desired time schedule provided by the analytics consumer request in step 1.

- NWDAF requests analytics from the MDA Function by using MDARequest as defined in clause 9.3.2 of TS 28.104 [xx]. The MDARequest may include the following parameters:

-	requestedMDAOutputs: MDAOutputPerMDAType (type of analytics) and MDAOutputIEFilters (which assist in indicating explicitly the analytics output requested from the set of MDA outputs and the conditions, considering i) threshold cross, for reporting numeric outputs and ii) timeout, when an output is no longer needed).

-	reportingMethod: method for analytics output (it can only be notification-based reporting since file-based, and streaming are not supported in NWDAF).

-	reportingTarget: indicates the reporting target of the MDA outputs (in case the MDA MnS consumer is different from the one that issued the request).

-	analyticsScope: Managed entities or geographical area information.

-	startTime: indicates the start time of the analytics requested by the MnS consumer.

-	stopTime: indicates the stop time of the analytics requested by the MnS consumer.

-	analyticsWindow: indicates the time duration related to MDA output. It can be in the past, when the analytics is statistics, or in the future for a prediction.

9.	The MDA Function provides the analytics to the NWDAF by triggering an MDAReporting service operation.

10. NWDAF provides the analytics response back to the analytics consumer after processing the MDA Function analytics together with other data receives from NF sources according to the Analytics ID defined in cluse 6.

* * * * End of Change * * * *