3GPP TSG SA WG2 Meeting #154-AHE	S2-2300137

# Elbonia, January 16 – 20, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3706 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Basic description of Service Function Chaining |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | SFC |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The FS_SFC study concluded to re-use N6-LAN traffic steering and it is useful to capture in 23.501 an overall description of the support of service function chaining. FS_SFC also concluded to introduce support for an AF to request the use of a preconfigured SFC for a traffic flow. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the basic description of service function chaining including AF influence on SFC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.X (new), 5.6.X.1 (new), 5.6.X.2 (new), 5.8.2.11.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 5.6.X 	Traffic steering control and service function chaining

#### 5.6.X.1 General

Traffic Steering Control refers to the capability to activate/deactivate traffic steering policies for the purpose of:

-	Service function chaining, i.e. steering subscriber's traffic flows to appropriate operator or 3rd party service functions (e.g. NAT, antimalware, parental control, DDoS protection) in the N6-LAN. This is supported in non-roaming and home-routed scenarios only.

-	AF influenced traffic diversion which enables the routing of the user traffic matching the traffic filters provided in the PCC rule to a local Data Network identified by the DNAI per AF request. This is described in clause 5.6.7.

The PCF controls traffic steering by provisioning and modifying traffic steering control information in PCC rules as described in TS 23.503 [45], e.g. clause 6.1.3.14. In the case of service function chaining, the traffic steering control information consists of a traffic description and, a reference to a traffic steering policy that is configured in the SMF/UPF. A traffic steering policy can be used for the uplink, the downlink or for both directions. In addition to the traffic steering policy, the PCF may also include in the PCC rule metadata provided by the AF. The SMF provides instructions to UPF for traffic steering as further detailed in clause 5.8.2.11.

To enforce the traffic steering policy for service function chaining, the UPF performs deployment specific actions as configured for that traffic steering policy. Based on the configuration, the UPF may for example perform encapsulation with packet marking where, for the traffic identified by the PDR, the UPF provides information for traffic steering, as part of the forwarded packets, to the N6-LAN. This information for traffic steering can then be used for steering the traffic to a set of service functions in a specific order in the N6-LAN. For enforcing the traffic steering policy, the UPF may support traffic steering related functions and user plane encapsulation protocols that are out of 3GPP scope (e.g. as defined by other standards organizations).

The mechanism used for forwarding the traffic between the service functions within the N6-LAN is out of 3GPP scope.

An AF may request the use of a preconfigured service function chain as described in clause 5.6.X.2.

### 5.6.X.2	Application Function influence on Service Function Chaining

An AF may request the steering of user plane traffic to a pre-configured chain of service functions on N6-LAN. The service functions may be deployed by the operator or a 3rd party service provider.

In a non-roaming scenario Application Function influence on Service Function Chain and Application Function influence on traffic routing (as described in clause 5.6.7.1) can be applied to the same traffic simultaneously.

For AF interacting with PCF directly or via NEF, the AF requests may contain the information as described in the Table 5.6.7-1:

Table 5.6.X.2-1: Information element contained in AF request

| Information Name | Applicable for PCF or NEF (NOTE 1) | Applicable for NEF only | Category |
| --- | --- | --- | --- |
| Traffic Description | Defines the target traffic to be influenced, represented by the combination of DNN and optionally S-NSSAI, and application identifier or traffic filtering information. | The target traffic can be represented by AF-Service-Identifier, instead of combination of DNN and optionally S-NSSAI. | Mandatory |
| Target UE Identifier(s) | Indicates the UE(s) that the request is targeting, i.e. an individual UE, a group of UE represented by Internal Group Identifier (NOTE 2), or any UE accessing the combination of DNN and S-NSSAI. | GPSI can be applied to identify the individual UE, or External Group Identifier can be applied to identify a group of UE. | Mandatory |
| Spatial Validity Condition | Indicates that the request applies only to the traffic of UE(s) located in the specified location, represented by areas of validity. | The specified location can be represented by geographical area. | Optional |
| AF transaction identifier | The AF transaction identifier refers to the AF request. | N/A | Mandatory |
| SFC identifier(s) | Indicates the pre-defined Service Function Chain(s) in downlink and/or uplink. | N/A | Mandatory |
| Metadata | Contains information that is passed to UPF and provided by UPF to the service functions in N6-LAN. |  | Optional |
| NOTE 1: When the AF request targets existing or future PDU Sessions of multiple UE(s) or of any UE and is sent via the NEF, as described in clause 6.3.7.2, the information is stored in the UDR by the NEF and notified to the PCF by the UDR. NOTE 2: Internal Group ID can only be used by an AF controlled by the operator and only towards PCF. |  |  |  |

The SFC identifier(s) corresponds to a pre-defined Service Function Chain policy in the PCF. The AF request may include separate SFC identifiers for Uplink and Downlink direction of the user traffic.

NOTE: 	It is assumed that the AF is aware of the SFC identifier(s) based on SLA.

For the PDU Session that corresponds to the AF request, the PCF provides the SMF with a PCC rule(s) that is generated or updated based on the AF request. The PCF sets in the PCC rule(s) the Traffic Steering Policy identifier(s) for N6-LAN traffic steering based on the SFC identifier(s) requested by the AF.

Traffic Steering Policy identifier(s) for both AF influenced Traffic Steering Enforcement Control information and N6-LAN Traffic Steering Enforcement Control information may be included in the PCC rule(s), as described in TS 23.503 [45]. The SMF takes the Traffic Steering Policy identifier(s) in a PCC rule(s) into account when generating or updating N4 rules towards the PSA UPF.

In case the AF provides Metadata, the information is forwarded to SMF in the PCC Rule(s), as described in TS 23.503 [45] and to UPF in N4 rule(s) as described in clause 5.8.2.11.6.

The PCF actions related to the parameters in Table 5.6.X.2-1 other than SFC identifier(s) and Metadata are the same as for the Application Function influence on traffic routing (as defined in clause 5.6.7.1).

**** Next Change ****

##### 5.8.2.11.6	Forwarding Action Rule

The following table describes the Forwarding Action Rule (FAR) that defines how a packet shall be buffered, dropped or forwarded, including packet encapsulation/decapsulation and forwarding destination.

Table 5.8.2.11.6-1: Attributes within Forwarding Action Rule

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to this FAR. | NOTE 9. |
| Rule ID | Unique identifier to identify this information. |  |
| Action | Identifies the action to apply to the packet | Indicates whether the packet is to be forwarded, duplicated, dropped or buffered. When action indicates forwarding or duplicating, a number of additional attributes are included in the FAR. For buffering action, a Buffer Action Rule is also included and the action can also indicate that a notification of the first buffered and/or a notification of first discarded packet is requested (see clause 5.8.3.2). For drop action, a notification of the discarded packet may be requested (see clause 5.8.3.2). |
| Network instance (NOTE 2) | Identifies the Network instance associated with the outgoing packet (NOTE 1). | NOTE 8. |
| Destination interface (NOTE 3) (NOTE 7) | Contains the values "access side", "core side", "SMF", "N6-LAN", "5G VN internal". | Identifies the interface for outgoing packets towards the access side (i.e. down-link), the core side (i.e. up-link), the SMF, the N6-LAN (i.e. the DN), or to 5G VN internal (i.e. local switch). |
| Outer header creation (NOTE 3) | Instructs the UP function to add an outer header (e.g. IP+UDP+GTP, VLAN tag), IP + possibly UDP to the outgoing packet. | Contains the CN tunnel info, N6 tunnel info or AN tunnel info of peer entity (e.g. NG-RAN, another UPF, SMF, local access to a DN represented by a DNAI) (NOTE 8). Any extension header stored for this packet shall be added. The time stamps should be added in the GTP-U header if QoS Monitoring is enabled for the traffic corresponding to the PDR(s). |
| Send end marker packet(s) (NOTE 2) | Instructs the UPF to construct end marker packet(s) and send them out as described in clause 5.8.1. | This parameter should be sent together with the "outer header creation" parameter of the new CN tunnel info. |
| Transport level marking (NOTE 3) | Transport level packet marking in the uplink and downlink, e.g. setting the DiffServ Code Point. | NOTE 8. |
| Forwarding policy (NOTE 3) | Reference to a preconfigured traffic steering policy or http redirection (NOTE 4). | Contains policies identified by TSP ID(s) in the PCC rule. The Forwarding policy may refer to: - an N6-LAN steering policy to steer the subscriber's traffic to the appropriate N6 service functions deployed by the operator, and/or - a local N6 steering policy to enable traffic steering in the local access to the DN according to the routing information provided by an AF as described in clause 5.6.7. - and/or a Redirect Destination and values for the forwarding behaviour (always, after measurement report (for termination action "redirect")). |
| Metadata (NOTE X) | Contains information received by SMF in the PCC rule and used by the UPF when encapsulating traffic sent to N6-LAN. | Metadata can be included only if Forwarding Policy is included. This information is transparent to SMF. |
| Request for Proxying in UPF | Indicates that the UPF shall perform ARP proxying and / or IPv6 Neighbour Solicitation Proxying as specified in clause 5.6.10.2. | Applies to the Ethernet PDU Session type. |
| Container for header enrichment (NOTE 2) | Contains information to be used by the UPF for header enrichment. | Only relevant for the uplink direction. |
| Buffering Action Rule (NOTE 5) | Reference to a Buffering Action Rule ID defining the buffering instructions to be applied by the UPF (NOTE 6) |  |
| NOTE 1: Needed e.g. if: - UPF supports multiple DNN with overlapping IP addresses; - UPF is connected to other UPF or NG-RAN node in different IP domains; - UPF "local switch" and N19 forwarding is used for different 5G LAN groups. NOTE 2: These attributes are required for FAR action set to forwarding. NOTE 3: These attributes are required for FAR action set to forwarding or duplicating. NOTE 4: The TSP ID is preconfigured in the SMF, and used to determine the Forwarding Policy included in the FAR according to the description in clauses 5.6.7 and 6.1.3.14 of 23.503 [45] for local N6 steering and in clauses 5.6.X and 6.1.3.14 of 23.503 [45] for N6-LAN steering. The Forwarding Policy action is enforced before the Outer header creation actions. NOTE 5: This attribute is present for FAR action set to buffering. NOTE 6: The buffering action rule is created by the SMF and associated with the FAR in order to apply a specific buffering behaviour for UL/DL packets requested to be buffered, as described in clause 5.8.3 and clause 5.2.4 of TS 29.244 [65]. NOTE 7: The use of "5G VN internal" instructs the UPF to send the packet back for another round of ingress processing using the active PDRs pertaining to another N4 session of the same 5G VN group. NOTE 8: When in architectures defined in clause 5.34, a FAR is sent over N16a from SMF to I-SMF, the FAR sent by the SMF may indicate that the I-SMF is to locally determine the value of this attribute in order to build the N4 FAR rule sent to the actual UPF controlled by the I-SMF. This is further defined in clause 5.34.6. NOTE 9: In the architecture defined in clause 5.34, the rules exchanged between I-SMF and SMF are not associated with a N4 Session ID but are associated with a N16a association. NOTE X: The use of Metadata is described in clause 5.6.X. How the UPF transforms the metadata into actual information sent with the traffic (e.g. in the encapsulation header) is based on local policies related with the Forwarding Policy and not specified. |  |  |

**** End of Changes ****