3GPP TSG-SA WG2 Meeting #154-AH-e 	S2- 2300147

# Elbonia, January 16-20, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0610 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Federate Learning Procedure Enhancement for Accuracy and Member Selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2022-12-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current description of clause 6.2C “Federated Learning among Multiple NWDAFs”, there are Editor notes regarding information exchanged between FL server and clients, and accuracy of ML model. This CR addresses these editor notes. Moreover, it introduces the member selection step and extends the concept concluded in NOTE2 in clause 8.8 of TR 23.700-81 to improve the accuracy of FL training via detecting data heterogeneity between the server and clients. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Explicit step for FL member selection - Specifying information exchanged between FL server and clients - Adding support of FL accuracy improvement via detecting data heterogeneity between the server and clients |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Outstanding editor notes in the TS and performance degradation of FL because of data heterogeneity |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

## *** Start of Change ***

## 6.2C	Federated Learning among Multiple NWDAFs

### 6.2C.1	Description

This clause specifies how NWDAF containing MTLF can leverage Federated Learning technique to train an ML model, in which there is no need for input data transfer (e.g. centralized into one NWDAF) but only need for cooperation among multiple NWDAFs (MTLF) distributed in different areas i.e. sharing of ML model(s) and of the learning results among multiple NWDAFs (MTLF).

### 6.2C.2	Procedures

#### 6.2C.2.1	General procedure for Federated Learning among Multiple NWDAF Instances

Figure 6.2C.2.1-1: General procedure for Federated Learning among Multiple NWDAF

- The consumer (NWDAF containing AnLF) sends a subscription request to NWDAF containing MTLF to retrieve a ML model, including Analytic ID and ML model filter information as described in TS 23.288 [5], the NWDAF containing MTLF can be a FL server (Server NWDAF) with FL server capability or a MTLF without FL server capability.

Editor’s Note: Procedure of MTLF registration and discovery with respect to FL is FFS.

Editor’s Note: Definition of FL capability is FFS.

1. Server NWDAF selects a subset of Client NWDAFs according to its internal policy (like NWDAF Serving Area) and taking into account the interoperability indicator to participate in federated learning process.

2.	Server NWDAF sends a request to the selected NWDAF containing MTLF (Client NWDAF) that participates in the Federated learning to perform the local model training for Federated Learning.

3.	Each Client NWDAF collects its local data, and optionally the Server NWDAF collects validation data,  by using the current mechanism in clause 6.2 of TS 23.288 [5].

4.	During Federated Learning training procedure, each Client NWDAF further trains the retrieved ML model from the server NWDAF based on its own data, and reports interim local ML model information to the Server NWDAF.

The ML model information are exchanged between the Client NWDAF(s) and the Server NWDAF during the FL training process. The information sent from the Client NWDAF(s) to the Server NWDAF includes:

- The information needed by the Server NWDAF to build the aggregated model based on the locally trained models.

NOTE 1: This information can be the locally trained model itself, or the parameters of local machine learning model (e.g., the weights of neural network), the updates to the local model parameters (e.g., the gradient of the weights of neural network) or etc. considering the interoperability indicator. The details of this information will be specified in Stage 3 specifications.

- Optionally, the accuracy of the locally trained model on the local data of the Client NWDAF.

- Optionally, a set of characteristics (e.g., average, and standard deviation of each dimension) of the local dataset of the Client NWDAF

Editor’ Note: The services in step 2, 4,  6 and 7 to enable FL based ML model training is for FFS, and the Figure 6.2X.2.1-1 will be updated accordingly.

5.	The Server NWDAF builds a global model. First, to guarantee the performance of training, the Server NWDAF attempts to detect data heterogeneity. If the characteristics of the local data of a Client NWDAF is quite different than the characteristics of the Server NWDAF's test data and the accuracy reported by the client is considerably lower than the accuracy of the aggregated model on the test data, this client is marked as non-participant client. Then the server aggregates all the local ML model information retrieved at step 4 except the non-participant clients, to update the global ML model.

6a.	Based on the consumer request, the Server NWDAF updates the training status (the accuracy level of the aggregated model on the test data) to the consumer periodically (one or multiple rounds of training or every 10 min, etc.) or dynamically when some pre-determined status (e.g. some accuracy level) is achieved.

6b.	[Optional] Consumer decides whether the current model can fulfil the requirement e.g. accuracy and time. The consumer modifies subscription if the current model can fulfil the requirement.

6c.	According to the request from the consumer, Server NWDAF updates or terminates the current FL training process.

7.	If the FL procedure continues, Server NWDAF sends the aggregated ML model information to each Client NWDAF for next round model training. It optionally informs each non-participant client that its last updates were not used in this round of FL training.

NOTE 2: The aggregated ML model information sent to clients can be the new ML model itself, the parameters of the new model (e.g., the weights of neural network) or etc. The details of this information will be specified in Stage 3 specifications.

8.	Each Client NWDAF updates its own ML model based on the aggregated ML model information distributed by the Server NWDAF at step 7.

9. [Optional] The non-participant clients may update its local data by using the current mechanism in clause 6.2 of TS 23.288 [5] to resolve the data heterogeneity issue.

NOTE:	The steps 4-9 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

After the training procedure is complete, the Server NWDAF may send the globally optimal ML model information to the consumer.

## *** End of Change ***