3GPP TSG-SA2 Meeting #154AHe eMeeting	S2-2300148

# Elbonia, January 16 – 20, 2023												 (revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3672 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 – 23.502 Solution for 5GC assistance to support Group-MBR Monitoring via NEF Event Exposure Extensions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, Oracle |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 20232-01-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to enable 5GC to assist the Application AI/ML operation traffic to comply to the SLA between the MNO and ASP on the aggregated traffic volume during the group data transmissions, it is essential to optimize and to minimize signaling overhead while allowing MNO to be aware/alerted of/to any crossing of the aggregated throughput threshold which is defined by Group-MBR threshold. Furthermore, it is important to recognize the short lifecycle of the group transmission of the Application AI/ML operation traffic, and that in case of the FL, the AF selected member UEs who participate in the group data transmission could be different and are not static during the entire specific FL operation. The Group-MBR threshold monitoring operation as proposed by this CR is initiated by the AF towards 5GC which enables both 5GC and AF to keep track of the overall aggregated group data transmission volume via the reuse of the existing NEF Event Exposure service with added new event, i.e. Group-MBR Monitoring. A new event, i.e. QoS Monitoring for Bit Rate Reporting for specific QoS flow, is also added to the existing UPF Event Exposure service. Leveraging the consolidated signaling request and response via the NEF Event Exposure service between the AF and 5GC (i.e. signaling optimization) for a group of target UEs enables the reliable operation for both 5GC and AF to detect the crossing of the Group-MBR threshold. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduces a new event, Group-MBR Monitoring, to NEF Event Exposure service - Introduces a new event, QoS Monitoring for Bit Rate Reporting for specific QoS flow, to UPF Event Exposure service - Updates the QoS Monitoring PCC rules to include also the bit rate reporting for specific QoS flow |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not able to provide a reliable and optimal mechanism to address the operator’s requirement to monitor the Application AI/ML traffic volume to determine the impacts towards non-Application AI/ML traffic. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.3.2.x, 5.2.5.3.2, 5.2.5.3.3, 5.2.26.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y | X | Other core specifications |  |  |  | TS/TR 23.503. CR 0823 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * * (All New)

##### 4.15.3.2.X	Information flow for Group-MBR Monitoring support by NEF

This clause describes how AF subscribes to the new Group-MBR monitoring event from NEF Event Exposure service which is responsible for coordinating a group of UPFs to report their respective bit rates of a specific set of QoS flows that are specified by the AF.  The AF identifies the set of QoS flows among the group of PDU sessions which serve a group of target UEs selected by the AF to support the specific application group transmission.   The AF may update the Group-MBR threshold, the frequency of the reporting and the maximum duration of the reporting which may have been provisioned in the UDR of the 5GC.

The PCF(s) which serve the target UE(s) will be requested by the NEF to update via the SMF the N4 policy to trigger or to terminiate the QoS Monitoring for Bit Rate Reporting for specific QoS flow(s) as the result of activating or terminiating the Group-MBR monitoring event, respectively.   The PCF will then trigger the N4 rules update between the respective SMF(s) and the UPF(s) which serve the target UE(s) to initiate/remove the new individual UPF event, i.e. QoS Monitoring for Bit Rate Reporting for specific QoS flow(s).

Based on the specified Frequency of Reporting cycle and the Target of Reporting, the UPF will report the bit rate for the specific QoS flow to the target NEF.   The NEF will then perform the aggregation among the group of reports from the UPFs and determine whether the aggregated bit rate exceeds the Group-MBR threshold.  If the aggregated bit rate exceeds the Group-MBR threshold, the NEF will then notify the AF for the subscribed event.

Figure 4.15.3.2.X-1: Information Flows of NEF Event Exposure coordination with UPF Event Exposure to support Group-MBR Monitoring

1.	Before the target Application group transmission operation is about to start, the AF which controls the operation initiates the Group-MBR monitoring event by subscribing/unsubscribing to the NEF Event Exposure Service with 5G Core as described in clause 4.15.3.1 for the specific set of QoS flows of a group of PDU sessions that support a group transmission for an Application (e.g. FL operation).

The AF’s subscribe request includes the Application Identifier, the target event which is the Group-MBR monitoring, the group of target UEs identified by their IP addresses, the Application Identifier and the traffic filtering information, DNN, S-NSSAI, as well as the Maximum Duration of Reporting, Group Reporting Guard Time,  Event reporting mode set to reporting when threshold is reached and Reporting Threshold as defined in the Event Reporting Information as specified in clause 4.15.1.   More specifically, the Reporting Threshold is corresponding to the Group-MBR threshold specified by the AF for the condition to receive the event notification from the NEF that the Group-MBR threshold is crossed.  The Group-MBR threshold may have different values for uplink and downlink. In addition, the Notification Target Address (+ Notification Correlation ID) is included.

The AF’s unsubscribe request includes the Notification Correlation ID.

2.&3. NEF authorizes the AF request.  The NEF determines the UEs’ corresponding serving PCF addresses based on the local configuration or the NEF consumes Nbsf_Management Discovery service operation to find out the address(es) of the UEs’ serving PCFs.

4.&5. Once the UE(s)’ serving PCF(s) are identified, the NEF invokes the Npcf_PolicyAuthorization_Create/Update service to each UE’s serving PCF to convert the AF’s request into specific QoS flow monitoring event and includes the UE IP address, DNN and S-NSSAI, Application Identifier, traffic filtering information, Maximum Duration of Reporting, Reporting frequency derived from Group Reporting Guard Time, Reporting Threshold, AF Transaction Id derived from the Notification Correlation ID and Target of reporting derived from the Notification Target Address.

If the NEF invokes the Npcf_PolicyAuthorization_Delete service, NEF notifies each UE’s serving PCF to remove the N4 policy by referring to the AF Transaction Id which is derived from the Notification Correlation ID,

The serving PCF authorizes the NEF request and responds to NEF with Npcf_PolicyAuthorization response accordingly.

6. 	NEF responds to the AF for the NEF_EventExposure subscribe/unsubscribe request for Group-MBR monitoring.

7.&8. 	If NEF requested Npcf_PolicyAuthorization_Create/Update is authorized by the PCF, the  PCF triggers the QoS Monitoring for Bit Rate Reporting per QoS flow and updates the associated SMF with corresponding policy information as described in clauses 6.1.3.21 and 6.3.1 of TS 23.503 [20] to trigger QoS Monitoring for Bit Rate Reporting per QoS Flow event for the corresponding PDU Session by invoking Npcf_SMPolicyControl_UpdateNotify service operation towards the SMF.  The target of the QoS Monitoring report is NEF.

If NEF requested Npcf_PolicyAuthorization_Delete is authorized by the PCF, the  PCF triggers Npcf_SMPolicyControl_Delete service operation towards the SMF.

9.&10.   If the SMF receives the create or update of the policy information for the PDU session provided by the PCF as described in step 7 above, SMF initiates the requested action to trigger the QoS Monitoring for Bit Rate Reporting per QoS Flow in PSA UPF over N4 for the UPF service as described in clause 5.33.3 and in clause 5.2.26.2 of TS 23.501[2].

NOTE: The SMF first performs QoS Flow binding as described in TS 23.503[20] clause 6.1.3.2.4.

If the SMF receives the deletion of the policy information for the PDU session, the SMF notifies the PSA UPF to terminate the QoS Monitoring for Bit Rate Reporting per QoS Flow.

SMF acknowledges the PCF request with a Npcf_SMPolicyControl_UpdateNotify response.

11.  When received the PCF request, the SMF is then to provision the PSA UPF to remove the reporting, or to initiate the reporting of the bit rate for the specific QoS flow via Nupf_EventExposure_Notify to the NEF periodically in accordance with the Reporting frequency until Maximum Duration of Reporting is expired, or the NEF explicitly cancel the event.

12.	If receiving the QoS Monitoring for Bit Rate Reporting per QoS Flow from UPFs, the NEF sums up the bit rates of all the active QoS flows during the Group Reporting Guard Time.  If the given Application AI/ML traffic is asymmetric, the bit rate aggregation is done per uplink and downlink separately; otherwise, the bit rate aggregation is done per either uplink or downlink. NEF compares the aggregated bit rate of the uplink or downlink against the provisioned Group-MBR threshold for the uplink or downlink, accordingly.

13. If the aggregate bit rate exceeds the Group-MBR threshold specified by the AF, the NEF will notify the Application AI/ML AF via Nnef_EventExposure_Notify with the AF transaction Identifier.

14. AF may apply traffic mediation action in responding to the Group-MBR monitoring report (e.g. requesting 5G Core to apply traffic gating on specific QoS flow such as asking 5GC to block some of the flows) or may decide not to take any action other than using it for its own statistics or charging. The decision is local implementation decision is not within the scope of this procedure.

15. The UPF bit rate monitoring event continues until the time for Maximum Duration of Reporting is expired, the time left until the Maximum duration of reporting is less than the Group Reporting Guard Time, or AF terminates the Group-MBR monitoring. The Group Reporting Guard Timer will restart and the UPF continues to perform the Bit Rate Reporting for the given QoS flow to the NEF periodically as described in steps 11-12 above.

If AF wants to terminate the Group-MBR monitoring, AF sends either Nnef_EventExposure_Unsubcribe as described in clause 4.15.3.1.

* * * Next Changes * * *

##### 5.2.5.3.2	Npcf_PolicyAuthorization_Create service operation

Service operation name: Npcf_PolicyAuthorization_Create

Description: Authorize the request and optionally determines and installs SM Policy Control Data according to the information provided by the NF Consumer or provides Port Management Information Container for ports on DS-TT or NW-TT, or User plane node Management Information Container.

Inputs, Required: UE (IP or MAC) address, identification of the application session context.

Inputs, Optional: GPSI or SUPI if available, Internal Group Identifier, DNN if available, S-NSSAI if available, Media type, Media format, bandwidth requirements, sponsored data connectivity information if applicable, flow description, AF Application Identifier, AF Communication Service Identifier, AF Record Identifier, Flow status, Priority indicator, emergency indicator, ASP Identifier, resource allocation outcome, AF Application Event Identifier, a list of DNAI(s) and corresponding routing profile ID(s) or N6 traffic routing information, AF Transaction Id, Early and/or late notifications about UP path management events, temporal validity condition, spatial validity condition, Information for EAS IP Replacement in 5GC, Indication for EAS Relocation, AF indication for simultaneous connectivity over source and target PSA at edge relocation as described in clause 5.6.7 in 23.501 [2], Background Data Transfer Reference ID, priority sharing indicator as described in clause 6.1.3.15 in TS 23.503 [20], pre-emption control information as described in clause 6.1.3.15 in TS 23.503 [20], Port Management Information Container and related port number, User plane node Management Information Container, TSN AF parameters provided by the TSN AF to the PCF as described in clause 6.1.3.23 of TS 23.503 [20], Requested Alternative QoS Parameter Set(s), QoS parameter(s) to be measured, Maximum Duration of Reporting, Reporting frequency, Target of reporting and optional an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20], individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20], Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order), MPS for Data Transport Service indicator as described in clause 6.1.3.11 of TS 23.503 [20].

NOTE:	When only one DNAI and corresponding routing profile ID(s) and the Indication for EAS Relocation are available, the presented DNAI is the target DNAI as defined in clause 6.3.7 of TS 23.548 [74].

Outputs, Required: Success or Failure (reason for failure, e.g. as defined in clauses 6.1.3.16 and clause 6.1.3.10 of TS 23.503 [20]).

Outputs, Optional: The service information that can be accepted by the PCF.

##### 5.2.5.3.3	Npcf_PolicyAuthorization_Update service operation

Service operation name: Npcf_PolicyAuthorization_Update

Description: Provides updated information to the PCF.

Inputs, Required: Identification of the application session context.

Inputs, Optional: Media type, Media format, bandwidth requirements, sponsored data connectivity information if applicable, flow description, AF Application Identifier, AF Communication Service Identifier, AF Record Identifier, Flow status, Priority indicator, resource allocation outcome, AF Application Event Identifier, a list of DNAI(s) and corresponding routing profile ID(s) or N6 traffic routing information, AF Transaction Id, Early and/or late notifications about UP path management events, temporal validity condition, spatial validity condition, Information for EAS IP Replacement in 5GC, Indication for EAS Relocation, AF indication for simultaneous connectivity over source and target PSA at edge relocation as described in clause 5.6.7 of TS 23.501 [2], Background Data Transfer Reference ID, priority sharing indicator as described in clause 6.1.3.15 of TS 23.503 [20], pre-emption control information as described in clause 6.1.3.15 of TS 23.503 [20], Port Management Information Container and related port number, User plane node Management Information Container, TSN AF parameters provided by the TSN AF to the PCF as described in clause 6.1.3.23 of TS 23.503 [20], individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20], Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order), Requested Alternative QoS Parameter Set(s), QoS parameter(s) to be measured, Maximum Duration of Reporting, Reporting frequency, Target of reporting and optional an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20], MPS for Data Transport Service indicator as described in clause 6.1.3.11 of TS 23.503 [20].

NOTE:	When only one DNAI and corresponding routing profile ID(s) and the Indication for EAS Relocation are available, the presented DNAI is the target DNAI as defined in clause 6.3.7 of TS 23.548 [74].

Outputs, Required: Success or Failure (reason for failure, e.g. as defined in clause 6.1.3.16 of TS 23.503 [20]).

Outputs, Optional: The service information that can be accepted by the PCF.

Provides updated application level information and communicates with Npcf_SMPolicyControl service to determine and install the policy according to the information provided by the NF Consumer. Updates an application context in the PCF.

* * * Next Changes * * *

##### 5.2.26.2.1	General

Service description: This service can expose UPF related information to other NFs. There is one operation for this service:

-	Notifying events on the PDU Session to the NFs.

The following events can be notified to a NF consumer:

-	QoS Monitoring for URLLC: the event notification may contain the QoS Monitoring report as described in clause 5.33.3.2 of TS 23.501 [2].

The event notification may contain following information:

-	QoS monitoring result e.g. end to end delay for specific QoS flow or for specific PDU session.

-	QoS Monitoring for Bit Rate Reporting for specific QoS Flow:

The event notification contains the bit rate for specific QoS Flow which could be GBR or non-GBR QoS flow as described in 5.16.X.1 of TS 23.501[2]. The SMF may provision the following information in UPF to support the reporting:

-	Traffic filter to indicate the target QoS flow required for the reporting

-	Frequency of reporting

-	Maximum Duration of Reporting.

-	Target of the QoS Monitoring Report

-    Indication for direct event notification

* * * End of Changes * * *