3GPP TSG-SA2 Meeting #154AHe	S2-2300152

# Elbonia, January 16 – 20, 2023													 (revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3674 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AIMLsys: KI#7 – NEF services description for assistance to member selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, Sony |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to enable 5GC to assist the Application AI/ML AF to preform candidate member selection, this CR defines a new NEF services for assistance to member selection. The solution describes the member selection assistance functionality is hosted in the NEF. The AF can request the NEF to provide a list of recommended UEs based on new services supported by the NEF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a new Network Capabilities for external exposure. - Describe a new member selection assistance procedure. - Add a new service provided by the NEF and the corresponding service operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not able to provide a 5GC mechanism to support the member selection to assist the Application AI/ML operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.2, 4.15.X, 5.2.6.1, 5.2.6.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is part of outcome of SI FS_AIMLsys. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 4.15.2	External Exposure of Network Capabilities

The Network Exposure Function (NEF) supports external exposure of capabilities of network functions. External exposure can be categorized as Monitoring capability, Provisioning capability, Policy/Charging capability, network status reporting capability, Analytics reporting capability and member selection assistance capability. The Monitoring capability is for monitoring of specific event for UE in 5GS and making such monitoring events information available for external exposure via the NEF. The Provisioning capability is for allowing external party to provision of information which can be used for the UE in 5GS. The Policy/Charging capability is for handling QoS and charging policy for the UE based on the request from external party. The Analytics capability is for allowing external party to acquire analytics information generated by 5G System. The member selection assistance capability is for allowing external party to acquire a list of candidate UEs recommend by the 5G System.

The details for the External Exposure of Analytics capabilities as well as interactions between NEF, external party and NWDAF are described in TS 23.288 [50].

* * * Next Changes * * *

### 4.15.X	Member selection assistance

The member selection assistance functionality hosted by the NEF is to provide a list of candidate UE based on the request receiving from the AF. NEF identifies which information need to be collected based on the criteria in the AF request and collect the corresponding information from other NFs, separately. NEF consolidates all the information collected from other 5GC NFs to derive a candidate UE list which meets the criteria requested by the AF and sends the candidate UE list to the AF. The member selection assistance functionality can be used to assist third party to select the group of UEs to support application service (e.g. application FL operation).

Figure 4.15.X-1: 5GC assistane to Member selection

1-2.	If the AF has not discovered its serving NEF, it queries NRF including the AF instance ID, Application Identity and member selection service indication to retrieve its serving NEF that supports the member selection assistance functionality.

3.	A PDU Session between the UE and the AF may have been established.

4.	AF subscribes the member selection assistance functionality by sending Nnef_ UESelectionAssistance_subscribe including the Application Identity, UE list and one or more criteria listed in the table 4.15.X-1.

5.	NEF verifies the authorization of the AF Request and maps the criteria into the corresponding UE filtering information, NEF identifies which information need to be collected and proceeds with the corresponding procedures which may involve a composite set of 5GC service procedures corresponding to a set of 5GC events, analytics and/or notifications (e.g. UPF, PCF UDM, NWDAF, AMF) listed in the table 4.15.X-1.

6.	NEF sends the Nupf_EventExposure_Subscribe to the UPF including the different event ID to collect the QoS monitoring, Throughput UL/DL and Packet transmission/ retransmission related information.

7.	NEF sends the Nudm_SDM_Subscribe to the UDM including the list of SUPIs or GPSIs to collect the S-NSSAI, DNN and MA PDU session related information.

8.	NEF sends the Npcf_SMPolicyControl_Subscribe to the PCF including the list of SUPIs or GPSIs to collect the QoS configuration related information.

9.	NEF sends the Nnwdaf_Analytics_Subscription to the NWDAF including the different analytics ID to collect the analytics related information.

10.	NEF sends the Namf_Communication_UEContextTransfer _Subscribe or the Namf_EventExposure_Subscribe to the AMF including the list of SUPIs or GPSIs and the AoI to collect the UE location and UE status related information.

11.	Based on the collected information from other 5GC NFs, NEF derives the candidate UE list which fulfill the criteria in the AF request.

12.	NEF sends the response to the AF using the Nnef_UESelectionAssistance_notify including the Application Identity and candidate UE list.

#### 4.15.X.1	5GS assistance for FL member selection based on UE's travel history, direction and separation distance.

Figure 4.15.X.1-1: 5GC assistane to Member selection based on the UE’s travel history, direction and separation distance

- AF requests 5GS assistance to support the UE selection by considering the UE's historical location, UE's current location, separation distance and direction. AF includes the UE lists and the following criteria as part of the FL UE selection request:

    - UE historical location: The Target AOI where the UEs have been roving over the historical nomadic period before moving into the FL coverage area.

    - FL Coverage Area: The FL training server's coverage area, i.e. the coverage area of the FL training server where the selected UEs participate in the FL operation

    - Separation distance: Select those UEs that are geographically separated, fulfilling the separation distance no smaller than the certain separation distance

    - Direction: Select the UE with the different direction in the FL coverage Area.

When providing a target area for FL, the AF may provide sub-areas, and provide a maximum number of UEs that should take part in FL from each sub-area.

2.	NEF translates the GPSIs to SUPIs and maps the criteria into the corresponding UE filtering information, NEF identifies which information need to be collected and proceeds with the corresponding procedures which may involve a composite set of 5GC service procedures corresponding to a set of 5GC events, analytics and/or notifications (e.g. UPF, PCF UDM, NWDAF, AMF) listed in the table 4.15.X-1.

3-4. The NEF invokes Namf_EventExposure_Subscribe service operation with the UE list, event ID = Number of UEs present in a geographical area. AMF will provide a list of UEs that are within the FL coverage area to the NEF using the Namf_EventExposure_Notify service operation. The NEF obtains the list of possible targets UEs from AMF within the FL training server's coverage.

5.	In order to identify the appropriate NWDAF which can provide analytic output to derive the visited AOI info and to get the UE direction for the possible target UEs above, the NEF needs to select the suitable NWDAF by initiating the NWDAF discovery request (Analytics ID = UE mobility, AOI = Target AOI) with UE list.

6-7. The NEF invokes NWDAF Analytics Info request (UE list, Analytics ID = UE mobility, Filters include "Visited AoI = Target AOI" and "target period = historical nomadic period"). NWDAF will then provide a list of UEs that were ever roving within the target AOI, at the minimum, over the historical nomadic period NWDAF and also provide a list of UEs location in the order of which the UE passes through to the NEF. Then the NEF gets the corresponding statistics of UE mobility and the UE's direction.

8-9. The NEF compares the UE list provided by the AMF in steps 3-4 against the UE list provided by the NWDAF in steps 6-7. The NEF will then determine the FL candidate UEs which are now within the FL coverage area but were roving within the target AOI over the historical nomadic period and UE with the different direction and in the different sub-area within the FL coverage Area as requested by AF. The NEF notifies AF for such UE candidate list.

10.	The NEF also needs to consider the list of UEs which are now within the FL coverage area, but may move out of the FL coverage area. Therefore, for each UE in the candidate list in steps 8-9, the NEF invokes the Namf_EventExposure_Notify service with UE ID = SUPI, Event ID=UE moving in/out of AOI (PRA) to keep track the movement of the UE(s) in/out of the AOI.

11-12.	If step 10 identifies any UE which is moving out of the FL coverage area, the NEF may further notify AF of the given UE which is moving out the coverage.

* * * Next Changes * * *

#### 5.2.6.1	General

The following table shows the NEF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_EventExposure | Subscribe | Subscribe/Notify | AF, NWDAF |
|  | Unsubscribe |  | AF, NWDAF |
|  | Notify |  | AF, NWDAF |
| Nnef_PFDManagement | Fetch | Request/Response | SMF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Notify |  | SMF |
|  | Unsubscribe |  | SMF |
|  | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_EASDeployment | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Unsubscribe |  | SMF |
|  | Notify |  | SMF |
| Nnef_ParameterProvision | Update | Request/Response | AF |
|  | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_Trigger | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
| Nnef_BDTPNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_TrafficInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | AppRelocationInfo | Subscribe/Notify | AF |
| Nnef_ChargeableParty | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_AFsessionWithQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Revoke | Request/Response | AF |
| Nnef_MSISDN-less_MO_SMS | Notify | Notify | AF |
| Nnef_ServiceParameter | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_APISupportCapability | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe | Subscribe/Notify | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_NIDDConfiguration | Create | Request/Response | AF |
|  | TriggerNotify | Subscribe/Notify | AF |
|  | UpdateNotify | Subscribe/Notify | AF |
|  | Delete | Request/Response | AF |
| Nnef_NIDD | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
|  | GroupDeliveryNotify | Notify | AF |
| Nnef_SMContext | Create | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
|  | DeleteNotify | Subscribe/Notify | SMF |
|  | Delivery | Request/Response | SMF |
| Nnef_AnalyticsExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
|  | Fetch | Request/Response | AF |
| Nnef_UCMFProvisioning | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ECRestriction | Get | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ApplyPolicy | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_Location | LocationUpdateNotify | Notify | AF |
| Nnef_TimeSynchronization | ConfigUpdate | Request/Response | AF |
|  | ConfigCreate | Request/Response | AF |
|  | ConfigDelete | Request/Response | AF |
|  | ConfigUpdateNotify | Subscribe/Notify | AF |
|  | CapsSubscribe | Subscribe/Notify | AF |
|  | CapsUnsubscribe | Subscribe/Notify | AF |
|  | CapsNotify | Subscribe/Notify | AF |
| Nnef_ASTI | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_AMPolicyAuthorization | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | Subscribe |  | AF |
|  | Unsubscribe |  | AF |
| Nnef_AMInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_UEId | Get | Request/Response | AF |
| Nnef_SMService | MoForwardSm | Request/Response | SMS-SC |
| Nnef_PDTQPolicyNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_UESelectionAssistance | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |

* * * Next Changes * * *

#### 5.2.6.X	Nnef_ UESelectionAssistance service

##### 5.2.6.X.1	General

Table 5.2.6.X.1-1: Description of UE Member filtering criteria corresponding to UE filtering information

| UE Member filtering criteria | Description | UE filtering information |
| --- | --- | --- |
| Time window | IndicateTime window for performing the AI/ML operation | Analytics Target period |
| QoS and performance related information |  |  |
| QoS configuration in PCF | Indicate the selected UEs should meet the required QoS in the specified areas | Npcf_SMPolicyControl service Filter: a list of GPSI(s) or SUPI(s) |
| QoS monitoring | Indicates monitored QoS performance for the selected UE should meet the threshold | Nupf_EventExposure Service Filter: Event ID= QoS monitoring |
| Expected QoS estimate in the area | Indicate the selected UE in a certain area regarding the QoS change statistics for an Analytics target period in the past or the likelihood of a QoS change for an Analytics target period in the future. | Nnwdaf_AnalyticsSubscription Filter: Analytics ID= QoS Sustainability |
| Throughput UL/DL | Indicate the bit rates for the selected UE should meet the threshold | Nupf_EventExposure Service Filter: Event ID= Throughput UL/DL |
| Packet transmission/ retransmission | Indicate the packet delay for the selected UE should meet the threshold | Nupf_EventExposure Service Filter: Event ID= Packet transmission/ retransmission |
| UE TransmissionLatency Performance | Indicate the UE transmission latency performance refers to a time delay for completing the transmission of a specific data volume between UE to AF | Nnwdaf_AnalyticsSubscription Filter: UE Transmission Latency Performance |
| Location related information |  |  |
| UE current location | Indicate the certain area that the selected UE appears | Namf_EventExposure service operation Filter: a list of GPSI(s) or SUPI(s) |
| UE historical location | Indicate the certain area that the selected UE appeared in a historical period of time | Nnwdaf_AnalyticsSubscription Filter: Analytics ID = UE mobility, Filters include "Visited AoI = Target AOI" and "target period = historical nomadic period |
| UE trajectory | Indicate the certain area trajectory that the selected UEs move in or out in the past or in the future | Nnwdaf_AnalyticsSubscription Filter: Analytics ID= UE Mobility |
| UE direction | Indicate the selected UEs should include different moving direction | Nnwdaf_AnalyticsSubscription Filter: Analytics ID= UE Mobility, Direction |
| UE separation distance | Indicate the selected UEs should comply with a minimum separation distance with each other | Namf_EventExposure service operation Filter: a list of GPSI(s) or SUPI(s) |
| UE status |  |  |
| UE mobility | Indicate whether the selected UE is moving out of the certain area. | Namf_EventExposure service operation Filter: a list of GPSI(s) or SUPI(s), AoI |
| CM state | Indicate the selected UE should in the CM-CONNECTED state | Namf_Communication_UEContextTransfer service operation Filter: a list of GPSI(s) or SUPI(s) |
| UE Reachability | Indicate the selected UE should reachable | Namf_Communication_UEContextTransfer service operation Filter: a list of GPSI(s) or SUPI(s) |
| UE Abnormal behaviour | Indicate the selected UE should have no abnormal behaviour. | Nnwdaf_AnalyticsSubscription Filter: Analytics ID= UE Abnormal behaviour |
| UE access type | Indicate the selected UE should support the 5GS and WLAN access. | Namf_Communication_UEContextTransfer service operation Filter: a list of GPSI(s) or SUPI(s) |
| PDU session attributes |  |  |
| S-NSSAI, DNN | Indicate the S-NSSAI and DNN the selected UE should support | Nudm_SubscriberDataManagement (SDM) Service Filter: a list of GPSI(s) or SUPI(s) |
| MA PDU Session | Indicate the selected UE should support the MA PDU session | Nudm_SubscriberDataManagement (SDM) Service Filter: a list of GPSI(s) or SUPI(s) |

Service description: This service provides candidate UE selection assistane functionalities.

##### 5.2.6.X.2	Nnef_ UESelectionAssistance _Subsrcribe operation

Service operation name: Nnef_ UESelectionAssistance_Subsrcribe

Description: AF requests a candidate UE selection assistane functionalities.

Inputs, Required: Application Identity, UE list (GPSIs, SUPIs, or IP addresses).

Inputs, Optional: list of criteria shown in the clasue 5.2.6.X.1 Table 5.2.6.X.1 -1.

Outputs, Required: Candidate UE list (GPSIs, SUPIs, or IP addresses).

Output, Optional: None.

See clause 4.15.X.1 for examples of this service operation to support the member selection assistance based on the UE location related criteria requested by the AF.

* * * End of Changes * * *