3GPP TSG-SA2 Meeting #154-AH-e	S2-2300211

# Online, , 16th Jan 2023 - 20th Jan 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0612 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Key Issue #9: Analytic ID that supports location accuracy estimate |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The key issue #9 conclusions in TR 23.700-81 contain the following: The following use cases will be addressed in normative work: … - NWDAF supports the following new Analytics IDs: … - Analytic ID that supports location accuracy estimate use case. NOTE 1: That new Analytics IDs to support these use cases are required (instead of enhancing Analytics IDs) will be confirmed during normative work. … To benefit LCS system, the NWDAF provides a new analytic ID with location accuracy estimates as required by the conclusions of the eLCS-Ph3 study. NOTE 6: The FS_eLCS_ph3 in TR 23.700-71 [30] contains a conclusion for Key Issue 4 to use this analytics. Solution #59 ("Enhancement of NWDAF with location accuracy prediction") contains more related details. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An Analytic ID that supports location accuracy estimates, as required by key issue #9 conclusions in TR 23.700-81, is being defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An Analytic ID that supports location accuracy estimates, as required by the conclusions of FS_eNA_Ph3 and FS_eLCS_ph3, is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************************ START OF CHANGE (all new text) *****************************

## 6.x	Location Accuracy Analytics

### 6.x.1	General

This clause specifies how an NWDAF can provide Location Accuracy Analytics which provides analytics for LCS QoS accuracy to an analytics consumer such as LMF.

The Location Accuracy Analytics provides one or a combination of the following information:

-	location accuracy (horizontal mandatory and optional vertical)

-	Indication on whether a UE is indoor or outdoor when a location measurement is made (optional)

-	Applicable area for location accuracy prediction (optional)

-	requested Confidence of location accuracy prediction (optional).

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = "Location Accuracy";

-	Target of Analytics Reporting: "any UE";

-	Location (horizontal and optional vertical) or area of interest;

-	Meassurement method;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

### 6.x.2	Input Data

A training should be first performed with correlated positioning estimates obtained by two positioning methods, one of them (e.g. GPS information) used as ground truth to evaluate the accuracy of the other, see Table 6.x.2-1.

Table 6.x.2-1: Data Collected by NWDAF for Location Accuracy Analytics

| Information | Source | Description |
| --- | --- | --- |
| (horizontal and optional vertical) UE location determined by two positioning methods | LMF, OAM (via MDT), | UE location as estimated by the LMF with the selected localization method. |
| UE ID | LMF, OAM (via MDT), | UE identifier. |
| Location methods/Assistance Data | LMF, OAM (via MDT), | Selected location method and corresponding parameters. |
| Area | LMF | The region of interest where the UE is located (e.g. could be list of cell IDs). |
| Indoor/outdoor indication | LMF | Indication of indoor/outdoor when LMF can distinguish indoor/outdoor environment. |

### 6.x.3	Output Analytics

The Location Accuracy analytics are shown in table 6.x.3-1 and table 6.x.3-2.

Table 6.x.3-1: Location Accuracy statistics

| Information | Description |
| --- | --- |
| List of Location Accuracy Analytics (1..max) |  |
| >Applicable Area or location (optional) | A list of TAIs or Cell IDs or below cell level location information that the analytics applies to. May be omitted if information applies for entire requested area or if statistics for a single location were requested. |
| >Applicable Time Period | The time period within the Analytics target period that the analytics applies to. |
| > Positioning Method | The positioning method used to measure location |
| > analytics accuracy (horizontal and optionaly vertical) | Statistics about the horizontal and optionally vertical location accuracy of applicable positioning method at at indicated location or location area. |
| > Percentage of UE in indoor/outdoor in target area (optional) | Provides a ration of UEs that are indoor/outdoor in the target area |
| > Indication whether location is indoor/outdoor (optional) | If a single location is location accuracy statistics, indication whether that location is indoor or outdoor |
| >Confidence | Confidence of the statistics |

Table 6.x.3-2: Location Accuracy predictions

| Information | Description |
| --- | --- |
| List of Accuracy sustainability Analytics (1..max) |  |
| >Applicable Area or location (optional) | A list of TAIs or Cell IDs or below cell level location information that the analytics applies to. May be omitted if information applies for entire requested area or if statistics for a single location were requested. |
| >Applicable Time Period | The time period within the Analytics target period that the prediction applies to. |
| > Positioning Method | The positioning method used to measure location |
| > analytics accuracy (horizontal and optionaly vertical) | predictions about the horizontal and optionally vertical location accuracy of applicable positioning method at at indicated location or location area. |
| > Percentage of UE in indoor/outdoor in target area (optional) | Provides a predicted ratio of UEs that are indoor/outdoor in the target area |
| > Indication whether location is indoor/outdoor (optional) | If a single location is location accuracy statistics, indication whether that location is predicted to be indoor or outdoor |
| >Confidence | Confidence of the prediction. |

### 6.x.4	Procedures to request Location Accuracy Analytics

Figure 6.59.2-1: Location accuracy prediction and usage

0.	Pre-condition: NWDAF has a trained supervised ML model for predicting location accuracy.

1.	LCS Client requests from LMF the UE location with QoS Class set as either "Multiple QoS Class" or "Assured" along with the location accuracy required.

2.	The LMF initiates the LCS session and derives the UE location estimate.

3.	The LMF queries the NWDAF for the location accuracy giving as inputs the corresponding Analytics ID and UE location estimate or corresponding area (cell ID/TAI).. Other inputs can be UE positioning method and assistance data.

NOTE 1:	Both Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo services can be used for step 3. above. The Nnwdaf_AnalyticsSubscription service can be used by a service consumer to receive notifications about location accuracy, e.g. when a change is detected.

4.	The NWDAF uses the corresponding model and provides the location accuracy as output to the LMF.

5.	LMF compares the location accuracy received in step 4 with the required location accuracy received in step 1 from the LCS Client.

6-7.	If the required location accuracy is not met, the LMF takes subsequent actions like re-executing the LCS procedure with more stringent parameters.

7.	The LMF returns the location estimate with the required accuracy.

.

************************************ END OF CHANGE ****************************************