SA WG2 Meeting #S2-154-AH-e(e-meeting)	S2-2300228

16 - 20 January 2023, Electronic Meeting	(revision of S2-22xxxxx)

Source:	Ericsson

Title:	KI#4, Conclusion updates

Document for:	Approval

Agenda Item:	9.3.1

Work Item / Release:	FS_PIN/Rel-18

Abstract of the contribution: Introduction conclusion updates for KI#4.

# 1. Introduction

This paper introduces conclusion updates for KI#4

# 2. Discussion

There are three ENs under the conclusions for KI#4

Editor's note:	It is FFS whether more parameters are needed in non-3GPP QoS assistance information.

For this EN, there is no further non-3GPP QoS assistance information discussion so far besides the information defined in 24.502. This EN can be removed from the TR.

Editor's note:	It is FFS whether one PDU session can serve more than one PIN.

For this EN, it shall be allowed to use one PDU session for different PINs. However, this shall be allowed in the pre-condition that there is no need of differentiation between PINs for the QoS control, routing steering in the 5GC.

Editor's note:	It is FFS whether PEMC UE needs a specific 5G subscription for providing PIN service.

For this EN, it’s addressed in conclusion for KI#5 and it can be removed in this KI#4.

Based on above, it’s proposed to support the conclusion updates below for KI#3.

# 3. Proposal

This paper proposes the following conclusion points to TR 23.700-88 clause 8.4.

* Start of change *

## 8.4	Conclusion on Key Issue #4

The normative work is based on the following principles

1)	When the communication between a PEMC and a PINE behind a PEGC takes place via 5GC, or when the communication between PINEs requires multiple PEGCs and 5GC, the existing traffic forwarding functionalities in 5GS via UPF(s) or N6 can be applied if available.

2)	Non-3GPP QoS assistance information (including QoS characteristics, GFBR/MFBR UL/DL, MPLR UL/DL) that contains the same parameters as the Additional QoS Information specified in table 9.3.1.1-2 of TS 24.502 [9] may be sent to PEGC from SMF to assist the deriving of N3GPP QoS parameters for PIN.

a)	Whether and how PEGC performs the deriving of QoS parameters and mapping procedure to be applied between the PINE and the PEGC is implementation specific and therefore it is not specified by 3GPP.

3)	Differentiated traffic routing and QoS control may be required by a PEGC.

4)	If AF for PIN is used, the AF can request the 5GC to exposes capabilities in order for the AF to provision parameters for resources configuration/deconfiguration for a PIN, QoS authorization for a PIN, QoS control for the PIN traffic, and routing control for the PIN traffic. The mechanism and criteria used by the AF to determine the need for a QoS modification for the PIN traffic are out of 3GPP scope.

5)	PDU session management functionality can be used by the PEGC.

a)	When the PEGC detects new packets (PIN signalling or PIN traffic or creation of PIN) from a device in the PIN, it may map the PIN or PIN packets to an existing PDU session or establish a new PDU session. The criteria for taking the decision can be based on existing mechanism or implementation.

NOTE 1:	The procedure is the same used when application generating the traffic resides directly on the UE.

b)	The PEGC initiates PDU Session Establishment/Modification Request with necessary information:

i)	To enable 5GC to manage system resources related to a PIN, which includes one or more PEGCs.

ii)	To differentiate QoS control on PIN traffic.

NOTE 2:	The AF relies on PIN signalling between the PINE/PEGC/PEMC and the PIN AF, which is transferred via UP transparently to the 5G system, to determine the need for a QoS modification.

6)	The procedure for supporting one PINE connected to multiple PEGCs in the same PIN and PINE to move between PEGCs is outside the 3GPP scope.

7)	PIN direct communication is not specified since it is implementation specific.

8)	PIN indirect communication via PEGC is managed within the PIN, which may be supported by 5GS.

9)	A PEGC may establish a single or multiple PDU Sessions used for PIN communication. One PEGC may serve more than one PINs but one PIN will be served by single PDU session as default. (See PIN Session models as described in Annex A). Single PDU session may serve multiple PINs if there is no differentiation handling needs among different PIN traffics.

10)	IPv6 Prefix Delegation as described in clause 4.6.2.3 of TS 23.316 [5], or DHCP proxy by PEGC, or Framed Routing as described in clause 5.6.14 of TS 23.501 [2] are applied for IP address allocation of PINEs connected to PEGC.

NOTE 3:	Framed Route support will be further considered during normative work.

11)	If AF for PIN is used, the AF may provide necessary parameters to 5GC which may be considered by PCF to generate the PIN Route Selection Policy for PDU Session selection by PEGC(s) and to generate the URSP accordingly for PEMC(s).

12)	Routing of traffic from/to PDU session and the PIN elements is left to implementation

13)	PIN is a service that needs user subscribing from operator, the user's PIN service subscription is used by operator for policy configuration to PEGCs/PEMCs.

14)	UDR is enhanced to support the storage and retrieval of PIN related policy and QoS parameters.

15)	The N3GPP network delay between PINE and PEGC may be signalled from PEGC to PCF, and be taken into account when PCF derives the PDB value of QoS flow for PEGC.

16)	The 5G system support for anchoring PDU Sessions of PEGCs and PEMCs at same SMF based on a combination of DNN, S-NSSAI.

NOTE 4:	Other possibility without anchoring at same SMF may be determined in normative phase.

17)	If AF for PIN is used, the 5GC authorizes the number of PIN that the AF requests to create, which results in the number of PDU Sessions per PEGC/PEMC for PIN, according to user's PIN service subscription, which reflect the agreement between user and operator for using PIN service.

* End of changes *