SA WG2 Meeting #S2-154-AH-e(e-meeting)	S2-2300229

16 - 20 January 2023, Electronic Meeting	(revision of S2-22xxxxx)

Source:	Ericsson

Title:	KI#5, Conclusion updates

Document for:	Approval

Agenda Item:	9.3.1

Work Item / Release:	FS_PIN/Rel-18

Abstract of the contribution: update conclusion for KI#5.

# 1. Introduction

This paper introduces some updates to the conclusions for KI#5

# 2. Discussion

There is one EN under the conclusions for KI#5

Editor's note:	It is FFS whether PEMC UE needs a specific 5G subscription for providing PIN service.

For this EN, it seems clear based on the text below in clause 4.2 of the TR 23.700-68, i.e. PEMC is a UE with 5G subscription.

- Only a 3GPP UE can act as PEGC and/or PEMC.

In addition, as conclusions for KI#4, the subscription data for PEMC may be enhanced with PIN related information from AF for PIN.

Based on above, it’s proposed to support the conclusion updates below for KI#3.

# 3. Proposal

This paper proposes the following conclusion points to TR 23.700-88 clause 8.1.

* Start of change *

## 8.5	Conclusion on Key Issue #5

The following conclusions are agreed for Authorization for PIN:

1)	PIN application-level Authentication and Authorization of PIN and PIN Elements are not specified by SA WG2.

2)	PEMC and PEGC are authorized by 5GC to provides PIN service via subscription in the UDM. The subscription data for PEMC and PEGC can be enhanced by authorization information for PIN from AF for PIN service.

3)	PEGC is subject to operator policies for any PIN services that operate over 5GC.

4)	A PINE is authorized by PEMC to join a PIN.

5)	A PINE is allowed or disallowed to connect to a PEGC by the PEGC based on the provisioned information.

* End of changes *