SA WG2 Meeting #S2-154-AH-e(e-meeting)	S2-2300230

16 - 20 January 2023, Electronic Meeting	(revision of S2-22xxxxx)

Source:	Ericsson

Title:	KI#7, Conclusion

Document for:	Approval

Agenda Item:	9.3.1

Work Item / Release:	FS_PIN/Rel-18

Abstract of the contribution: Introduction interim conclusion for KI#7.

# 1. Introduction

This paper introduces interim conclusion for KI#7

# 2. Discussion

There is one EN under the conclusions for KI#7

Editor's note:	It is FFS whether 5GC knows PINE ID.

Based on the conclusions for KI#4, the AF for PIN can, based on exposure of the capabilities from 5GC, the steer QoS and routing of the PIN traffic. It’s also agreed that the AF for PIN may provide necessary parameters to 5GC to generate URSP for PEGC or PEMC.

The conclusion for KI#4 further indicates that one PDU session is used for one PIN while one PEGC may supports multiple PINs. Based on this, we consider the granularity to control/steer PIN traffic is sufficient at the PIN level and no need to reach each individual PIN element.

Based on above, it’s proposed to support the conclusions below for KI#7.

# 3. Proposal

This paper proposes the following conclusion points to TR 23.700-88 clause 8.7.

* Start of change *

## 8.7	Conclusion on Key Issue #7

The following principles are concluded for Key Issue #7 "Identification of PIN and PIN Elements":

1)	NEF/UDM supports storing PIN ID in UDR, which may be received from AF.

NOTE 1:	Whether external and internal PIN ID are needed, and who allocates the PIN ID, are determined in normative phase.

2)	PIN ID is uniquely identifiable within the 5G network,

NOTE 2:	It will be determined during the normative phase whether the PIN ID is defined as an external PIN identifier in similar way of the External Group Identifier and GPSI, and an 5G internal PIN identifier similar as Internal Group Identifier.

3)	PINE ID awareness in 5GC is not needed.

* End of changes *