3GPP TSG-SA2 Meeting #154	S2-2200239

Online, January 16th – 20th February 2023

Source:	Rakuten Mobile

Title:	Discussion on PSAP resolution with UE mobility analytics

Document for:	Discussion

Agenda Item: 	9.23

Work Item / Release:	FS_eNA Ph3 /Rel-18

1	Background

For the conclusion of Key Issue #9: Enhancement of NWDAF with finer granularity of location information that the following use cases was addressed in normative work:

The following use cases will be addressed in normative work:

-	Existing analytics IDs that request UE location (i.e. Observed Service Experience, UE mobility, and Dispersion Analytics) are enhanced:

-	to support the PSAP resolution use case

2	Discussion

Figure 2-1

The above figure shows the case of PSAP resolution where a UE moved from a normal cell#1 which belonging to PSAP Area#1 to the large cell#0 which spreads across the multiple PSAP areas(#1-3). Depending on the UE mobility within and/or around the large cell#0, estimated PSAP area can be changed. When the UE originates the emergency call from the large cell#0 and the NW cannot specify which PSAP areas(#1-3) the UE belongs to, then NW ask the NWDAF.

---------------------

The consumer of these analytics may indicate in the request:

-	Analytics ID = "UE Mobility".

-	Target of Analytics Reporting: a single UE (SUPI) or a group of UEs (an Internal Group ID);

-	Analytics Filter Information optionally containing:

-	Area of Interest (AOI): restricts the scope of the UE mobility analytics to the provided area;

-	Visited Area(s) of Interest (visited AOI(s)): additional filter to only consider UEs that are currently (i.e. now) in the "AOI" and had previously (i.e. in the "Analytics target period") been in at least one of the Visited AOI(s). If this parameter is provided, the Analytics target period shall be in the past (i.e. supported for statistics only);

NOTE 1:	For LADN service, the consumer (e.g. SMF) provides the LADN DNN to refer the LADN service area as the AOI.

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

NOTE 2:	For regular analytics scenarios, the Analytics target period is associated with the Analytics Filter Information = AOI, while for the scenario that Analytics ID=UE Mobility and Analytics Filter Information = (AOI and visited AOI(s)), as described in this clause, the Analytics target period is associated with the visited AOI(s) and to obtain the statistics for those UEs that currently reside in the AOI and had previously (i.e. in the "Analytics target period") been in at least one of the Visited AOI(s).

-------------

In case of PSAP resolution use case the AOI and the Analytic target period can be determined by the large cell#0 and the nomal cells belonging to the multiple PSAP area(#1-3). Current UE mobility analytics specified in 6.7.2 says that the consumer need to indicate the appropriate Area of Interest and the Analytics target period, which can be changed depending on the location and the actual deployment of the large cell(s) and the neighbor cell(s). The consumer need to manage such information.

Observation1: For the PSAP resolution case, it is not feasible for the consumer NF to specify the appropriate Area of Interest and the Analytics target period.

2	Proposal

As the observation1 in the above, for the PSAP scenario it is not feasible for the consumer to set the appropriate Area of interest and Analytic target period.

It is proposed:

Proposal 1 : For the PSAP resolution use case, NWDAF may provide the “minimal” output without the Area of interest and Analytic target period indicate by the consumer

Proposal 2 : For the PSAP resolution case, the consumer may require such “minimal” output by indicating the new IE