3GPP TSG-SA2 Meeting #154-AH-e	S2-2300268

# Online, , 16th Jan 2023 - 20th Jan 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0616 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#7 - 23.288 CR for adding UE Transmission Latency Performance Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, OPPO, SK Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Proposal to update TS 23.288 to add a new analytic as concluded in TR 23.700-80 KI#7 5GS Assistance to Federated Learning Operation, where the conclusions state the following: AF can request NWDAF (via NEF if needed) to provide existing, enhanced or new analytics to assist with federated learning operation. NWDAF Analytics services will be enhanced as follows: - New latency performance analytics are introduced for NWDAF. NOTE 1: The specific input and output parameters for the latency performance analytics among the group of UEs to assist the Application AI/ML FL operation will be determined during the normative phase. It is concluded that the AF as a consumer can request NWDAF to get a new Analytics ID on UE latency performance analytics. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a new clause under 6.7 to define a new analytic on UE latency performance analytics |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusions agreed in TR 23.700-88 (AIMLsys) KI#7 cannot be fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is part of outcome of SI FS_AIMLsys |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** Start of Changes (all new) ***************

### 6.7.x UE transmission latency performance analytics

#### 6.7.x.1 General

The clause 6.7.x describes how NWDAF can provide UE transmission latency performance analytics, in the form of statistics or predictions or both, to a service consumer. The UE transmission latency performance refers to a time delay for completing the transmission of a specific data volume from UE to AF, or from AF to UE.  If an expected number of repeating data transmissions or an expected time interval between data transmissions is given, the UE transmission latency performance can be provided as an average value of every data transmission latency performance within the Analytics target period.

The service consumer may be an AF, NEF or PCF.

The consumer of these analytics may indicate in the request:

-	Analytics ID = "UE Transmission Latency Performance".

-	Target of Analytics Reporting: a single UE (SUPI/GPSI) or a list of UEs (a list of SUPIs/GPSIs).

-	Analytics Filter Information optionally including:

-	DNN;

-	S-NSSAI;

-	Application ID;

-	Area of Interest (AOI(s)): restrics the scope of the UE latency performance analytics to the provided area;

-	Data Volume UL/DL: indicates a specific data volume trasmitted once from UE to AF, or from AF to UE;

-	QoS requirements (e.g., 5QI, QoS Characteristics);

-	Optionally, an expected number of repeating data transmissions within the Analytics target period;

-	Optionally, an expected time interval between data transmissions

-	Optionally, a request for geographical distribution of the UEs.

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

#### 6.7.x.2 Input Data

The NWDAF supporting analytics on UE transmission latency performance shall be able to collect UE performance information from OAM and 5GC. The detailed information collected by the NWDAF is defined in the Table 6.7.x.2-1.

Table 6.7.x.2-1: Input data related to UE latency performance

| Information | Source | Description |
| --- | --- | --- |
| Radio link quality (RSRP) | OAM | Radio quality in the specified area where UE(s) will be located |
| RAN UE throughput | OAM TS 28.554 [10] | Average UE bitrate in the cell (Payload data volume on RLC level per elapsed time unit on the air interface, for transfers restricted by the air interface), per timeslot, per cell, per 5QI and per S-NSSAI. |
| Latency and delay of 5G networks | OAM TS 28.554 [10] | Average packet transmission delay through the RAN part to the UE, per timeslot, per 5QI and per S-NSSAI. |
| Current location or predicted location at the time window of UE | GMLC/ NWDAF/OAM/AMF | Current location of the UE(s) needs to be selected via OAM/AMF if the application needs to be started at the same time. If the AoI indicated by the AF is a finer granularity area than the Cell level, the current location of the UE(s) needs to be selected via GMLC instead. The UE predicted laction needs to be selected via NWDAF if the application operation time window is different |

NOTE 1:	Whether the UE(s) is supporting a Slice or not can be checked by retrieving the registered AMF details from UDM or by asking AMF about what Slice is used by the UE(s) at the current registration (Alternatively, if NSACF is deployed, NSACF can provide a report on what slices are used by the UE(s))

NOTE 2:	User consent checking from UDM can apply for these analytics.

#### 6.7.x.3 Output Analytics

The NWDAF supporting UE transmission latency performance analytics provides the analytics results to consumer NFs, e.g., AF, NEF or PCF. The analytics results provided by the NWDAF could be 1UE transmission latency performance predictions as defined in table 6.7.x.3-1.

Table 6.7.x.3-1: UE transmission latency performance predictions

| Information | Description |
| --- | --- |
| UE transmission latency performance | This information refers to a time delay for completing the transmission of a specific data volume from UE to AF, or from AF to UE. If an expected number of repeating data transmissions or an expected time interval between data transmissions is given in the request, the UE transmission latency performance can be provided as an average value of every data transmission latency performance within the Analytics target period, which is also provided in the request. |
| Time window | If network is congested, suggested time window can be provided for better result of the application. |
| Geographical distribution of the UE(s) | If requeted, a list of UEs per location information |
| Confidence | Confidence of this prediction. |

#### 6.7.x.4 Procedures

The NWDAF can provide UE transmission latency performance analytics to a 5GC NF (e.g., AF, NEF or PCF).

Figure 6.7.x.4-1: Procedure for UE latency performance analytics

1.	The Consumer NF, e.g., AF, NEF or PCF, requests analytics for UE transmission latency performance analytics to NWDAF (possibly via NEF in case the consumer NF is AF) and provides the input information as specified in 6.7.x.1 to 5GC.

2.	An NWDAF supporting UE transmission latency performance analytics collects data about the requested UE(s) as specified in 6.7.x.2 including Radio link quality, RAN UE throughput and Latency and delay of 5G networks from OAM, and UE’s current or predicted location information from AMF, OAM, GMLC or NWDAF.

3.	The NWDAF estimates the UE transmission latency performance analytics using the collected data in step 2.

4.	The NWDAF notifies the analytics to the consumer NF, e.g., AF, NEF or PCF (possibly via NEF in case the consumer NF is AF), with the analytic result, which includes the output as specified in 6.7.x.3.

********** End of Changes ***************