3GPP TSG-WG SA2 Meeting #154-AH-E	S2-2300272

# Electronic Meeting, Jan 16th – 20th

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3862 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New Annex on PIN Session Model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-1-9 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | PIN Session Models (i.e. Shared PIN Session Model and Dedicated PIN Session Model) are already agreed during the study phase (see TR 23.700-88 Annex A). In order to specify the Session Management procedure and PEGC behavior for supporting PIN, it is essential to describe the PIN Session Model. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add PIN Session Models (i.e. Shared PIN Session Model and Dedicated PIN Session Model) into the new Annex. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PIN related Session Management and PIN Routing Selection Policy (or extension of URSP for supporing PIN) can not be described. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New Annex X.Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ….CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st Changes (All new text) * * * *

Annex X (informative):
Personal IoT Network

X.Y	PIN Session Model

X.Y.1	General

In this Annex, we describe the PIN Session models with the assumption that one PDU Session serves one PIN.

5GC can enforce PIN level QoS by applying the QoS for the PDU Session since the PIN and the PDU Session has one-to-one relationship. For example, if the operator applies the PIN level AMBR, it can enforce PIN-AMBR by enforcing PDU Session AMBR through the existing policy framework.

X.Y.2	Shared PIN Session Model

With this model, the applications of the PEGC also can use the same PDU Session that is used for relaying the PIN traffic as depicted in the Figure X.Y.2-1.

Figure X.Y.2-1: Shared PIN Session Model

With this model, a PDU Session can serve one PIN of PEGC.

Activation or deactivation of the PIN of PEGC does not require the PDU Session Establishment procedures. The QoS control on PIN may be limited without the PEGC engagement.

X.Y.3	Dedicated PIN Session Model

With this model, the applications of the UE do not share the same PDU Session that is used for relaying the PIN traffic as depicted in the Figure X.Y.3-1.

Figure X.Y.3-1: Dedicated PIN Session Model

With this model, one PDU Session is mapped to one PIN network and other applications of the UE do not share the same PDU Session.

Activation or deactivation of the PIN of PEGC can involve the PDU Session Establishment/Modification/Release procedure.

# * * * * End Changes * * * *