3GPP TSG-SA2 Meeting #154-AH-e	S2-2300282

# Online,  16th Jan 2023 - 20th Jan 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0619 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ML Model storage and retrieval from ADRF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo Mobile Com. Technology |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CR introduces description text for ML Model storage and retrieval from ADRF based on the conclusions of KI#4 in the TR 23.700-81. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New text is added in the existing Analytics Data Repository Functional Description to introduce ML Model storage and retrieval from ADRF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5B.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************************ START OF CHANGE (all new text) *****************************

# 5B	Analytics Data Repository Functional Description

## 5B.1	General

The ADRF offers services that enable a consumer to store and retrieve data and analytics. The analytics are produced by the NWDAF as described in clause 6.1 and data are collected as described in clause 6.2.

Data may be stored in the ADRF by:

-	a consumer sending the ADRF an Nadrf_DataManagement_StorageRequest containing the data or analytics to be stored. The ADRF response provides a result indication.

-	a consumer sending the ADRF an Nadrf_DataManagement_StorageSubscriptionRequest requesting that the ADRF subscribes to receive data or analytics for storage. The ADRF then subscribes to the NWDAF or DCCF for data or analytics, providing ADRF Notification Address (+Notification Correlation ID). Analytics or Data are subsequently provided as notifications using DCCF, NWDAF or MFAF services (Ndccf_DataManagement Nnwdaf_DataManagement or Nmfaf_3caDataManagement service).

Data may be retrieved from the ADRF by:

-	a consumer sending an Nadrf_DataManagement_RetrievalRequest request to the ADRF to retrieve data or analytics for a Storage Transaction Identifier or a Fetch Instructions received from the ADRF in an Nadrf_DataManagement_RetrievalNotify. The ADRF determines the availability of the data or analytics in its repository and sends in the response to the consumer either the data or analytics.

-	a consumer sending an Nadrf_DataManagement_RetrievalSubscribe request to the ADRF to retrieve data or analytics for a specified data or analytics collection time window. If the time window includes the future and the ADRF has subscribed to receive the data or analytics, subsequent notifications received by the ADRF are sent by the ADRF to the notification endpoint.

The ADRF determines the availability of the data or analytics and sends a success/failure indication in the response to the consumer. The ADRF then sends one or more notifications using an Nadrf_DataManagement_RetrievalNotify to the Notification Address (+Notification Correlation ID) specified by the consumer. The notification(s) either provide the data or analytics or provide instructions to the endpoint to fetch the data or analytics using an Nadrf_DataManagement_RetrievalRequest.

Data may be deleted from the ADRF by:

-	a consumer sending an Nadrf_DataManagement_Delete request.

An ADRF may be configured to register the data it is collecting with a DCCF. The registration uses the Ndccf_ContextManagement service specified in clause 8.3.2. The registration may subsequently be used by the DCCF to obtain data from the ADRF as described in clause 6.2.6.3.6.

ML Model may be stored in the ADRF by:

-	a consumer sending the ADRF an Nadrf_MLModelManagement_StorageRequest containing the ML Model and ML Model profile or ML Model address to be stored. The ADRF response provides a result indication.

-	a consumer sending the ADRF an Nadrf_MLModelManagement_StorageSubscriptionRequest requesting that the ADRF subscribes to receive the ML Model and ML Model profile or ML Model address for storage. The ADRF then subscribes to the NWDAF MTLF or DCCF for the ML Model and ML Model profile or ML Model address, providing ADRF Notification Address (+Notification Correlation ID). ML Model and ML Model profile or ML Model address are subsequently provided as notifications using DCCF, NWDAF or MFAF services (Ndccf_MLModelManagement Nnwdaf_MLModelManagement or Nmfaf_3caMLModelManagement service).

ML Model may be retrieved from the ADRF by:

-	a consumer sending an Nadrf_DataManagement_RetrievalRequest request to the ADRF to retrieve ML Model and ML Model profile or ML Model address for a Storage Transaction ML Model Identifier or a Fetch Instructions received from the ADRF in an Nadrf_ML ModelManagement_RetrievalNotify. The ADRF determines the availability of the ML Model and ML Model profile or ML Model address in its repository and sends in the response to the consumer either the data or analytics.

-	a consumer sending an Nadrf_ML ModelManagement_RetrievalSubscribe request to the ADRF to retrieve ML Model and ML Model profile or ML Model address for a specified time window. If the time window includes the future and the ADRF has subscribed to receive the ML Model and ML Model profile or ML Model address, subsequent notifications received by the ADRF are sent by the ADRF to the notification endpoint.

The ADRF determines the availability of the ML Model and ML Model profile or ML Model address and sends a success/failure indication in the response to the consumer. The ADRF then sends one or more notifications using an Nadrf_ML ModelManagement_RetrievalNotify to the Notification Address (+Notification Correlation ID) specified by the consumer. The notification(s) either provide the ML Model and ML Model profile or ML Model address or provide instructions to the endpoint to fetch the ML Model and ML Model profile or ML Model address using an Nadrf_ML ModelManagement_RetrievalRequest.

ML Model may be deleted from the ADRF by:

-	a consumer sending an Nadrf_MLModelManagement_Delete request.

An ADRF may be configured to register the ML Model and ML Model profile, or ML Model address it is collecting with a DCCF.

************************************ END OF CHANGE ****************************************