3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2300287

January 16th – 20th, 2023; Elbonia

Source:	Huawei, HiSilicon

Title:	Discussion on NSSRG restriction across different access types over different PLMNs

Document for:	Discussion

Agenda Item:	4.1

Work Item / Release:	eNS_Ph2/ Rel-17

Abstract: Discussion on NSSRG restriction across different access types over different PLMNs.

1. Introduction

This discussion paper is about how to reply to the LS on NSSRG restriction across different access types over different PLMNs from CT1 (S2-2300023/ C1-227101).

Question #1: Shall the Requested NSSAI across different access types over different PLMNs share common NSSRG value(s)?

Question #2: If the answer of Question #1 is yes, how does the network (i.e. different AMFs in different PLMNs) enforce the common NSSRG value(s) restriction on the Requested NSSAI?

1.1  Discussion on Question#1

The incoming LS relates to 3GPP TS 23.501 clause 5.15.12 which specifies subscription-based restrictions to simultaneous registration of network slices:

“The subscription information for a UE may include for each S-NSSAI Network Slice Simultaneous Registration Group (NSSRG) information constraining which S-NSSAIs can be simultaneously provided to the UE in the Allowed NSSAI.

When S-NSSAIs have associated NSSRG information, then the S-NSSAIs in the Allowed NSSAI shall share at least one NSSRG.”

“When the serving AMF provides the Configured NSSAI to the UE, and the UE has indicated it supports the subscription-based restrictions to simultaneous registration of network slices feature, the AMF also provides the UE with the NSSRG information related to the S-NSSAIs of the HPLMN which are in the mapping information of the Configured NSSAI. A UE which receives the NSSRG values in the network slicing configuration information shall only include in the Requested NSSAI S-NSSAIs that share a common NSSRG as per the received information.”

“At any time, a UE supporting subscription-based restrictions to simultaneous registration of network slices feature and that has received NSSRG information together with the Configured NSSAI shall only request S-NSSAIs, across all Access Type(s), that share one or more common NSSRG.”

Observation 1: The NSSRG information is subscription-based restrictions per UE, regardless of UE’s registration with single PLMN or different PLMNs. The NSSRG information configured in the UE is related to the S-NSSAIs of the HPLMN which are in the mapping information of the Configured NSSAI. A UE which receives the NSSRG values shall only include in the Requested NSSAI S-NSSAIs that share a common NSSRG.

Observation 2: The answer to Question#1 is YES.

1.2  Discussion on Question#2

Regarding Question#2, network enforcement is only needed when the UE registers with multiple PLMNs but doesn’t check the NSSRG information of the Requested NSSAI over different PLMNs. There are two alternatives of network enforcement as listed in the following:

- Option A: Allowed NSSAI restriction based solution.

The main principle is that the Serving AMF restricts the Allowed NSSAI for the current PLMN based on the existing Allowed NSSAI for a different PLMN stored in the UDM, if any, so that UE’s Allowed NSSAI over different PLMNs shares the common NSSRG.

When the serving AMF receives a Requested NSSAI from UE and there is no NSSRG information related to the Allowed NSSAI for a different PLMN stored in the UDM, the serving AMF determines Allowed NSSAI based on the received NSSRG information. After that, the serving AMF stores the NSSRG information related to the current Allowed NSSAI per PLMN in the UDM. When the serving AMF receives a Requested NSSAI from UE and there is NSSRG information related to the Allowed NSSAI for a different PLMN stored in the UDM, the serving AMF checks whether the S-NSSAIs in the Requested NSSAI share a common NSSRG with the Allowed NSSAI for different PLMN.

Figure 1

For example as depicted in Figure 1, it is assumed that UE’s subscription data includes NSSRG#A {HPLMN S-NSSAI-1, HPLMN S-NSSAI-2} and NSSRG#B { HPLMN S-NSSAI-1, HPLMN S-NSSAI-3}.

At T1, UE registers to PLMN#1; At T2, UE registers to PLMN#2. See Table 1.

Table 1

|  | Requested NSSAI | Allowed NSSAI/Rejected NSSAI |
| --- | --- | --- |
| PLMN#1 | Requested NSSAI = S-NSSAI-A + S-NSSAI-B; Mapping of Requested NSSAI = S-NSSAI-A < ─ > HPLMN S-NSSAI-1, S-NSSAI-B < ─ > HPLMN S-NSSAI-2; | Allowed NSSAI = S-NSSAI-A + S-NSSAI-B; Mapping of Allowed NSSAI = S-NSSAI-A < ─ > HPLMN S-NSSAI-1, S-NSSAI-B < ─ > HPLMN S-NSSAI-2; |
| PLMN#2 | Requested NSSAI = S-NSSAI-C + S-NSSAI-D; Mapping of Requested NSSAI = S-NSSAI-C < ─ > HPLMN S-NSSAI-1, S-NSSAI-D < ─ > HPLMN S-NSSAI-3; | Allowed NSSAI = S-NSSAI-C; Mapping of Allowed NSSAI = S-NSSAI-C < ─ > HPLMN S-NSSAI-1; Rejected NSSAI = S-NSSAI-D; |

Observation 3: Option A aligns with the Rel-17 principles that S-NSSAIs in the Allowed NSSAI shall share at least one NSSRG. However, the frequent signalling to store and delete the Allowed NSSAI per Serving PLMN in the UDM will bring some impacts on the AMF and UDM.

- Option B: PDU session restriction based solution

The main principle is that if the Requested NSSAI over different PLMNs doesn’t share a common NSSRG, the AMF determines Allowed NSSAI for serving PLMN based on the received NSSRG information but it cannot enforce the restriction on the Allowed NSSAI over different PLMNs.The UDM restricts the PDU session over different PLMNs based on the NSSRG value related to the HPLMN S-NSSAI which is used by the SMF to register for the PDU session in the UDM.

In this option, when UE requests a new PDU session establishment and the SMF registers for the PDU session with the UDM using the HPLMN S-NSSAI, the UDM when the SMF registers for the PDU session with the UDM using the HPLMN S-NSSAI, the UDM checks whether the HPLMN S-NSSAI for the existing PDU Session stored in the UDM shares a common NSSRG with the HPLMN S-NSSAI for new PDU Session. If no, the UDM de-registers the SMF which has previously registered. Then this SMF initiates the PDU Session Release procedure. Otherwise, the UDM accepts the SMF’s registration for new PDU session.

Figure 2

For example as depicted in Figure 2, it is assumed that UE’s subscription data includes NSSRG#A {HPLMN S-NSSAI-1, HPLMN S-NSSAI-2} and NSSRG#B {HPLMN S-NSSAI-1, HPLMN S-NSSAI-3}.

At T1, UE registers to PLMN#1; At T2, UE registers to PLMN#2. See Table 2.

Table 2

|  | Requested NSSAI | Allowed NSSAI/Rejected NSSAI |
| --- | --- | --- |
| PLMN#1 | Requested NSSAI = S-NSSAI-A + S-NSSAI-B; Mapping of Requested NSSAI = S-NSSAI-A < ─ > HPLMN S-NSSAI-1, S-NSSAI-B < ─ > HPLMN S-NSSAI-2; | Allowed NSSAI = S-NSSAI-A+ S-NSSAI-B; Mapping of Allowed NSSAI = S-NSSAI-A < ─ > HPLMN S-NSSAI-1, S-NSSAI-B < ─ > HPLMN S-NSSAI-2; |
| PLMN#2 | Requested NSSAI = S-NSSAI-C+ S-NSSAI-D; Mapping of Requested NSSAI = S-NSSAI-C < ─ > HPLMN S-NSSAI-1, S-NSSAI-D < ─ > HPLMN S-NSSAI-3; | Allowed NSSAI = S-NSSAI-C+ S-NSSAI-D; Mapping of Allowed NSSAI = S-NSSAI-C < ─ > HPLMN S-NSSAI-1, S-NSSAI-D < ─ > HPLMN S-NSSAI-3; |

At T3, UE initiates a new PDU Session for S-NSSAI-B in the PLMN#1. At T4, UE initiates a new PDU Session for S-NSSAI-D in the PLMN#2. The UDM determines the S-NSSAI-B and S-NSSAI-D cannot share a common NSSRG value and decides to de-register the SMF serving the PDU Session in PLMN#1.

Observation 4: Option B cannot make sure S-NSSAIs in the Allowed NSSAI over different PLMNs shall share at least one NSSRG. The PDU sessions over different PLMNs can be restricted based on the NSSRG value associated with S-NSSAIs for PDU sessions.

2. Conclusion and proposal(s)

Conclusion 1: The answer to Question#1 is YES. Because the NSSRG information is subscription-based restrictions per UE, regardless of UE’s registration with single PLMN or different PLMNs.

Conclusion 2: The NSSRG information configured in the UE is related to the S-NSSAIs of the HPLMN which are in the mapping information of the Configured NSSAI. A UE which receives the NSSRG values shall only include in the Requested NSSAI S-NSSAIs that share a common NSSRG. Regarding Question#2, network enforcement is only needed when the UE registers with multiple PLMNs but doesn’t check the NSSRG information of the Requested NSSAI over different PLMNs.

Conclusion 3: Compared to Option A, Option B has minimized impacts (e.g., the enhancement is to check the NSSRG value associated with HPLMN S-NSSAI for PDU sessions over different PLMNs).

Proposal: It is proposed that SA2 agrees to reply the LS on NSSRG restriction across different access types over different PLMNs from CT1 (S2-2300023/ C1-227101) based on the above discussion.