3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2300309

# Elbonia, January 16 – 20, 2023		(revision of S2-2210348)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0564 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding Accuracy Checking Capability to NWDAF Architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The conclusions of KI#1 in TR 23.700-81 define that NWDAF can have the accuracy checking capability and consumers of NWDAF can request the accuracy information to such NWDAFs with such capability. Such definitions need to be included in NWDAF capabilities, procedures, and services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the description of NWDAF providing accuracy information about analytics IDs and/or ML models to consumers. It has been agreed that NWDAF has the accuracy checking capability. Therefore the descriptions of NWDAF capabilities as well as how NWDAFs are discovered are updated. Conclusions in KI#1 defines the following: “An analytics consumer may request or subscribe to accuracy information about Analytics ID(s) from the AnLF with the performance checking capabilities. Accuracy information can be included in an accuracy report, scoped in the same way as Analytics requests are scoped, i.e., per Analytics ID, for a specific area, slice, (group of) UEs, in a given time window, etc. Such request or subscription triggers the monitoring and check of Analytics ID(s) and generation of analytics accuracy information.” If the accuracy information is scoped in the same way that an analytics output request, it is more appropriated that at subscription to the analytics ID, the consumer also indicates to NWDAF that it requests also the accuracy information for the analytics ID. As per R17, there exist already the definition of “Preferred Level of Accuracy”, “Output strategy” and “Confidence” (as output parameter) that are all related to individual analytics outputs provided by NWDAF. Clause 6.1.3, defines the “output strategy” which regulates when NWDAF provides an analytics output for a subscription to an analytics ID, and it clarifies how the “preferred level of accuracy” influences the NWDAF providing the response/notification of an analytics output to the NF consumer. These parameters (“preferred level of accruacy” and “output strategy”) combined provide a clarification of the text described in Clause 6.2.1, defining when NWDAF provides an analytics output: “The value of the confidence depends on the level or urgency expressed by the parameter "preferred level of accuracy" as listed in clause 6.1.3, the parameter "time when analytics information is needed" as listed in clause 6.1.3 and the availability of data. If no sufficient data is collected to provide an estimation for the preferred level of accuracy before the time deadline, the service shall return a zero confidence. Otherwise, the NWDAF may wait until enough data is collected before providing a response or a first notification.” Therefore, the parameters defined in the “Analytics Reporting Information”, such as the “Preferred Level of Accuracy” regulate when an analytics output will be provided by NWDAF. The term “Analytics Accuracy Information” defined in R18 KI#1 conclusions refers to the actual value of an analytics accuracy. Furthermore, for the definition a value for the accuracy information, NWDAF might need to collect information such as ground truth, which not necessarily is the same data collected for the inference processes of the analytics output. Therefore, to avoid any ambiguity in R18 NWDAF x R17 NWDAF already defined parameters, we propose the following definitions related to accuracy information: - The Analytics Accuracy Information defines the value determined for the accuracy or correctness or efficiency of an analytics ID used in the network. Optionally it may include an indication that the determined accuracy value for the analytics ID does not meet the preferred level of accuracy requested for the analytics ID. - The new set of parameters below are included in the Nnwdaf_AnalyticsSubscriptio_Subscribe and Nnwdaf_AnalyticsInfoRequest: “Analytics Accuracy Reporting Information”, composed of the following parameters: - Analytics Accuracy Information request - Analytics Accuracy Information Periodicity - Accuracy Information Target Period - Minimal number of analytics output occurrences A new Clause in 6.1X is included in the specification to describe the definition related to Accuracy Information Checking and Provisioning In addition, it has been agreed that MTLF can requests/subscribes AnLF to provide accuracy of the analytics generated from the model that is provided by the MTLF. Conclusions in KI#1 defines the following: “When MTLF provides an ML model to an AnLF, the MTLF requests/subscribes AnLF to determine accuracy of the analytics generated from that model by comparing predictions and its corresponding ground truth data, if the AnLF indicates it can provide accuracy feedback.” Therefore, a new Clause in 6.2X is included in the specification to describe the procedure related to Accuracy Information Checking and Provisioning between MTLF and AnLF. Accordingly, new Clause in 7.X and 7.Y are added to describe the Accuracy related new services, i.e., Nnwdaf_AccuracyProvision service and Nnwdaf_Accuracy service. A description is also added in Clause 6.2A.0 to explain the relationship between accucary feedback and ML model provision. Besides, the new services are added to the table in Clause 7.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | R18 agreements on KI#1 will not be properly captured in R18 NWDAF description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 5.1, 5.2, 6.1.3, 6.1X (NEW), 6.2X (NEW), 6.2A.0, 7.1, 7.X (NEW), 7.Y (NEW), 7.2.2, 7.2.4, 7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.1	General

The NWDAF (Network Data Analytics Function) is part of the architecture specified in TS 23.501 [2] and uses the mechanisms and interfaces specified for 5GC in TS 23.501 [2] and OAM services (see clause 6.2.3.1).

The NWDAF interacts with different entities for different purposes:

-	Data collection based on subscription to events provided by AMF, SMF, PCF, UDM, NSACF, AF (directly or via NEF) and OAM;

-	[Optionally] Analytics and Data collection using the DCCF (Data Collection Coordination Function);

-	Retrieval of information from data repositories (e.g. UDR via UDM for subscriber-related information);

-	[Optionally] Storage and retrieval of information from ADRF (Analytics Data Repository Function);

-	[Optionally] Analytics and Data collection from MFAF (Messaging Framework Adaptor Function);

-	Retrieval of information about NFs (e.g. from NRF for NF-related information);

-	On demand provision of analytics to consumers, as specified in clause 6.

-	Provision of bulked data related to Analytics ID(s).

-	Provision of Accuracy information about Analytics ID(s).

-	Provision of ML model accuracy degradation about a ML Model.

A single instance or multiple instances of NWDAF may be deployed in a PLMN. If multiple NWDAF instances are deployed, the architecture supports deploying the NWDAF as a central NF, as a collection of distributed NFs, or as a combination of both. If multiple NWDAF instances are deployed, an NWDAF can act as an aggregate point (i.e. Aggregator NWDAF) and collect analytics information from other NWDAFs, which may have different Serving Areas, to produce the aggregated analytics (per Analytics ID), possibly with Analytics generated by itself.

NOTE 1:	When multiple NWDAFs exist, not all of them need to be able to provide the same type of analytics results, i.e. some of them can be specialized in providing certain types of analytics. An Analytics ID information element is used to identify the type of supported analytics that NWDAF can generate.

NOTE 2:	NWDAF instance(s) can be collocated with a 5GS NF.

# * * * * Second change * * * *

5.1	General

The NWDAF provides analytics to 5GC NFs and OAM as defined in clause 7. An NWDAF may contain the following logical functions:

-	Analytics logical function (AnLF): A logical function in NWDAF, which performs inference, derives analytics information (i.e. derives statistics and/or predictions based on Analytics Consumer request) and exposes analytics service i.e. Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo. An NWDAF containing AnLF NWDAF with Accuracy checking capability is able to provide or notify the accuracy information of Analytics IDs to the consumers of such service.

-	Model Training logical function (MTLF): A logical function in NWDAF, which trains Machine Learning (ML) models and exposes new training services (e.g. providing trained ML model) as defined in clause 7.5 and clause 7.6. An NWDAF containing MTLF with Accuracy checking capability is able to provide or notify the ML model accuracy degradation to the consumers of such service.

NOTE 1:	NWDAF can contain an MTLF or an AnLF or both logical functions.

NOTE 2:	Pre-trained ML model storage and provisioning to NWDAF is out of the scope of 3GPP.

Analytics information are either statistical information of the past events, or predictive information.

Different NWDAF instances may be present in the 5GC, with possible specializations per type of analytics. The capabilities of a NWDAF instance are described in the NWDAF profile stored in the NRF.

To guarantee the accuracy of analytics output for an Analytics ID, based on the UE abnormal behaviour analytics from itself or other NWDAF including abnormal UE list and the observed time window, the NWDAF is to detect and may delete the input data from the abnormal UE(s) and then may generate a new ML model and/or analytics outputs for the Analytics ID without the input data related to abnormal UE list during the observed time window and then send/update the ML Model Information and/or analytics outputs to the subscribed NWDAF service consumer.

In order to support NFs to discover and select an NWDAF instance containing MTLF, AnLF, or both, that is able to provide the required service (e.g. analytics exposure or ML model provisioning) for the required type of analytics, each NWDAF instance should provide the list of supported Analytics ID(s) (possibly per supported service) when registering to the NRF, in addition to other NRF registration elements of the NF profile. NFs requiring the discovery of an NWDAF instance that provides support for some specific service(s) for a specific type of analytics may query the NRF for NWDAFs supporting the required service(s) and the required Analytics ID(s).

The consumers, i.e. 5GC NFs and OAM, decide how to use the data analytics provided by NWDAF.

The interactions between 5GC NF(s) and the NWDAF take place within a PLMN.

The NWDAF has no knowledge about NF application logic. The NWDAF may use subscription data but only for statistical purpose.

The NWDAF architecture allows for arranging multiple NWDAF instances in a hierarchy/tree with a flexible number of layers/branches. The number and organisation of the hierarchy layers, as well as the capabilities of each NWDAF instance remain deployment choices.

In a hierarchical deployment, NWDAFs may provide data collection exposure capability for generating analytics based on the data collected by other NWDAFs, when DCCF, MFAF are not present in the network.

In order to make NWDAF discoverable in some network deployments, NWDAF may be configured (e.g. for UE mobility analytics) to register in UDM (Nudm_UECM_Registration service operation) for the UE(s) it is serving and for the related Analytics ID(s). Registration in UDM should take place at the time the NWDAF starts serving the UE(s) or collecting data for the UE(s). Deregistration in UDM takes place when NWDAF deletes the analytics context for the UE(s) (see clause 6.1B.4) for a related Analytics ID.

NOTE 3:	The procedures for data collection for UE related analytics need to take user consent into account. The user consent for analytics is defined in clause 6.2.9.

# * * * * Third change * * * *

5.2	NWDAF Discovery and Selection

The NWDAF service consumer selects an NWDAF that supports requested analytics information and required analytics capabilities and/or requested ML Model Information by using the NWDAF discovery principles defined in clause 6.3.13 of TS 23.501 [2].

Different deployments may require different discovery and selection parameters. Different ways to perform discovery and selection mechanisms depend on different types of analytics/data (NF related analytics/data and UE related analytics/data). NF related refers to analytics/data that do not require a SUPI nor group of SUPIs (e.g. NF load analytics). UE related refers to analytics/data that requires SUPI or group of SUPIs (e.g. UE mobility analytics).

In order to discover an NWDAF containing AnLF using the NRF:

-	If the analytics is related to NF(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 1:	If the selected NWDAF cannot provide the requested data analytics, e.g. due to the NF(s) to be contacted being out of serving area of the NWDAF, the selected NWDAF might reject the analytics request/subscription or it might query the NRF with the service area of the NF to be contacted to determine another target NWDAF.

-	If the analytics is related to UE(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 2:	If a selected NWDAF cannot provide analytics for the requested UE(s) (e.g. the NWDAF serves a different serving area), the selected NWDAF might reject the analytics request/subscription or it might determine the AMF serving the UE as specified in clause 6.2.2.1, request UE location information from the AMF and query the NRF with the tracking area where the UE is located to discover another target NWDAF serving the area where the UE(s) is located.

If the NWDAF service consumer needs to discover an NWDAF that is able to collect data from particular data sources identified by their NF Set IDs or NF types, the consumer may query NRF providing the NF Set IDs or NF types in the discovery request.

NOTE 3:	The NF Set ID or NF Type of a data source serving a particular UE, can be determined as indicated in Table 5A.2-1.

In order to discover an NWDAF that has registered in UDM for a given UE:

-	NWDAF service consumers or other NWDAFs interested in UE related data or analytics, if supported, may make a query to UDM to discover an NWDAF instance that is already serving the given UE.

If an NWDAF service consumer needs to discover NWDAFs with data collection exposure capability, the NWDAF service consumer may discover via NRF the NWDAF(s) that provide the Nnwdaf_DataManagement service and their associated NF type of data sources or their associated NF Set ID of data sources as defined in clause 6.3.13 of TS 23.501 [2].

In order to discover an NWDAF containing MTLF via NRF:

-	an NWDAF containing MTLF shall include the ML model provisioning services (i.e. Nnwdaf_MLModelProvision, Nnwdaf_MLModelInfo) as one of the supported services during the registration in NRF when trained ML models are available for one or more Analytics ID(s). The NWDAF containing MTLF may provide to the NRF a (list of) Analytics ID(s) corresponding to the trained ML models and possibly the ML Model Filter Information for the trained ML model per Analytics ID(s), if available. In this Release of the specification, only the S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information for the trained ML model per Analytics ID(s) may be registered into the NRF during the NWDAF containing MTLF registration. For each Analytics ID, the NWDAF containing MTLF may also include, in the registration to the NRF, an ML Model Interoperability indicator.

-	The ML Model Interoperability indicator comprises a list of NWDAF providers (vendors) that are allowed to retrieve ML models from this NWDAF containing MTLF. It also indicates that the NWDAF containing MTLF supports the interoperable ML models requested by the NWDAFs from the vendors in the list.

Editor's note:	It is FFS if the ML Model Interoperability indicator is per Analytics ID.

NOTE 4:	The S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information are within the indicated S-NSSAI and NWDAF Serving Area information in the NF profile of the NWDAF containing MTLF, respectively.

-	During the discovery of NWDAF containing MTLF, a consumer (i.e. an NWDAF containing AnLF) may include in the request the target NF type (i.e. NWDAF), the Analytics ID(s), the S-NSSAI(s), Area(s) of Interest of the Trained ML Model required and NF consumer information. The NRF returns one or more candidate for instances of NWDAF containing MTLF to the NF consumer and each candidate for instance of NWDAF containing MTLF includes the Analytics ID(s) and possibly the ML Model Filter Information for the available trained ML models, if available.

NOTE 5:	NF consumer information such as Vendor ID is defined in stage 3.

A PCF may learn which NWDAFs being used by AMF, SMF and UPF for a specific UE, via signalling described in clause 4.16 of TS 23.502 [3]. This enables a PCF to select the same NWDAF instance that is already being used for a specific UE.

If the NWDAF service consumer needs to discover an NWDAF containing an AnLF with Accuracy checking capability for a certain area of interest and Analytics IDs, the consumer may query NRF providing the accuracy checking capability, the area of interest, and Analytics ID in the discovery request.

If the NWDAF service consumer needs to discover an NWDAF containing an MTLF with Accuracy checking capability for a certain area of interest and Analytics IDs, the consumer may query NRF providing the accuracy checking capability, the analytics ID, the ML model provisioning services in the discovery request.

# * * * * Fourth change * * * *

6.1.3	Contents of Analytics Exposure

The consumers of the Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operations described in clause 7 provide the following input parameters listed below.

-	A list of Analytics ID(s): identifies the requested analytics.

-	Analytics Filter Information: indicates the conditions to be fulfilled for reporting Analytics Information. This set of optional parameter types and values enables to select which type of analytics information is requested. Analytics Filter Information is defined in the analytics related clauses.

-	Target of Analytics Reporting: indicates the object(s) for which Analytics information is requested, entities such as specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) A Notification Target Address (+ Notification Correlation ID) as defined in clause 4.15.1 of TS 23.502 [3], allowing to correlate notifications received from NWDAF with this subscription.

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Subscription Correlation ID: identifies an existing analytics subscription that is to be modified.

-	Related to analytic consumers that aggregate analytics from multiple NWDAF subscriptions:

-	[OPTIONAL] (Set of) NWDAF identifiers of NWDAF instances used by the NWDAF service consumer when aggregating multiple analytics subscriptions. See clause 6.1A.

-	Analytics Reporting Information with the following parameters:

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Analytics Reporting Parameters as per Event Reporting parameters defined in Table 4.15.1-1 of TS 23.502 [3].

NOTE 1:	When the Analytics Reporting Parameters indicates a periodic reporting mode and the periodicity of the report is equal to or greater than the Supported Analytics Delay associated with the Analytics ID (if available) defined in clause 6.2.6.2 of TS 23.501 [2], it is expected that the periodic reporting can be provided by the NWDAF as requested.

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Reporting Thresholds, which indicate conditions on the level of each requested analytics that when reached shall be notified by the NWDAF.

-	[OPTIONAL] Matching direction: A matching direction may be provided such as below, above, or crossed. If no matching direction is provided, the default direction is crossed.

-	[OPTIONAL] Acceptable deviation: An acceptable deviation from the threshold level in the non-critical direction (i.e. in which the QoS is improving) may be set to limit the amount of signalling.

-	Analytics target period: time interval [start..end], either in the past (both start time and end time in the past) or in the future (both start time and end time in the future). An Analytics target period in the past is a request or subscription for statistics. An Analytics target period in the future is a request or subscription for predictions. The time interval is expressed with actual start time and actual end time (e.g. via UTC time). When the Analytics Reporting Parameters indicate a periodic reporting mode, the time interval can also be expressed as positive or negative offsets to the reporting time, which indicates a subscription for predictions or statistics respectively. By setting start time and end time to the same value, the consumer of the analytics can request analytics or subscribe to analytics for a specific time rather than for a time interval.

-	[OPTIONAL] Data time window: if specified, only events that have been created in the specified time interval are considered for the analytics generation.

-	[OPTIONAL] Preferred level of accuracy of the analytics ("Low", "Medium", "High" or "Highest").

-	[OPTIONAL] Preferred level of accuracy per analytics subset ("Low", "Medium", "High" or "Highest"). When a preferred level of accuracy is expressed for a given analytics subset, it takes precedence for this subset over the above preferred level of accuracy of the analytics. Analytics subsets are defined in the "Output Analytics" clause of applicable analytics.

-	[OPTIONAL] Dataset Statistical Properties: information in order to influence the data selection mechanisms to be used for the generation of an Analytics ID, assuring that the generated Analytics ID reflects the statistical characteristics of the data that are relevant for the NWDAF consumer. The following dataset statistical properties are allowed:

-	Uniformly distributed datasets, which indicates the use of data samples that are uniformly distributed according to the different aspects of the requested analytics (e.g. equivalent data samples for each UE listed as a Target of Analytics Reporting or for S-NSSAIs included in the Analytics Filter Information).

-	Datasets with or without outliers, which indicates that the data samples shall consider or disregard data samples that are at the extreme boundaries of the value range.

-	Time when analytics information is needed (if applicable): indicates to the NWDAF the latest time the analytics consumer expects to receive analytics data provided by the NWDAF. It should not be set to a value less than the Supported Analytics Delay of the selected NWDAF if applicable. If the time is reached the consumer does not need to wait for the analytics information any longer, yet the NWDAF may send an error response or error notification to the consumer. "Time when analytics information is needed" is a relative time interval as the gap with respect to analytics request /subscription (e.g. "in 10 minutes").

NOTE 2:	If the analytics request contains the parameter "Time when analytics information is needed" for Analytics ID(s), this parameter takes precedence over the requested periodicity, if a periodic reporting mode is requested.

NOTE 3:	If the Time when analytics information is needed is provided and it is less than the Supported Analytics Delay per Analytics ID (if available) defined in clause 6.2.6.2 of TS 23.501 [2], it is expected that the NWDAF might not be able to treat the Analytics ID on time.

-	[OPTIONAL] Maximum number of objects requested by the consumer (max) to limit the number of objects in a list of analytics per Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response.

-	[OPTIONAL] Preferred granularity of location information: TA level or cell level.

-	[OPTIONAL] Preferred order of results when a list of analytics is returned, possibly with a criterion for identifying the property of the results to which the preferred ordering is applied.

-	[OPTIONAL] Maximum number of SUPIs (SUPImax) requested by the consumer to limit the number of SUPIs in an object. When SUPImax is not provided, the NWDAF shall return all SUPIs concerned by the analytics object. When SUPImax is set to 0, the NWDAF shall not provide any SUPI.

-	[OPTIONAL] Output strategy: indicates the relevant factors for determining when the analytics reported. The following values are allowed:

-	Binary output strategy: indicates that the analytics shall only be reported when the preferred level of accuracy is reached within a cycle of periodic notification as defined in the Analytics Reporting Parameters.

NOTE 4:	If preferred level of accuracy is more important than providing an output, then the binary strategy is used so that all analytics outputs have equivalent confidence in the prediction.

-	Gradient output strategy: indicates that the analytics shall be reported according with the periodicity defined in the Analytics Reporting Parameters irrespective if the preferred level of accuracy has been reached.

NOTE 5:	If having an analytics output is more important than reaching the preferred level of accuracy, then the gradient output strategy is used so that each NWDAF will timely provide the output indicating the confidence of the prediction at the moment of the output generation.

NOTE 6:	When no output strategy is included in the subscription, the analytics output will be generated based on the gradient strategy and includes the confidence of the prediction for the reporting period.

-	[OPTIONAL] Analytics metadata request: indicates a request from one NWDAF to another NWDAF to provide the "analytics metadata information" related to the produced output analytics. This input parameter indicates which parameters in "analytics metadata information" are required to aggregate the output analytics for the requested Analytics ID(s).

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Consumer NF's serving area or NF ID. During a pending analytics subscription transfer, this information can be used by the NWDAF to find out if the analytics consumers may change as described in clause 6.1B.2.

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Information of previous analytics subscription. When setting up the analytics generation, this information may be used to retrieve analytics context from the previous NWDAF in order to build upon the context that is already related to this subscription as described in clause 6.1B.2.1.

-	[OPTIONAL] Use case context: indicates the context of use of the analytics to select the most relevant ML model.

NOTE 7:	The NWDAF can use the parameter "Use case context" to select the most relevant ML model, when several ML models are available for the requested Analytics ID(s). NWDAF containing AnLF can additionally provide the parameter "Use case context" when requesting an ML model from an NWDAF containing MTLF. The values of this parameter are not standardized. For example, the AMF can use a given value of "Use case context" when requesting UE Mobility analytics for optimizing the definition of a Registration Area, and a different value of "Use case context" when requesting UE Mobility analytics for determines a paging strategy.

-	Analytics Accuracy Reporting Information with the following parameters:

-	Analytics Accuracy Information request: is an indication to NWDAF to start the accuracy checking for the requested analytics ID(s) and to provide the determined accuracy information to the consumer.

-	[OPTIONAL] Analytics Accuracy Information Periodicity: determines the intervals of time when NWDAF provides the accuracy information to the consumer.

-	[OPTIONAL] Accuracy Information Target Period: determines the interval [start, end] of time to be considered in the generation of the Accuracy Information for the analytics ID.

-	[OPTIONAL] Minimal number of analytics output occurrences: determines the minimal number of analytics output provided by NWDAF that have to be considered in the determination of the accuracy information.

-	[OPTIONAL] Pause Analytics Subscription request: is a flag indicating to NWDAF to stop sending the notifications of analytics ouptuts for a subscribed analytics ID, without unsusbscribing to such analytics ID.

-	[OPTIONAL] Pause Analytics Subscription time window: indicates the period of time that NWDAF should stop sending the notifications of analytics outputs for a subscribed analytics ID.

-	[OPTIONAL] Resume Analytics Subscription request: is a flag indicating to NWDAF to resume the notification of analytics outputs for an existing analytics ID(s) subscription(s) that have been previously paused.

The NWDAF provides to the consumer of the Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request service operations described in clause 7, the output information listed below:

-	(Only for Nnwdaf_AnalyticsSubscription_Notify) The Notification Correlation Information.

-	For each Analytics ID the analytics information in the requested Analytics target period.

-	Timestamp of analytics generation: allows consumers to decide until when the received information shall be used. For instance, an NF can deem a received notification from NWDAF for a given feedback as invalid based on this timestamp;

-	Validity period: defines the time period for which the analytics information is valid.

-	Confidence: probability assertion:, i.e. confidence in the prediction.

-	[OPTIONAL] For each Analytics ID the Termination Request, which notifies the consumer that the subscription is requested to be cancelled as the NWDAF can no longer serve this subscription, e.g. due to user consent revoked, NWDAF overload, UE moved out of NWDAF serving area, etc.

-	[OPTIONAL] Analytics metadata information: additional information required to aggregate the output analytics for the requested Analytics ID(s). This parameter shall be provided if the "Analytics metadata request" parameter was provided in the corresponding Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operation.

-	Number of data samples used for the generation of the output analytics;

-	Data time window of the data samples;

-	Dataset Statistical Properties of the analytics output used for the generation of the analytics;

-	[OPTIONAL] Data source(s) of the data used for the generation of the output analytics;

-	[OPTIONAL] Data Formatting and Processing applied on the data;

-	Output strategy used for the reporting of the analytics.

-	(Only for error response or error notification) Revised waiting time: indicates to the consumer a revised waiting value for "Time when analytics information is needed". Each NWDAF may include this as part of error response or error notification to "Time when analytics information is needed" as described in clause 6.2.5. Revised waiting time is the minimum time interval recommended by NWDAF to use as "Time when analytics information is needed" for similar future analytics requests/subscriptions.

-	Analytics Accuracy Information generated for each analytics ID, including:

-	The determined accuracy value for an analytics ID,

-	(Optionally) An indication that the determined accuracy value for the analytics ID does not meet the preferred level of accuracy requested for the analytics ID.

-	Pause Analytics Output Consumption indication: NWDAF provides to consumer an indication to pause the consumption of the analytics outputs for the analytics ID(s) subscription due to the verification that such analytics ID(s) is not fulling the requested accuracy.

-	Resume Analytics Output Consumption indication: NWDAF provides to consumer an indication to resume the consumption of analytics output for existing subscription to the analytics ID(s) that was previously paused.

# * * * * Fifth change * * * *

6.1X	Analytics Accuracy Checking and Provisioning

6.1X.1	General

The Analytics Accuracy Information comprises the value determined for the accuracy of an analytics ID used in the network. Optionally it may include an indication that the determined accuracy value for the analytics ID does not meet the preferred level of accuracy requested for the analytics ID.

When multiple NWDAFs are deployed, some NWDAFs may be specialized with the analytics accuracy checking capability. When an NWDAF containing AnLF has the accuracy checking capability, such NWDAF is able to:

-	Receive a subscription or request for analytics IDs via Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operations with the indication for activating the mechanisms for checking the accuracy of such analytics ID.

-	Provide the accuracy information to the consumer via Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Response.

The request or subscription to analytics accuracy information triggers the monitoring and check of Analytics ID(s) and generation of analytics accuracy information. The Analytics accuracy information can be requested per Analytics ID and scoped using the same parameters defined in the Target of Analtyics Reporting (i.e., UEs, any UE, group of UEs) and Analytics Filter Information (e.g., for a specific area, specific slice) of the requested Analytics ID.

When the accuracy checking is activated in an NWDAF containing the ANLF, the NWDAF stores for a period of time the necessary information to determine the analytics IDs accuracy and provide the accuracy information to consumers when requested or use it for its internal processes.

NWDAF containing AnLF generates the accuracy information based on any of the following information:

- Comparing predictions and its corresponding ground truth data.

NOTE 3:	The ground truth data and the corresponding prediction is to be defined per Analytics ID.

- Comparing changes in internal configuration for the analytics ID generation (e.g., data collection parameters).

- Previous existent records of analytics accuracy information.

6.1X.2	Procedures for Analytics Accuracy Information Subscription

This procedure is used by NF consumers of analytics ID to subscribe simultaneously to receive analytics output and analytics accuracy information related to the requested analytics ID for NF consumer. Figure 6.1X.2-1 shows the procedure for accuracy information subscription and provisioning.

Figure 6.1X.2-1: Analytics Accuracy Information Subscribe

1.	The NWDAF with AnLF, register the NWDAF NF Profile in NRF including the accuracy checking capability.

2.	The NWDAF service consumer discovers at NRF the NWDAF with accuracy checking capability.

3.	The NWDAF service consumer subscribes to analytics accuracy information by invoking the Nnwdaf_AnalyticsSubscription_Subscribe service operation. The parameters that can be included in the subscription to trigger the accuracy information checking and provisioning are listed in clause 6.1.3.

4.	When a subscription request is received, the NWDAF determines whether the subscription is only for analytics output generation or if it includes the Analytics Accuracy Information request. In case Analytics Accuracy Information request is included, NWDAF starts the moniroting, checking, and generation of the accuracy information related to the analytics ID indicated in the subscription and according with the parameters defined in Analytics Accuracy Reporting Information in clause 6.1.3.

5.	NWDAF performs the data collection and analytics output generation.

6a.	NWDAF provides the analytics output according with the parameters defined in Analytics Reporting Information included in the subscription request.

6b.	NWDAF provides the analytics accuracy information for the analytics ID according with the parameters defined in the Analytics Accuracy Reporting Information included in the subscription request.

7.	NWDAF may determine the low or insufficient accuracy for an analytics ID and may notify the NWDAF Service consumer with the Pause Analytics Output Consumption indication.

8.	NWDAF Service Consumer may decide to pause the consumption of analytics output without unsubscribing to the analytics ID, based on its own logic or based on a received notification from NWDAF with the Pause Analytics Output Consumption indication. The NWDAF Service Consumer invokes the Nnwdaf_AnalyticsSubscription_Subscribe service operation including the Subscription Correlation ID to modify an existing subscription and provide the parameter Pause Analytics Subscription request in the Analytics Accuracy Reporting Information.

9.	When NWDAF determines an improvement in the accuracy of an analytics ID (at least higher than before the indication to pause the analytics output consumption) or when the Pause Analytics Subscription time window requested by an NWDAF Service Consumer is finished, NWDAF notifies the NWDAF Service Consumer of the accuracy information for the analytics ID to resume the consumption of the analytics output, therefore reactivating an existing analytics ID subscription that have been previously paused.

10.	NWDAF Service Consumer based on its own logic can notify the NWDAF Service Consumer to resume the provisioning of analytics output, therefore reactivating an existing subscription to analytics ID that have been paused either by NWDAF Service Consumer request (Step 8) or by NWDAF indication (Step 7). The NWDAF Service Consumer invokes the Nnwdaf_AnalyticsSubscription_Subscribe service operation including the Subscription Correlation ID to modify an existing subscription and provide the parameter Resume Analytics Subscription request in the Analytics Accuracy Reporting Information.

6.1X.3	Procedures for Analytics Accuracy Information Request

This procedure is used by NF consumers of analytics ID to request simultaneously to receive analytics output and analytics accuracy information related to the requested analytics ID for NF consumer. Figure 6.1X.3-1 shows the procedure for accuracy information request and provisioning.

Figure 6.1X.3-1: Analytics Accuracy Information Request

1.	The NWDAF with AnLF, register the NWDAF NF Profile in NRF including the accuracy checking capability.

2.	The NWDAF service consumer discovers at NRF the NWDAF with accuracy checking capability.

3.	The NWDAF service consumer request the analytics accuracy information by invoking the Nnwdaf_AnalyticsInfo_Request service operation. The parameters that can be included in the request to trigger the accuracy information checking and provisioning are listed in clause 6.1.3.

4.	When a request is received, the NWDAF determines whether the request is only for analytics output generation or if it includes the Analytics Accuracy Information request. In case Analytics Accuracy Information request is included, NWDAF starts the moniroting, checking, and generation of the accuracy information related to the analytics ID indicated in the request and according with the parameters defined in Analytics Accuracy Reporting Information in clause 6.1.3.

5.	NWDAF performs the data collection and analytics output generation.

6.	NWDAF provides the analytics output according with the parameters defined in Analytics Reporting Information included in the subscription request and the analytics accuracy information for the analytics ID according with the parameters defined in the Analytics Accuracy Reporting Information included in the request.

# * * * * Sixth change * * * *

7.2.2	Nnwdaf_AnalyticsSubscription_Subscribe service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Subscribe.

Description: Subscribes to NWDAF analytics with specific parameters.

Inputs, Required:

-	(Set of) Analytics ID(s) as defined in Table 7.1-2;

-	Target of Analytics Reporting;

-	Notification Target Address (+ Notification Correlation ID);

-	Analytics Reporting Parameters (including Analytics target period, etc.).

NOTE 1:	Target of Analytics Reporting can be provided per individual Analytics ID.

Inputs, Optional:

-	Analytics Filter Information;

-	Subscription Correlation ID (in the case of modification of the analytics subscription);

-	preferred level of accuracy of the analytics;

-	preferred level of accuracy per analytics subset;

-	Reporting Thresholds;

-	maximum number of objects requested (max);

-	preferred order of results, maximum number of SUPIs requested (SUPImax);

-	time when analytics information is needed;

-	Analytics Metadata Request;

-	(Set of) NWDAF identifiers used by the NWDAF service consumer when aggregating multiple analytic subscriptions;

-	Dataset Statistical Properties;

-	Output strategy;

-	Data time window;

-	consumer NF's serving area or NF ID;

-	information of previous analytics subscription, i.e. NWDAF identifier (i.e. Instance ID or Set ID), Analytics ID(s) (including SUPIs and Analytics Filter Information for UE-related Analytics) and Subscription Correlation ID;

-	Use case context.

-	Analytics Accuracy Reporting Information, including:

-	Analytics Accuracy Information request;

-	Analytics Accuracy Information Periodicity;

-	Accuracy Information Target Period;

-	Minimal number of analytics output occurrences;

-	Pause Analytics Subscription request;

-	Pause Analytics Subscription time window;

-	Resume Analytics Subscription request

NOTE 2:	Analytics Filter Information, Reporting Thresholds, maximum number of objects requested (max), maximum number of SUPIs requested (SUPImax), Analytics Accuracy Reporting Information, Analytics Metadata Request, Dataset Statistical Properties, Output strategy, Data time window and time when analytics information is needed can be provided per individual Analytics ID.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: None.

NOTE 3:	When the Target of Analytics Reporting is a SUPI or a GPSI then the subscription may not be accepted, e.g. user consent is not granted and an error is sent to the consumer. When the Target of Analytics Reporting is an Internal Group Id, or a list of SUPIs/GPSI(s) or any UE, no error is sent, but a SUPI or GPSI is skipped if user consent is not granted.

# * * * * Eighth change * * * *

7.2.4	Nnwdaf_AnalyticsSubscription_Notify service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Notify.

Description: NWDAF notifies the consumer instance of the analytics that has subscribed to the specific NWDAF service. See also clause 6.1.3 for contents of the Analytics Exposure.

Inputs, Required: Notification Correlation Information: this parameter indicates the Notification Correlation Id that has been assigned by the consumer during analytics subscription.

Inputs, Optional:

-	Set of the tuple (Analytics ID, Analytics specific parameters): this parameter shall be present if output analytics are reported.

-	Timestamp of analytics generation (required when ADRF is deployed).

-	Validity period.

-	Confidence

-	Revised waiting time.

-	Analytics Metadata Information.

-	Analytics Accuracy Information.

-	Pause Analytics Output Consumption indication.

-	Resume Analytics Output Consumption indication.

-	Termination Request: this parameter indicates that NWDAF requests to terminate the analytics subscription, i.e. NWDAF will not provide further notifications related to this subscription, with cause value (e.g. user consent revoked, NWDAF overload, UE moved out of NWDAF serving area, etc.).

NOTE:	Validity period can also be provided as part of Analytics specific parameters for some NWDAF output analytics.

-	Subscription Change Notification Correlation ID: this parameter shall be present if the notification is for informing the assignment of a new Subscription Correlation Id by the NWDAF. It is set to the old Subscription Correlation ID.

-	Subscription Correlation ID: this parameter shall be present if the notification is for informing the assignment of a new Subscription Correlation Id by the NWDAF. It is set to a new Subscription Correlation ID assigned by the NWDAF.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * Ninth change * * * *

7.3.2	Nnwdaf_AnalyticsInfo_Request service operation

Service operation name: Nnwdaf_AnalyticsInfo_Request.

Description: The consumer requests NWDAF operator specific analytics.

Inputs, Required: (Set of) Analytics ID(s) as defined in Table 7.1-2, Target of Analytics Reporting, Analytics Reporting Parameters (including Analytics target period, etc.).

NOTE 1:	Target of Analytics Reporting can be provided per individual Analytics ID.

Inputs, Optional: Analytics Filter Information, preferred level of accuracy of the analytics, preferred level of accuracy per analytics subset, time when analytics information is needed, maximum number of objects requested (max), preferred order of results, maximum number of SUPIs requested (SUPImax), Analytics Accuracy Reporting Information, Analytics Metadata Request, Dataset Statistical Properties, Output strategy, Data time window, Use case context.

NOTE 2:	Analytics Filter Information, Maximum number of objects requested (max), Maximum number of SUPIs requested (SUPImax), Analytics Accuracy Reporting Information, Analytics Metadata Request, Dataset Statistical Properties, Output strategy, Data time window and time when analytics information is needed can be provided per individual Analytics ID.

Outputs, Required: If the request is accepted, then set of the tuple (Analytics ID, Analytics specific parameters). When the request is not accepted, an error response.

Outputs, Optional: Timestamp of analytics generation (required when ADRF is deployed), validity period, confidence, revised waiting time, Analytics Metadata Information, Analytics Accuracy Reporting Information. See clause 6.1.3.

NOTE 3:	Validity period can also be provided as part of Analytics specific parameters for some NWDAF output analytics.

NOTE 4:	When the Target of Analytics Reporting is a SUPI or a GPSI then the request may not be accepted, e.g. user consent is not granted and an error is sent to the consumer. When the Target of Analytics Reporting is an Internal Group Id, or a list of SUPIs/GPSI(s) or any UE, no error is sent, but a SUPI or GPSI is skipped if user consent is not granted.

# * * * * Tenth change * * * *

6.2A	Procedure for ML Model Provisioning

6.2A.0	General

This clause presents the procedure for the ML Model provisioning.

An NWDAF containing AnLF may be locally configured with (a set of) IDs of NWDAFs containing MTLF and the Analytics ID(s) supported by each NWDAF containing MTLF to retrieve trained ML models or may use the NWDAF discovery procedure specified in clause 5.2 for discovering NWDAFs containing MTLF. An NWDAF containing MTLF may determine that further training for an existing ML model is needed when it receives the ML model subscription or the ML model request. To improve the correctness of NWDAF, after providing the ML model to the NWDAF containing AnLF, MTLF can subscribes or requests AnLF with accuracy checking capability to provide accuracy of the analytics generated from the model. Then the NWDAF containing AnLF can provide the ML model accuracy or notify the ML model accuracy degradation to the NWDAF containing MTLF. Based on the provided accuracy information, the MTLF can reselect a new ML model or retrain the existing ML model and provide the reselect new ML model or retrained ML model to the AnLF.

NOTE:	ML Model provisioning/sharing between multiple MTLFs is not supported in this Release of the specification.

# * * * * Eleventh change * * * *

6.2X	ML Model related Accuracy Checking and Provisioning

6.2X.1	General

This clause presents the procedure for the ML Model related accuracy checking and provisioning.

In this Release of the specification an NWDAF containing AnLF is locally configured with (a set of) IDs of NWDAFs containing MTLF and the Analytics ID(s) supported by each NWDAF containing MTLF to retrieve trained ML models via ML Model Provisioning service. When MTLF provides an ML model to an AnLF with accuracy checking capability, the MTLF subscribes/requests AnLF to determine accuracy of the analytics generated from that model.

The NWDAF containing AnLF generates the accuracy information of the analytics generated from that model by comparing predictions of the ML model and its corresponding ground truth data. The accuracy information defines the value determined for the accuracy of the analytics using the trained ML model. Optionally it may include an indication that the determined accuracy value for the trained ML model does not meet the requirement of accuracy for the analytics ID.

After receiving the accuracy information, the MTLF reselect a new ML model or retrain the existing ML model and provides the new ML model to the AnLF.

When an NWDAF containing AnLF with Accuracy checking capability receives trained ML models from an NWDAF containing MTLF, the NWDAF containing MTLF is able to:

-	Request accuracy information of provided model via Nnwdaf_AccuracyProvision_Subscribe or Nnwdaf_Accuracy_Request service operations to activate the mechanisms for checking the accuracy.

-	Provide a new ML model (reselect or retrained) to the AnLF via Nnwdaf_MLModelProvision_Notify or Nnwdaf_MLModelInfo_Request response.

According, the NWDAF containing AnLF with Accuracy checking capability is able to:

-	Receive a subscription or request for accuracy information of received model via Nnwdaf_AccuracyProvision_Subscribe or Nnwdaf_Accuracy_Request service operations.

-	Provide the accuracy information to the MTLF via Nnwdaf_ AccuracyProvision_Notify or Nnwdaf_ Accuracy_Request response.

The request or subscription to accuracy information triggers the monitoring and checking for accuracy information of the analytics generated from ML model. The accuracy information can be requested per Analytics ID or ML Model identifier.

NOTE:	The ML Model identifier is used to uniquely identify an ML model indicate the ML model. The structure and format of the ML Model identifier and its uniqueness are to be defined.

6.2X.2	Procedures for Accuracy Information Subscription

This procedure is used by an NWDAF containing the MTLF to subscribe to receive accuracy information of the analytics generated from ML model which is provided to an NWDAF containing the AnLF. Figure 6.2X.2-1 shows the procedure for accuracy information subscription and provisioning.

Figure 6.2X.2-1: Accuracy Information Subscribe

1.	The NWDAF containing AnLF subscribes or requests for a (set of) trained ML Model(s) associated with a (set of) Analytics ID(s) by invoking the Nnwdaf_MLModelProvision_Subscribe / Nnwdaf_MLModelInfo_Request service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.2A.2.

2.	The NWDAF containing MTLF notifies or responds the NWDAF service consumer (the NWDAF containing AnLF) with the trained ML Model Information (containing a (set of) file address of the trained ML model) by invoking Nnwdaf_MLModelProvision_Notify / Nnwdaf_MLModelInfo_Request response service operation. The content of trained ML Model Information that can be provided by the NWDAF containing MTLF is specified in clause 6.2A.2.

3.	The NWDAF containing MTLF sends a subscription to NWDAF containing AnLF for the accuracy information of the analytics generated from ML model by invoking Nnwdaf_AccuracyProvision_Subscribe service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.2X.4.

4.	When a subscription request is received, the NWDAF containing AnLF starts the monitoring, checking, and generation of the accuracy information related to the ML model indicated in the subscription. The accuracy information is generated by comparing predictions of ML model and its corresponding ground truth data.

5.	NWDAF containing AnLF notifies the accuracy information by invoking Nnwdaf_AccuracyProvision_Notify service operation. The parameters that can be provided by the NWDAF containing AnLF are listed in clause 6.2X.4.

6. 	When the accuracy information is received, the NWDAF containing MTLF determines ML model degradation and reselects a new ML model or retrains the existing ML model that is provided to the NWDAF containing AnLF in step 2.

7. 	The NWDAF containing MTLF also invokes the Nnwdaf_MLModelProvision_Notify / Nnwdaf_MLModelInfo_Request response service operation to notify or respond the NWDAF containing AnLF an available reselected or re-trained ML model.

6.2X.3	Procedures for Accuracy Information Request

This procedure is used by an NWDAF containing the MTLF to request ML model accuracy information of the analytics generated from ML model which is provided to an NWDAF containing the AnLF. Figure 6.2X.3-1 shows the procedure for accuracy information request.

Figure 6.2X.3-1: Accuracy Information Request

1.	The NWDAF containing AnLF subscribes or requests for a (set of) trained ML Model(s) associated with a (set of) Analytics ID(s) by invoking the Nnwdaf_MLModelProvision_Subscribe / Nnwdaf_MLModelInfo_Request service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.2A.2.

2.	The NWDAF containing MTLF notifies or responds the NWDAF service consumer (the NWDAF containing AnLF) with the trained ML Model Information (containing a (set of) file address of the trained ML model) by invoking Nnwdaf_MLModelProvision_Notify / Nnwdaf_MLModelInfo_Request response service operation. The content of trained ML Model Information that can be provided by the NWDAF containing MTLF is specified in clause 6.2A.2.

3.	The NWDAF containing MTLF sends a request to NWDAF containing AnLF for the accuracy information of the analytics generated from ML model by invoking Nnwdaf_Accuracy_Request service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.2X.4.

4.	When a request is received, the NWDAF containing AnLF starts the monitoring, checking, and generation of the accuracy information related to the ML model. The accuracy information is generated by comparing predictions of ML model and its corresponding ground truth data.

5.	NWDAF containing AnLF responds the accuracy information by invoking Nnwdaf_Accuracy_Request response service operation. The parameters that can be provided by the NWDAF containing AnLF are listed in clause 6.2X.4.

6. 	When the accuracy information is received, the NWDAF containing MTLF determines ML model degradation and reselects a new ML model or retrains the existing ML model that is provided to the NWDAF containing AnLF in step 2.

7. 	The NWDAF containing MTLF also invokes the Nnwdaf_MLModelProvision_Notify / Nnwdaf_MLModelInfo_Request response service operation to notify or respond the NWDAF containing AnLF an available reselected or re-trained ML model.

6.2X.4	Contents of Accuracy Provisioning

The consumers of the accuracy provisioning service (i.e. an NWDAF containing MTLF) as described in clause 7 may provide the input parameters as listed below:

-	A list of Analytics ID(s): identifies the analytics for which the monitored ML model is used.

-	 [OPTIONAL] ML Model identifier: identifies the ML model for which the accuracy information needs to be checking.

-	[OPTIONAL] ML model accuracy metrics: indicates the metrics to calculate the accucacy information, e.g, Accuracy, Precision, Mean Absolute Error (MAE), etc.

-	 [OPTIONAL] Accuracy Information Periodicity: determines the intervals of time when AnLF provides the accuracy information to the MTLF.

-	[OPTIONAL] Accuracy Information reporting threshold: identifies the preferred level or value of accuracy requested for the ML model.

-	(Only for Nnwdaf_AccuracyProvision_Subscribe) A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from the NWDAF containing AnLF with this subscription.

The NWDAF containing AnLF provides to the consumers of the accuracy provisioning service (i.e. an NWDAF containing MTLF) as described in clause 7, the output information as listed below:

-	(Only for Nnwdaf_MLModelProvision_Notify) The Notification Correlation Information.

-	Accuracy information generated for each analytics ID or ML model, including the determined accuracy value of the analytics using the trained ML model for an analytics ID or ML model, and optionally an indication that the determined accuracy value does not meet the requirement of accuracy for the analytics ID or the ML model.

# * * * * Twelfth change * * * *

7	Nnwdaf Services Description

7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF |
| Nnwdaf_AccuracyProvision | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_Accuracy | Request | Request/ Response | NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF. NOTE 4: The Nnwdaf_AccuracyProvision service and the Nnwdaf_Accuracy service are provided by an NWDAF containing AnLF and consumed by an NWDAF containing MTLF. |  |  |  |

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |

# * * * * Thirteenth change * * * *

7.X	Nnwdaf_AccuracyProvision services

7.X.1	General

Service Description: This service enables the consumer to receive a notification when the accuracy information matching the subscription parameters becomes available.

When the subscription is accepted by the NWDAF containing AnLF, the consumer NF, i.e. the NWDAF containing MTLF, receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription. The modification of ML model accuracy subscription can be enforced by NWDAF based on operator policy and configuration.

7.X.2	Nnwdaf_AccuracyProvision_Subscribe service operation

Service operation name: Nnwdaf_AccuracyProvision_Subscribe.

Description: Subscribes to NWDAF accuracy provision with specific parameters.

Inputs, Required: (set of) Analytics ID(s) defined in Table 7.1-2, Notification Target Address (+ Notification Correlation ID).

Inputs, Optional: Subscription Correlation ID (in the case of modification of the ML model subscription), ML Model identifier which identifies the ML model for which the accuracy information needs to be checking, ML model accuracy metrics, Accuracy Information Periodicity, Accuracy Information reporting threshold.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: None.

7.X.3	Nnwdaf_AccuracyProvision_Unsubscribe service operation

Service operation name: Nnwdaf_AccuracyProvision_Unsubscribe.

Description: unsubscribe to NWDAF accuracy provision.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

7.X.4	Nnwdaf_AccuracyProvision_Notify service operation

Service operation name: Nnwdaf_AccuracyProvision_Notify.

Description: NWDAF notifies the accuracy information to the consumer instance which has subscribed to the specific NWDAF service.

Inputs, Required: Set of the tuple (Analytics ID, Accuracy information), Notification Correlation Information.

Inputs, Optional: ML Model identifier.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

7.Y	Nnwdaf_Accuracy service

7.Y.1	General

Service description: this service enables the consumer to request and get from NWDAF containing AnLF Accuracy Information.

7.Y.2	Nnwdaf_Accuracy_Request service operation

Service operation name: Nnwdaf_Accuracy_Request

Description: The consumer requests NWDAF Accuracy Information.

Inputs, Required: (Set of) Analytics ID(s) defined in Table 7.1-2.

Inputs, Optional: ML Model identifier which identifies the ML model for which the accuracy information needs to be checking, ML model accuracy metrics.

Outputs, Required: Set of the tuple (Analytics ID, Accuracy information).

Outputs, Optional: ML Model identifier.

# * * * * End of changes * * * *