SA WG2 Meeting #S2-154-AH-e (e-meeting)	S2-2300312

16 - 20 January 2023, Electronic Meeting	(revision of S2-22xxxxx)

Source:	Ericsson

Title:	KI#4 Update Conclusion: Update SL Positioning server UE functionalities in case of OoO scenario

Document for:	Approval

Agenda Item:	9.5.1

Work Item / Release:	FS_Ranging_SL / Rel-18

Abstract: This contribution proposes to update SL Positioning Server UE functionalities in OoO scenario.

# 1. Discussion

SL positioning server UE was introduced for that purpose when target or reference UE have not the capability to calculate location information. This is valid only for Out of Coverage scenario since if at least for partitional coverage network is available and LMF can execute location calculation.

Additionally RAN decided (R2-2213131) that SL positioning server UE can be used for method determination, assistant data distribution and SL reference UE selection.

# 2. Proposal

SL Positioning Server UE can be used for positioning calculation, method determination, assistant data distribution and SL reference UE selection when network and LMF is not available in case of Out of Coverage scenario.

* Start of change *

## 8.4	Key Issue #4: Control of Operations for Ranging/Sidelink positioning

For Key Issue #4 "Control of Operations for Ranging/Sidelink positioning", the followings are taken as conclusions:

-	A Ranging/SL Positioning layer is introduced on the Target UE/Reference UE/SL Positioning Server UE under Application layer and above AS layer to handle service request received from application layer and to control the Sidelink Positioning and Ranging operation:

-	Functionalities supported by the Ranging/SL Positioning layer include discovery of the UE(s) in proximity that can participate in Sidelink Positioning and Ranging service sessions and control signalling between UEs or among a group of UEs or between UE and LMF to manage and coordinate the Sidelink Positioning and Ranging operations.

Editor's note:	Whether group management is supported by the Ranging/SL Positioning layer or the Application layer is FFS.

-	Information exchanged between Ranging/SL Positioning layer and lower layer includes one time or periodic ranging, ranging for distance or direction measurement or both.

Editor's note:	The above information may be updated.

Editor's note:	Whether the Ranging/SL Positioning layer is over V2X/ProSe layer or AS layer is up to RAN WG to decide. Whether Layer 2 ID or Application Layer ID is exchanged between Ranging/SL Positioning layer and lower layer depends if the lower layer includes V2X/ProSe layer.

-	Ranging/Sidelink Positioning Protocol (RSPP) is introduced for SR5 over the PC5 reference point between the UEs (i.e. Target UE, Reference UE, Assistant UE, Located UE and SL Positioning Server UE), which includes the following procedures:

-	exchange the Ranging/Sidelink Positioning capability.

-	exchange the Ranging/Sidelink Positioning assistant data.

NOTE 1:	What assistant data are required to be exchanged will be coordinated with RAN WGs.

-	exchange Ranging/Sidelink positioning measurement data/result.

Editor's note:	The above information may be updated.

-	V2X Communication procedures as defined in TS 23.287 [3] and 5G ProSe Direct Communication procedures as defined in TS 23.304 [4] are used for the RSPP communication between UEs over PC5.

Editor's note:	Whether RSPP is over PC5-S or PC5-U or (partially) over PC5-D is up to RAN WG to decide.

-	A LMF may be involved when at least one of the Target UE and the Reference UE are in the network coverage. In that case, a suitable LMF needs to be selected by the AMF based on the LMF Ranging/SL Positioning capabilities defined in clause 6.27.2 and the service requests. The protocol used between UE and LMF will be decided by RAN and could be an extension of LPP, a new protocol or both, including the following procedures (to be confirmed by RAN WGs):

-	Triggering Ranging/Sidelink positioning.

-	exchange the Ranging/Sidelink Positioning capability.

-	exchange the Ranging/Sidelink Positioning assistant data.

NOTE 2: what assistant data are required to be exchanged will be coordinated with RAN WGs.

-	exchange Ranging/Sidelink positioning signal measurement data/result.

-	deliver Ranging/Sidelink positioning service request/response.

NOTE 3:	The full support of LPP is not required for operating only SL Positioning/Ranging; in that case only the extension of LPP, a new protocol or both is required.

Editor's note:	The above information may be updated.

-	A SL Positioning Server UE can be discovered and selected for result calculation for out of coverage when LMF cannot support calculation assistant due to missing network coverage. Additionally, functionalities of positioning method determination, assistant data distribution and SL reference UE selection can be performed by SL positioning server UE for out-of-coverage scenario.

-	Multiple UEs may be involved in a single Ranging/Sidelink Positioning session.

-	Whether measurement results or Ranging/Sidelink Positioning results are exchanged over the SR5 depends on the negotiation during the control signalling.

* End of change *