3GPP TSG-SA WG2 Meeting #154AH (e-meeting) 	S2-2300332

# January 16 - 20, 2023, Elbonia		 		 			 						   					   (revison of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0161 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve ENs on Group Message Delivery and corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | #1 EN Resolution In SA2#154, Group Message Delivery function has been introduced in TS 23.247 with ENs: - EN for service operation definition: Editor’s Note: It requires further investigation on whether the service operation should align more with T8 interface (delivery & modify) or follow the SBI design principles (CRUD). The group message delivery service is offered as a 5G service, which should follow the SBI design principles. It is proposed to adopt Nnef_MBSGroupMsgDelivery service (Create, Update, Delete, StatusNotify). - EN for TLTRI vs. Group Message Correlation ID: Editor’s Note: It is FFS whether TLTRI (T8 Long Term Transaction Reference ID) should be kept or use Group Message Correlation ID instead. TLTRI is a EPS specific parameter name which applies to T8 interface. For group message delivery service, to follow SBI design principles, it is recommended to adopt Group Message Correlation ID. - EN for service area mismatch: Editor’s Note: It is FFS for the case that the service area in the request is different with the service area associated with the TMGI which is provided during the TMGI allocation. The NEF receives Group Message Delivery Request with service area information, and then the NEF performs TMGI allocation or MBS session creation. The NEF can provide MBS service area to the MB-SMF in TMGI allocation request or MBS session create request. Therefore, service area mismatch in Group Message Delivery Request will not happen. If the MB-SMF cannot handle the whole service area provided by the AF, it follows the normal error handling. Based on the above, The EN can be removed. - EN for StatusSubscribe and StatusUnsubscribe: Editor’s Note: It requires further investigation on whether StatusSubscribe and StatusUnsubscribe are needed or not. The NEF needs to notify the AF about the delivery status of the group message. When AF requests a group message delivery, it implicitly subscribe to the delivery status. The explicit subscription is not considered necessary, and the EN can be removed. - EN for service operation and parameters: Editor’s Note: The service operation and parameters are FFS. Considering the service operation and parameters have been clarified above, the EN can be resolved. #2 In clause 6.15 - References to procedures are not needed in high level feature description - “Object Delivery Method” should be “Object Distribution Method” per TS 26.502. - Text related to eMBMS is not needed. - “MBS Session start for broadcast” is part of “MBS Session creation”, thus rewording is needed. #3 In clause 7.5.1 (Group Message Delivery via MBS Broadcast), NEF notifies the AF of the status of group message delivery after getting status notification from the MBSF (but not any time after user data ingestion in step 11). It’s proposed to clarify the MBSF notification to NEF. #4 In clause 7.5.2, move the cancellation to a new sub-clause. #5 Miscellaneous correction/improvement are also needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | #1 Resolve the ENs as follows: - Update the service operations to follow the SBI design principles - Adopt Group Message Correlation ID, instead of TLTRI - Resolve the EN of the service area mismatch. - State that explicit subscription is not specified in this release - Clarify the service operation and parameters #2 Clause 6.15 - Remove the references to the procedures. - “Object Delivery Method” updated to “Object Distribution Method” - The text related about the relation with the File Delivery Method in eMBMS is removed. - Remove “MBS session start for broadcast” as this is part of MBS Session creation. - Remove text “which is essential for group message delivery”. - Other wording improvement #3 Clause 7.5.1 clarify that the NEF notifies the AF of the status of group message delivery after getting status notification from the MBSF (which involves MBSF and MBSTF interaction) #4 Clause 7.5.2 Move the description related to cancellation of previously submitted Group message to a new subclause 7.5.X. #5 Miscelleneous editorial fix and wording improvement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete function Not compliant with SBI design principle |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.15, 7.5.1, 7.5.2, 7.5.X (new), 9.4.1, 9.4.4.2, 9.4.4.3, 9.4.4.3a (new), 9.4.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS23.501 CR 3755 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

## 6.15	Group Message Delivery

Group Message Delivery via MBS Session is a feature that allows an AF to deliver a payload to a group of UEs located in a particular geographical area via a broadcast MBS Session. The AF may request to deliver group message and the AF may also request to recall (i.e. cancel) or replace a previously submitted group message.

Group Message Delivery utilizes the Object Distribution Method in MBSTF specified in TS 26.502 [18]. The Object Distribution Method can benefit from Application Layer Forward Error Correction (AL-FEC) to achieve reliable delivery.

In Group Message Delivery via MBS Session, the NEF is responsible for handling the group message delivery request from the AF, that is, the NEF transforms the group message into a file and determines the meta data information of the file. Over control plane, the NEF provisions Application Service (i.e., MBS User Service creation and MBS User Data Ingest Session creation as specified in TS 26.502 [18]), which then triggers the MBS session creation towards 5GC and NG-RAN. Over user plane, the NEF is responsible for ingesting the file to the MBSTF, which then delivers the file to the UE(s) via 5GC Shared MBS traffic delivery.

NOTE:	The AF can invoke the Nmbsmf service operations offered by the MB-SMF (optionally via the NEF) for Transport Only Mode, as supported in Rel-17.

* * * * Next Changes * * * *

## 7.5	MBS procedures for Group Message Delivery

### 7.5.1	Group Message Delivery via MBS Broadcast

Figure 7.5.1-1: Group Message Delivery via MBS Broadcast

NOTE 1:	 The steps in this clause for group message delivery to the UE(s) follow the procedures specified in TS 26.502 [18], where the NEF acts as an MBS Application Provider.

1.	The AF sends Nnef_MBSGroupMsgDelivery_Create Request to the NEF. The Request message contains the Group Message Payload, MBS service area, Group Message Delivery Start Time, Stop Time and External Group Identifier.

2.	The NEF checks authorization of the AF. If geographical area information or civic address information was provided by the AF as MBS service area, the NEF translates the MBS service area to Cell ID list or TAI list. The NEF may further check the MBS capability within the MBS service area.

NOTE 2:	The NEF is mandated for group message delivery.

3.	The NEF transforms the group message payload into a file, and determines the meta data information of the file (e.g. File URL, etc.). The NEF assigns a Group Message Correlation ID that identifies this group message delivery request.

If Application Service Provisioning specified in TS 26.502 [18] has not been performed, step 4 to step 8 are executed. Otherwise, they can be skipped.

4.	The NEF performs Application Service Provisioning by invoking Nmbsf_MBSUserService_Create and Nmbsf_MBSUserDataIngestSession_Create towards the MBSF using Object Distribution Method as specified in TS 26.502 [18].

5.	The MBSF performs MBS Session Creation as specified in clause 7.1.1.2 or clause 7.1.1.3.

6.	The MBSF performs Distribution Session Provisioning as specified in TS 26.502 [18]. The MBSF invokes Nmbstf_MBSDistributionSession_Create on the MBSTF, passing the parameters of the MBS Distribution Session received in step 4 to the MBSTF.

7.	The MB-SMF initiates the MBS Session Start for Broadcast procedure as specified in step 2 to step 9 in clause 7.3.1.

8.	If the MBSF performs the service announcement, it initiates the MBS User Service Announcement as specified in TS 26.502 [18]. The application may receive the appropriate information through the MBS-6 API from the MBS Client (see TS 26.502 [18]). The NEF may receive the service announcement information via Nmbsf_MBSUserDataIngestSession_StatusNotify callback service operation (see TS 26.502 [18]).

9.	The NEF sends Nnef_MBSGroupMsgDelivery_Create Response (Group Message Correlation ID, Acceptance Status, Cause) to the AF. The Acceptance Status indicates whether the group message delivery request is accepted or not. If not, the Cause is included indicating the appropriate failure reason. If the AF performs the service announcement, the service announcement information containing the file meta data can be optionally included in the Group Message Response. The NEF may further include the area where MBS is not supported in the Nnef_MBSGroupMsgDelivery_Create response to the AF.

10.	If the AF needs to perform the Service Announcement, the AF sends the application service announcement to the UE as specified in TS 26.502 [18].

11.	The NEF performs the User Data Ingestion towards the MBSTF as specified in TS 26.502 [18]. The NEF may push the file to the MBSTF or let MBSTF pull the file from the NEF.

12.	The MBSTF performs packetization and optionally FEC encoding as specified in TS 26.502 [18].

13.	The MBSTF delivers the packets to the MB-UPF to NG-RAN, and NG-RAN broadcast to the UE as specified in step 13 to step 15 in clause 7.3.1.

14.	Based on the service announcement information received in step 8 or step 10, when the UE receives the packets, the MBS client performs FEC decoding to restore the file and gets the group message from the file as specified in TS 26.502 [18]. The MBS Client can expose the file towards the application in the UE using the MBS-7 API (see TS 26.502 [18]).

15. The MBSTF notifies the MBSF of User Data distribution status, and the MBSF then notifies the MBSF of the User Data ingestion status as specific in TS 26.502 [18].

16.	The NEF sends Nnef_MBSGroupMsgDelivery_StatusNotify to the AF, containing Group Message Correlation ID, Delivery Status. The Delivery Status indicates whether delivery of Group Message Payload is successful or not. The delivery status information is received by the NEF via Nmbsf_MBSUserDataIngestSession_StatusNotify service operation (see TS 26.502 [18]).

### 7.5.2	Modification of previously submitted Group message

Figure 7.5.2-1: Modify Group Message Delivery via MBS Broadcast

NOTE:	 The steps in this clause for modification of group message delivery to the UE(s) follow the procedures specified in TS 26.502 [18]  where the NEF acts as an MBS Application Provider.

1.	The AF sends Nnef_MBSGroupMsg_Update Request to the NEF. The Request message contains the Group Message Correlation ID, Group Message Payload, MBS service area, Group Message Delivery Start Time, Stop Time and External Group Identifier. The NEF identifies the associated MBS Service by the external Group Identifier. The Group Message Correlation ID indicates the transaction to be modified.

2.	The NEF checks authorization of AF. If geographical area information or civic address information was provided by the AF as MBS service area, the NEF translates the MBS service area to Cell ID list or TAI list. The NEF may further check the MBS capability within the MBS service area.

For Group Message Payload update, steps 3 to step 6 are executed:

3.	The NEF transforms the group message payload into a file, and use the determined file meta data (e.g. File URL, etc.) in clause 7.5.1.

4.	If the NEF pushes the updated file to the MBSTF, step 4 to step 5 can be skipped. If the MBSTF pulls the updated file from the NEF, the NEF updates MBS User Data Ingest Session on the MBSF as specified in TS 26.502 [18]. The update service operation needs to indicate an update of the file containing the updated group message.

5.	The MBSF updates MBS Distribution Session on the MBSTF as specified in TS 26.502 [18]. The update service operation needs to indicate an update of the file containing the updated group message.

6.	The NEF pushes the update file to the MBSTF or the MBSTF pulls the updated file from the NEF. And the MBSTF delivers the updated file towards the MB-UPF in 5GC as specified in clause 4.3.3.2 of TS 26.502 [18].

For MBS service area update, steps 7 and step 8 are executed:

7.	The NEF updates MBS User Service on the MBSF as specified in TS 26.502 [18]. The update service operation indicates MBS service area update.

8.	The MBSF performs MBS Session Update as specified in clause 7.1.1.6 or clause 7.1.1.7 to update MBS service area, which triggers MBS Session Update for Broadcast as specified in clause 7.3.3.

9.	The NEF sends Nnef_MBSGroupMsgDelivery_Update Response (Acceptance Status, Cause) to the AF . The Acceptance Status indicates whether the group message update request is accepted or not. If not, the Cause is included indicating the appropriate failure reason. The NEF may further include the area where MBS is not supported in the Nnef_MBSGroupMsgDelivery_Update response to the AF.

After the modified group message is delivered, the NEF sends Nnef_MBSGroupMsgDelivery_StatusNotify to the AF as described in step 16 in clause 7.5.1.

### 7.5.X	Cancellation of previously submitted Group message

Figure 7.5.X-1: Cancelling Group Message Delivery via MBS Broadcast

NOTE:	The steps in this clause for cancelling the group message delivery to the UE(s) follow the procedures specified in TS 26.502 [18] where the NEF acts as an MBS Application Provider.

1.	The AF sends Nnef_MBSGroupMsgDelivery_Delete Request (Group Message Correlation ID, External Group Identifier) to the NEF. The NEF identifies the associated MBS Service by the External Group Identifier. The Group Message Correlation ID indicates the transaction to be cancelled.

2.	The NEF checks authorization of the AF.

Steps 3 to step 5 are executed if the group message delivery has started and MBSF needs to inform the MBS Client about the cancellation of the file delivery. Otherwise, they can be skipped.

3.	The NEF updates MBS User Data Ingest Session on the MBSF as specified in TS 26.502 [18] indicating the cancellation of the file delivery.

4.	If the MBSTF pulled the file from the NEF previously, the MBSF updates MBS Distribution Session on the MBSTF as specified in TS 26.502 [18] indicating cancellation of the file delivery and the MBSTF stops the file delivery.

5.	If the NEF pushed the file previously to the MBSTF, the NEF may send HTTP DELETE to the MBSTF to cancel the file delivery. The MBSTF stops the file delivery.

6.	The NEF destroys MBS User Data Ingest Session on the MBSF as specified in TS 26.502 [18].

7.	The MBSF destroys MBS Distribution Session on the MBSTF as specified in TS 26.502 [18].

8.	The MBSF performs MBS Session Deletion as specified in clause 7.1.1.4 or clause 7.1.1.5, which includes MBS Session Release for Broadcast as specified in clause 7.3.2.

9.	The NEF sends Nnef_MBSGroupMsgDelivery_Delete Response (Acceptance Status, Cause) to the AF. The Acceptance Status indicates whether the Nnef_MBSGroupMsgDelivery_Delete Request is accepted or not. Cause is included when the Nnef_MBSGroupMsgDelivery_Delete Request is not accepted.

* * * * Next Changes * * * *

### 9.4.1	General

The following table illustrates the NEF Services for MBS.

Table 9.4.1-1: NF services provided by NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer (s) |
| --- | --- | --- | --- |
| Nnef_MBSTMGI | Allocate | Request/Response | AF |
|  | Deallocate | Request/Response | AF |
|  | ExpiryNotify | Subscribe/Notify | AF |
| Nnef_MBSSession | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | StatusNotify | Subscribe/Notify | AF |
|  | StatusSubscribe |  | AF |
|  | StatusUnsubscribe |  | AF |
| Nnef_MBSGroupMsgDelivery | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | StatusNotify | Subscribe/Notify | AF |

* * * * Next Changes * * * *

### 9.4.4	Nnef_MBSGroupMsgDelivery Service

#### 9.4.4.1	General

Service description: NF Service Consumer can use this service to request group message delivery or to modify or to cancel previously submitted group message delivery.

#### 9.4.4.2	Nnef_MBSGroupMsgDelivery_Create service operation

Service operation name: Nnef_ MBSGroupMsgDelivery_Create

Description: This service is used by the NF Service Consumer to request group message delivery.

Inputs, Required: Group Message Payload, MBS service area, Group Message Delivery Start Time, Stop Time, External Group Identifier.

Inputs, Optional: None.

Outputs, Required: Group Message Correlation ID, Delivery Status

Outputs, Optional: Cause, service area without MBS capability.

#### 9.4.4.3	Nnef_MBSGroupMsgDelivery_Update service operation

Service operation name: Nnef_MBSGroupMsgDelivery_Update

Description: This service is used by the NF Service Consumer to replace group message delivery.

Inputs, Required: Group Message Correlation ID, External Group Identifier.

Inputs, Optional: Group Message Payload, MBS service area, Group Message Delivery Start Time, Stop Time.

Outputs, Required: Acceptance Status

Outputs, Optional: Cause, service area without MBS capability.

#### 9.4.4.3a	Nnef_MBSGroupMsgDelivery_Delete service operation

Service operation name: Nnef_MBSGroupMsgDelivery_Delete

Description: This service is used by the NF Service Consumer to recall (or cancel) group message delivery.

Inputs, Required: Group Message Correlation ID, External Group Identifier.

Inputs, Optional: None.

Outputs, Required: Acceptance Status

Outputs, Optional: Cause.

#### 9.4.4.4	Nnef_MBSGroupMsgDelivery_StatusNotify service operation

Service operation name: Nnef_ MBSGroupMsgDelivery_StatusNotify

Description: This service is used by the NEF to notify the NF service consumers of the status of a group message delivery.

NOTE:	This notification corresponds to an implicit subscription.

Inputs, Required: Status Information, Group Message Correlation ID.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

* * * * End of Changes * * * *