3GPP TSG-SA WG2 Meeting #154AH-E	S2-2300367

# E-Meeting, 16 – 20 Jan. 2023	was S2-21xxxxx

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3882 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enable the UE to access Localized services after the UE registers to a Hosting network |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on the conlcusion in KI#5 in TR 23.700-08: - When the hosting network is an SNPN which provides localized services and the home network is an HPLMN, the architecture specified in clause 5.30.2.7 of TS 23.501 [3] is reused for the UE to access both home network services (using the SNPN as underlay network) and localized services (via the SNPN). - If the hosting network is a PNI-NPN, existing mechanisms (e.g. roaming architecture, network slicing, etc.) are reused for the UE to access home network services. - The existing URSP rules or LADN feature can be re-used for a UE to access the Localized services after the UE has registered to a Hosting network. URSP rules are pre-configured in the UE, provisioned by the PCF of the home network or provisioned by the PCF of the hosting network according to existing principles. Based on the above conclusion, the existing mechanisms are re-uesed and applicable to the UE for access the Localized services after the UE has registered to a Hosting network. The CR is propoed to include the abvoe conclusion in an Annex since there is no changes to be made to the existing mechanism. The Annex is used to described how the UE accesses the Localized services after the UE has registered to a Hosting network by reusing the existing mechanism. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The description is added to describe - The network architecture in clause 5.30.2.7 is used for an SNPN as Hosting network. The existing Roaming architecture is used for a PNI-NPN as Hosting network - How the network assigns the information of the Localized services to the UE using the existing mechanism - How the UE accesses the Localized services based the information provisioned by the network |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE cannot access the Localize services after the UE registers to a Hosting network |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex x (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** FIRST CHANGE ****

X.Y	Enable the UE to access the Localized services

If a Hosting network is an SNPN, the network architecture in clause 5.30.2.7 is reused and if a Hosting network is a PNI-NPN, the existing Roaming architecture is used.

After the UE has successfully registered to a Hosting network (i.e. SNPN as Hosting network or PNI-NPN as Hosting network), the existing URSP rules or LADN feature can be re-used for a UE to access the Localized services. The URSP rules are pre-configured in the UE, provisioned by the PCF of the home network or provisioned by the PCF of the hosting network according to existing principles.

**** END OF CHANGES ****