3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2300373

# Jan 16 - 20, 2023, Electronic Meeting							                   (revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3884 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DL PDU Set Handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-01-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR describes how to identify a PDU belonging to a PDU Set and to provide PDU Set information in the extended GTP-U header to assist NG-RAN to handle diffentiated and integrated packets. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describe how PDUs from a PDU set via N6/19 is encapsulated within a GTP-U header and sent over the N3 interface to NG-RAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.x (new), 5.37.y (new), 5.37.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is the result of SI FS_XRM. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** FIRST CHANGE ****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[x]	draft-ietf-avtext-framemarking: "IETF experimental draft-ietf-avtext-framemarking-13 - Frame Marking RTP Header Extension".

**** NEXT CHANGE ****

### 5.37.x 	Support for PDU Set Identification

The PSA UPF may identify DL PDUs that belong to a PDU Set based on instruction from SMF and inspecting the protocol header of the traffic received on the N6/N19 interface. The PDU Set Identification procedure to derive the PDU Set information is according to one of the following procedures indicated in the assistance information AF may provide SMF (described in clause 5.37.z of TS 23.501):

- By matching the codec agnostic RTP/SRTP header extension of non-scaleable streams described in draft-ietf-avtext-framemarking-13 [x], where all PDUs belonging to a PDU Set have the same RTP Payload Type (e.g., Dynamic allocation), RTP SSRC and RTP timestamp. In addition, the RTP/SRTP header extensions identify:

  - first PDU of a PDU Set.

  - end PDU of a PDU Set.

  - independent PDU Set, e.g., intra-frame H.264/H.265 IDR, VPX keyframe.

  - discardable PDU Set.

- UPF implementation specific information provision locally, details of which are not standardized and may be based on traffic characteristics or IP-tuple.

### 5.37.y 	PDU Set Information

For all PDUs identified as belonging to the same PDU Set, described in clause 5.37.x, the differentiated and integrated characteristics are determined by the PSA UPF to generate PDU Set Information containing:

- PDU Set Sequence Number:

  - a cyclical counter to uniquely identify the PDU Set.

- PDU Sequence Number:

  - a cyclical counter to uniquely identify each PDU of the PDU Set

  - counter is rest when a new PDU Set is identified.

- End PDU of the PDU Set

- PDU Set Size (optional):

  - if provided by AF as part of assistance information or in a new RTP extension header.

- PDU Set Importance:

  - to identify the importance of a PDU Set within a QoS flow. NG-RAN may use this indication for PDU Set level packet discarding in the presence of congestion.

### 5.37.z 	Delivering PDU Set Information to NG-RAN

The PDU Set Information described in clause 5.37.x, is provided by PSA UPF to NG-RAN via an extended GTP-U header.

**** END OF CHANGES ****