3GPP TSG-SA WG2 Meeting #154-AH-e	S2-2300391

# Jan 16 - 20, 2023

Title:	Draft LS reply on reply LS on XR and Media Services

Response to:	Draft (R1-2212994/S2-2300042)

Release:	Rel-18

Work Item:	FS_XRM, XRM, FS_NR_XR_enh

Source:	3GPP TSG SA WG2

To:	RAN WG1, RAN WG2

Cc:	RAN WG3,SA WG4

Contact Person:

Name:	Xiaowan Ke

Tel. Number:

E-mail Address:	xiaowan(dot)ke(at)vivo(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 would like to thank RAN1’s reply LS (R1-2212994/S2-2300042) on XR and Media Services. SA2 would like to clarify the following aspects as requested by RAN1’s reply LS:

Q1-1: how to understand the “short period of time” in the definition of data burst.

SA2 Answer: The “short period of time” means the interval between the reception time of the first packet and the reception time of the last packet of the Data Burst at the destination. Ideally, there is an interval without any data arrival between two successive Data Bursts.

Q1-2: whether “the period of time” for data burst can be varied between the data burst in different data burst periods, and if the period of time can be varied, what is the expected range of the period of time for data burst.

SA2 Answer: generally, “the period of time” is varied for different Data Bursts and its duration is related to the burst data amount of the Data Burst and the reception time interval of each two successive packets of a Data Burst. SA4(in S2-2210181) used to clarify the server is unknow the data amount of a Data Burst: “In addition, as initial parts of Data Burst are typically released prior to the completion of the entire Data Burst, the size of the Data Burst is generally unknown when the first part of the Data Burst is released to the network.” And the reception time interval of each two successive packets of a Data Burst may be varied.

Generally, either “the period of time” or the range of “the period of time” is not explicitly signalled in the XR bitstream from the server. “the period of time” for each Data Burst is unknown until receiving the last packet of Data Burst. So far, End of Data Burst indication is supported to be indicated in the header of the last packet of the Data Burst from the CN.

Q2: RAN1 would like to ask SA2 whether the following information could be provided from the core network

- Data burst start time information (e.g., the time of 1st arrival packet of Data burst)

- Notification of changes in XR traffic parameters, e.g., video frame periodicity, jitter statistics

SA2 Answer: The arrived time of first packet of Data burst cannot be provided by 5GC to the NG RAN since it cannot be predicted by the 5GC or provided by the application in advance due to N6 jitter. The RTP protocol header information (e.g. the timestamp, SN) may be useful to NG RAN to decide the 1st arrival packet of the Data Burst. Also, SA2 has agreed to provide the periodicity and the periodicity associated N6 jitter range to the NG RAN via NG message.

XR traffic parameters is assumed to be semi-static and not change frequently. The notification of changes will be triggered in time by the CN to the RAN in case changes happens.

2. Actions:

To RAN WG1 and RAN WG2:

ACTION: 	SA2 kindly asks RAN WG1 and RAN WG2 to take the above information into account and reply, if needed.

3. Date of Next TSG SA WG2 Meetings:

SA WG2 Meeting #154-AH-e      	16 – 20 January 2023			   e-meet

SA WG2 Meeting #155     	20 – 24 February 2023			   Athens, Greece