3GPP TSG-WG SA2 Meeting #154AH-e	S2-2300462

# 16 – 20 January, 2023, Electronic meeting		(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0266 | rev | 0 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Content, source and procedure of positioning assistance data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Inspur |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2022-12-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following 3 reasons for change are proposed. Please refer to S2-2300462 for more details. - In 5G_eLCS_Ph3, two types of positioning assistance data are proposed， which are GNSS positioning assistance data and PRU positioning assistance data. The positioning assitance data is related to the positioning reference equipment and its own known location and LMF can utilize the reference location information to optimize the positioning accuracy of target UE. But the current TS does not describe the content systematically. - As for PRU, the current positioning assistance data and its procurement procedure are all misleading. - Moreover, the positioning assistance data procurement procedure for GNSS positioning assistance is introduce to cover the procurement of both GNSS positioning assistance data and PRU positioning assistance data. It is propsed to extent subclause 5.11 to generalize the description regarding positioning assistance data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Extend subclause 5.11 to cover the PRU positioning assistance data and generalize the description into positioning assistance data. - Revise the contents of PRU positioning assistance data. - Reuse the procedure of GNSS positioning assistance data collection and generalize the procedure of positioning assistance data procurement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The lack of description on PRU at the high level. - There is no general description on positioning assistance data. - The content of PRU positioning assistance data is misleading. - There is no general procedure of positioning assistance data procurement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.8, 5.11, 6.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.3.8	Location Management Function, LMF

The LMF manages the overall co-ordination and scheduling of resources required for the location of a UE that is registered with or accessing 5GCN. It also calculates or verifies a final location and any velocity estimate and may estimate the achieved accuracy. The LMF receives location requests for a target UE from the serving AMF using the Nlmf interface. The LMF interacts with the UE in order to exchange location information applicable to UE assisted and UE based position methods and interacts with the NG-RAN, N3IWF or TNAN in order to obtain location information.

The LMF shall determine the result of the positioning in geographical co-ordinates as defined in TS 23.032 [8] and/or in local co-ordinates as defined in TS 23.032 [8]. If requested and if available, the positioning result may also include the velocity of the UE. The coordinate type(s) is determined by LMF when receiving a location request, based on LCS Client type and supported GAD shapes. If the location request indicates regulatory LCS Client type the LMF shall determine a geographical location and optionally a location in local coordinates. For location request indicates a value added LCS Client type, the LMF may determine the UE location in local coordinates or geographical co-ordinates or both. If the supported GAD shapes is not received or Local Co-ordinates is not included in the supported GAD shapes, the LMF shall determine a geographical location.

NOTE 1:	Some RAT independent position methods (e.g. GNSS based position methods) can only determine a UE location in geographical co-ordinates. In such a case, the LMF may translate a UE location in geographical co-ordinates into a location in local co-ordinates when an origin for the local co-ordinates has known global coordinates. When an origin for the local co-ordinates does not have known global coordinates, position methods that can only determine a UE location in geographical co-ordinates cannot be used to determine a UE location in local co-ordinates.

Additional functions which may be performed by an LMF to support location services include the following.

-	Support a request for a single location received from a serving AMF for a target UE.

-	Support a request for periodic or triggered location received from a serving AMF for a target UE.

-	Determine type and number of position methods and procedures based on UE and PLMN capabilities, QoS, UE connectivity state per access type, LCS Client type, co-ordinate type and optionally service type.

-	Report UE location estimates directly to a GMLC for periodic or triggered location of a target UE.

-	Support cancelation of periodic or triggered location for a target UE.

-	Support the provision of broadcast and GNSS positioning assistance data to UEs via NG-RAN in ciphered or unciphered form and forward any ciphering keys to subscribed UEs via the AMF.

-	Support change of a serving LMF for periodic or triggered location reporting for a target UE.

-	Support of receiving stored UE Positioning Capability from AMF and support of providing updated UE Positioning Capability to AMF.

-	Map the UE location to a geographical area where the PLMN is or is not allowed to operate based on the request from AMF.

-	Support determination of a UE location at a scheduled location time.

-	Determine whether to use user plane or control plane for positioning.

-	Support handling of 5GC-MT-LR, 5GC-NI-LR, 5GC-MO-LR and deferred 5GC-MT-LR for periodic or triggered location over a user plane connection between UE and LMF.

Editor's note:	LMF functionality of user plan connection handling will be updated according to the protocol stack decision made by CT WG1 and SA WG3 groups.

-	Support collection of positioning assistance data from AFs.

NOTE 2:	Country, area within a country, or an international area can be supported as different types of geographical area.

-	Support a request for user plane reporting from a UE to an LCS Client or AF for a periodic or triggered 5GC-MT-LR. Subsequently, support the transfer of cumulative event reports from the target UE via control plane back to the H-GMLC and LCS Client or AF. Also support any request for assistance data received in a cumulative event report.

# * * * * Second change * * * *

5.11	Positioning Assistance Data

Positioning assistance data can provide a known location to assist to improve positioning performance. Two types of terminal entity can provide positioning assistance data: PRU as defined in TS 33.305 [9] and GNSS reference receiver. Positioning assistance data including the identity of the terminal equipment that provides positioning assistance data and its known location information is provisioned by OAM and stored in NRF. LMF can retrieve positioning assistance data from NRF with an area information (e.g. TAI(s) or CGI(s)).

LMF can collect positioning assistance data by reusing Data Collection from AF as described in clause 5.20a of TS 23.501 [18] and Naf_EventExposure service described in clause 5.2.19.2 of TS 23.502 [19]. The collection may be performed on a periodic or demand basis.

After the collection, LMF may send the collected GNSS positioning assistance data to UEs via the procedures described in clause 6.14.1 (Broadcast of Assistance Data by an LMF) or clause 6.11.1 (UE Assisted and UE Based Positioning Procedure) during a positioning session. LMF may obtain the registration state of PRU from UDM and serving AMF, conduct positioning measurements according to the procedures defined in clause 6.11 and capture the measurements results for location correction of nearby target UE.

# * * * * Third change * * * *

6.15	Positioning Assistance Data Collection Procedure

The following procedure is used by LMF to collect positioning assistance data information from AFs. If the AFs is internal to 5GS, the NEF may be ignored in the procedure.

NOTE 1:	If the AF is external to 5GS, AF registers its available data to LMF via OAM configuration at NEF.

The AF collectable data information includes: AF identification, positioning assistance data type, available positioning assistance data to be collected (e.g. identification and its known location of positioning assistance entity).

Figure 6.15-1: Positioning assistance data collection procedure

1a-1c.	Reuse the steps 1a-1c in figure 6.2.2.3-1 of TS 23.288 [37] with following difference:

-	NEF or AF should map the known location of positioning assistance entityinto TA(s) and/or Cell(s) where the positioning assistance entity is located and update the NEF profile in NRF including the serving area information.

1d-1e.	Reuse the steps 1d-1e in figure 6.2.2.3-1 of TS 23.288 [37] with following difference:

-	When LMF needs to discovery the available data from AFs and the appropriated NEF(s) (if any) to collect positioning assistance data, LMF additionally provides an area information (e.g. TAI(s) and/or CGI(s)) in the Nnrf_NFDiscovery_Request_request service operation.

2-5.	Reuse the steps 2-5 in figure 6.2.2.3-1 of TS 23.288 [37].

6a.	If the positioning assistance entity is PRU, LMF gets the registeration state of the target PRU(s) from UDM and the serving AMF. If the target PRU is in registered state, the LMF can obtain the positioning measuremens (e.g. RSTD, RSRP, UE Rx-TS Time Difference measurements, etc.) from the PRU according to the procedures defined in clause 6.11.

6b.	If the positioning assistance entity is GNSS reference receiver, LMF can send the relevant GNSS positioning assistance data to UE via the procedures described in clauses 6.14 or 6.11.1 during a positioning session.

# * * * * End of changes * * * *