3GPP TSG-SA WG2 Meeting #154AH-E	S2-2300484

# Elbonia, January 16 – 20, 2023	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3916 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network exposure for Application Layer AI/ML Operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This contribution aims to specify the Network exposure for Application Layer AI/ML Operation, based on the conclusion agreed in the clause 8.3 of TR 23.700 – 80. Expecially, we need to use the clause 5.2.26.2.1 of TS 23.501 which is agreed as the normative work of UPEAS at the previous meeting, as a biasis. - UL/DL data rate exposure to AF based on AF request for data rate monitoring with solution without RAN impact. - The AF could subscribe AF Session/ PDU Session related information for the specific AF Session and reuse the existing notification and subscription mechanism. NOTE 2: UL/DL data rate exposure to AF may consider the conclusions of FS_UPEAS KI#2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding the new clause introducing Network exposure for Application Layer AI/ML Operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5GC cannot completely support the network exposure for Application Layer AI/ML Operation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.b (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.X.b Network exposure for Application Layer AI/ML Operation

The 5GS supports network information exposure to authorized 3rd party AF to assist the application layer AI/ML operations.

The AF may subscribe AF Session and/or PDU Session related information for the specific AF, and based on QoS monitoring request, the measurements information (e.g. UL/DL data rate per QoS flow or PDU session as described clause 5.2.26.2.1 of TS 23.502 [3]) may be exposured from UPF via NEF to AF.

* * * * End of Changes * * * *