SA WG2 Meeting #S2-154AH	S2-2300492

# Online, 16th Jan 2023 - 20th Jan 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3920 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of PDU Set Information identification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The conclusion for XR KI#4 has been agreed in TR 23700-60. UPF supports the function of PDU Set Information identification at User plane. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR adds PDU Set Information identification at UPF in 23.501 based on the TR conclusion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not able to support PDU Set handling for XRM service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.8.1, 5.8.2.11.3, 5.8.2.x, Annex Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of 1st Change * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

……

[x]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications", STD 64, July 2003.

[y]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)", March 2004.

[z]	IETF RFC 6184: "RTP Payload Format for H.264 Video", May 2011.

* * * Start of 2nd Change * * *

5.8.1	General

User Plane Function(s) handle the user plane path of PDU Sessions. 3GPP specifications support deployments with a single UPF or multiple UPFs for a given PDU Session. UPF selection is performed by SMF. The details of UPF selection is described in clause 6.3.3. The number of UPFs supported for a PDU Session is unrestricted.

For an IPv4 type PDU Session or an IPv6 type PDU Session without multi-homing or an IPv4v6 type PDU Session, when multiple PDU Session Anchors are used (due to UL CL being inserted), only one IPv4 address and/or IPv6 prefix is allocated for the PDU Session. For an IPv6 multi-homed PDU Session there are multiple IPv6 prefixes allocated for the PDU Session as described in clause 5.6.4.3.

If the SMF had requested the UPF to proxy ARP or IPv6 Neighbour Solicitation for an Ethernet DNN, the UPF should respond to the ARP or IPv6 Neighbour Solicitation Request, itself.

Deployments with one single UPF used to serve a PDU Session do not apply to the Home Routed case and may not apply to the cases described in clause 5.6.4.

Deployments where a UPF is controlled either by a single SMF or multiple SMFs (for different PDU Sessions) are supported.

UPF traffic detection capabilities may be used by the SMF in order to control at least following features of the UPF:

-	Traffic detection (e.g. classifying traffic of IP type, Ethernet type, or unstructured type)

-	Traffic reporting (e.g. allowing SMF support for charging).

-	QoS enforcement (The corresponding requirements are defined in clause 5.7).

-	Traffic routing (e.g. as defined in clause 5.6.4. for UL CL or IPv6 multi-homing).

-	PDU Set Information identification (e.g. identify PDU Set Sequence Number, End PDU of the PDU Set, PDU SN within a PDU Set, PDU Set Size in bytes, PDU Set Importance).

* * * Start of 3rd Change * * *

5.8.2.11.3	Packet Detection Rule

The following table describes the Packet Detection Rule (PDR) containing information required to classify a packet arriving at the UPF. Every PDR is used to detect packets in a certain transmission direction, e.g. UL direction or DL direction.

Table 5.8.2.11.3-1: Attributes within Packet Detection Rule

| Attribute |  |  | Description | Comment |
| --- | --- | --- | --- | --- |
| N4 Session ID |  |  | Identifies the N4 session associated to this PDR. NOTE 5. |  |
| Rule ID |  |  | Unique identifier to identify this rule. |  |
| Precedence |  |  | Determines the order, in which the detection information of all rules is applied. |  |
| Packet | Source interface |  | Contains the values "access side", "core side", "SMF", "N6-LAN", "5G VN internal". | Combination of UE IP address (together with Network instance, if necessary), CN tunnel info, |
| Detection | UE IP address |  | One IPv4 address and/or one IPv6 prefix with prefix length (NOTE 3). | packet filter set, application identifier, Ethernet PDU Session |
| Information. NOTE 4. | Network instance (NOTE 1) |  | Identifies the Network instance associated with the incoming packet. | Information and QFI are used for traffic detection. Source interface identifies the |
|  | CN tunnel info |  | CN tunnel info on N3, N9 interfaces, i.e. F-TEID. | interface for incoming packets |
|  | Packet Filter Set |  | Details see clause 5.7.6. | where the PDR applies, e.g. from access side (i.e. up-link), |
|  | Application identifier |  |  | from core side (i.e. down-link), |
|  | QoS Flow ID |  | Contains the value of 5QI or non-standardized QFI. | from SMF, from N6-LAN (i.e. the |
|  | Ethernet PDU Session Information |  | Refers to all the (DL) Ethernet packets matching an Ethernet PDU session, as further described in clause 5.6.10.2 and in TS 29.244 [65]. | DN), or from "5G VN internal" (i.e. local switch). |
|  | Framed Route Information |  | Refers to Framed Routes defined in clause 5.6.14. | Details like all the combination possibilities on N3, N9 interfaces are left for stage 3 decision. |
| Packet replication and detection carry on information | Packet replication skip information NOTE 7 |  | Contains UE address indication or N19/N6 indication. If the packet matches the packet replication skip information, i.e. source address of the packet is the UE address or the packet has been received on the interface in the packet replication skip information, the UP function neither creates a copy of the packet nor applies the corresponding processing (i.e. FAR, QER, URR). Otherwise the UPF performs a copy and applies the corresponding processing (i.e. FAR, QER, URR). |  |
| NOTE 6 | Carry on indication |  | Instructs the UP function to continue the packet detection process, i.e. lookup of the other PDRs. |  |
| Outer header removal |  |  | Instructs the UP function to remove one or more outer header(s) (e.g. IP+UDP+GTP, IP + possibly UDP, VLAN tag), from the incoming packet. | Any extension header shall be stored for this packet. |
| Forwarding Action Rule ID (NOTE 2) |  |  | The Forwarding Action Rule ID identifies a forwarding action that has to be applied. |  |
| Multi-Access Rule ID (NOTE 2) |  |  | The Multi-Access Rule ID identifies an action to be applied for handling forwarding for a MA PDU Session. |  |
| List of Usage Reporting Rule ID(s) |  |  | Every Usage Reporting Rule ID identifies a measurement action that has to be applied. |  |
| List of QoS Enforcement Rule ID(s) |  |  | Every QoS Enforcement Rule ID identifies a QoS enforcement action that has to be applied. |  |
| PDU Set instruction and assistance Information |  | PDU Set instruction | Instructs the UP function to identify the PDU Set Information |  |
|  |  | Assistance information | Indicates Service Protocol e.g. RTP/SRTP header type to be used for PDU Set Information identification. |  |
| NOTE 1: Needed e.g. if: - UPF supports multiple DNN with overlapping IP addresses; - UPF is connected to other UPF or AN node in different IP domains. - UPF "local switch", N6-based forwarding and N19 forwarding is used for different 5G LAN groups. NOTE 2: Either a FAR ID or a MAR ID is included, not both. NOTE 3: The SMF may provide an indication asking the UPF to allocate one IPv4 address and/or IPv6 prefix. When asking to provide an IPv6 Prefix the SMF provides also an IPv6 prefix length. NOTE 4: When in the architecture defined in clause 5.34, a PDR is sent over N16a from SMF to I-SMF, the Packet Detection Information may indicate that CN tunnel info is to be locally determined. This is further defined in clause 5.34.6. NOTE 5: In the architecture defined in clause 5.34, the rules exchanged between I-SMF and SMF are not associated with a N4 Session ID but are associated with a N16a association. NOTE 6: Needed in the case of support for broadcast/multicast traffic forwarding using packet replication with SMF-provided PDRs and FARs as described in clause 5.8.2.13.3.2. NOTE 7: Needed in the case of packet replication with SMF-provided PDRs and FARs as described in clause 5.8.2.13.3.2, to prevent UPF from sending the broadcast/multicast packets back to the source UE or source N19/N6. |  |  |  |  |

* * * Start of 4th Change * * *

5.8.2.x	PDU Set Information Identification

5.8.2.x.1	General

This clause describes the process at the UPF to identify the PDU Set Information, e.g. PDU Set Sequence Number, End PDU of the PDU Set, PDU SN within a PDU Set, PDU Set Size in bytes, PDU Set Importance. UPF marks the PDU Set Information in the encapsulation header on the N3 (or N9) reference point for the PDU Set.

The SMF is responsible for instructing the UPF to identify the PDU Set Information, and may provide the Service Protocol as assistant information for the PDU Set Information identification. The PDU Set instruction and assistant information from SMF are included in a PDR for the corresponding user data traffic that matches the Packet Detection Information as defined in clause 5.8.2.11.3.

5.8.2.x.2	PDU Set Information

For the user data traffic that matches the Packet Detection Information of a PDR with PDU Set instruction, UPF identifies the following PDU Set information of each PDU Set:

-	PDU Set Sequence Number: identify the Sequence Number of a PDU Set within a QoS flow.

-	End PDU of the PDU Set: identify the last packet sending from AS of a PDU Set.

-	PDU SN within a PDU Set: identify the Sequence Number of a packet sending from AS of a PDU Set.

-	PDU Set Size in bytes, if provided by application.

Editor note:	The PDU Set Size is pending SA WG4 progress on SA WG4 5G_RTP WI.

-	PDU Set Importance: identify the importance of a PDU Set within a QoS flow.

5.8.2.x.3	PDU Set Information identification on UPF

The UPF supports to identify the PDU Set Information based on information carried in the RTP/SRTP header and payload as described in Annex Y.

The UPF may also identify the PDU Set Information based on its implementation.

* * * Start of 5th Change * * *

Annex Y (normative):
PDU Set Information identification on UPF based on RTP/SRTP

The UP function with the capability of PDU Set Information identification supports to get information from RTP/SRTP header and payload of the receiving DL packets, and can derive the PDU Set Information based on the information of RTP/SRTP header and payload. This annex provides description about how UPF derives the PDU Set Information based on different types of RTP/SRTP header and payload.

Y.1	PDU Set Information identification based on RTP Fixed Header

For the DL packets with RTP Fixed Header as definded in IETF RFC 3550 [x], but the payload header is not readable by UPF, e.g. the payload header is encrypted as defineded in IETF RFC 3711 [y]. UPF UPF derives the PDU Set Information as following:

1)	PDU Set Sequence Number

UPF allocates a PDU Set Sequence Number for the packets of the same PDU Set, i.e. the packets with the same timestamp field in the RTP Fixed Header. The values of timestamp field and sequence number field of the RTP Fixed Header is also considered to decide the value of PDU Set Sequence Number.

2)	End PDU of the PDU Set

UPF decides a packet is the End PDU of the PDU Set based on the marker field of the RTP Fixed Header.

3)	PDU SN within a PDU Set

UPF uses the sequence number field of the RTP Fixed Header as the PDU SN within a PDU Set.

Y.2	PDU Set Information identification based on RTP Fixed Header and RTP payload header

This clause is used for the DL packets with RTP Fixed Header as definded in IETF RFC 3550 [x], and the payload header is readable by UPF.

For the packets with the payload header as defineded in IETF RFC 6184 [z], and the payload type is set to Fragmentation Units. UPF derives the PDU Set Information as following:

1)	PDU Set Sequence Number

UPF allocates a PDU Set Sequence Number for the packets of the same PDU Set, i.e. the packets with the same timestamp field in the RTP Fixed Header and from the packet with S field of the RTP payload header to the packet with E field of the RTP payload header. The values of timestamp field and sequence number field of the RTP Fixed Header is also considered to decide the value of PDU Set Sequence Number.

2)	End PDU of the PDU Set

UPF decides a packet is the End PDU of the PDU Set based on the E field of the RTP payload header.

3)	PDU SN within a PDU Set

UPF uses the sequence number field of the RTP Fixed Header as the PDU SN within a PDU Set.

4)	PDU Set Importance

UPF decides the PDU Set Importance based on the NRI field of the RTP payload Header.

* * * End of Changes * * *