3GPP TSG-SA2 Meeting #154-AH-e	S2-2300495

# Online, , 16th Jan 2023 - 20th Jan 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0173 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update for 5G ProSe UE-to-UE Relay Communication with integrated Discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1)The term of “5G ProSe Discovery integrated into PC5 unicast link establishment” is not correct, because the PC5 unicast link modification procedure may also be used. 2) The relationship of clause 6.7.3.2 and 6.7.3.3 is not clear. 3) For clause 6.7.3.2, after the first hop Security established, relay UE receives end-to-end QoS from UE-1, thus relay UE decides the first hop QoS and second hop QoS after step7. 4) For the communication via L2 U2U relay, end-to-end connection between the peer UEs is not mentioned in the procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1)change the term to “5G ProSe UE-to-UE Relay Communication with integrated Discovery” 2)clarify clause 6.7.3.2 is the procedure for L3 U2U relay, clause 6.7.3.3 is the procedure for L2 U2U relay. 3) For clause 6.7.3.2, add link modification procedure at the second hop PC5 interface after relay UE receives end-to-end QoS from UE-1 at step7. 3)Add a new figure in clause 6.7.3.3 for L2 U2U relay, the the step of end-to-end connection between the peer UEs is in this procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The term of “5G ProSe Discovery integrated into PC5 unicast link establishment” is misleading. QoS control is not supported for L3 U2U relay, communication via L2 U2U relay is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.5, 6.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change* * * *

5.2.5	5G ProSe UE-to-UE Relay Discovery

For 5G ProSe UE-to-UE Relay Discovery, both Model A and Model B discovery are supported:

-	Model A uses a single discovery protocol message (Announcement).

-	Model B uses two discovery protocol messages (Solicitation and Response).

The procedures for 5G ProSe UE-to-UE Relay Discovery with Model A/Model B are defined in clause 6.3.2.4.

5G ProSe UE-to-UE Relay Communication with integrated Discoveryis also supported, i.e. without performing 5G ProSe UE-to-UE Relay Discovery with Model A/Model B.

# * * * * Second change* * * *

6.7.3	5G ProSe UE-to-UE Relay Communication with integrated Discovery

6.7.3.1	General

5G ProSe Communication via 5G ProSe UE-to-UE Relay with integrated Discovery is supported.

For 5G ProSe UE-to-UE Relay Communication with integrated Discovery, when a UE allows a UE-to-UE relay to be involved in the Direct Communication Request to the other UE, the UE indicates it by including a relay_indication in the broadcasted Direct Communication Request message.

When a UE-to-UE relay receives a Direct Communication Request including a relay_indication, it decides whether to forward the message according to e.g. Relay Service Code if there is any, Application ID, operator policy per Relay Service Code, signal strength, and local policy.

6.7.3.2	Procedure for 5G ProSe UE-to-UE Relay Communication with integrated Discovery via Layer-3 UE-to-UE Relay

Figure 6.7.3.2-1: 5G ProSe UE-to-UE Relay Communication with integrated Discovery via Layer-3 UE-to-UE Relay

0.	UEs are authorized and provisioned with parameters to use the service provided by the UE-to-UE relays. UE-to-UE relays are authorized and provisioned with parameters to provide service of relaying traffic among UEs.

1.	UE-1 wants to establish a unicast communication with UE-2 and broadcasts a Direct Communication Request. The parameters included in the Direct Communication Request message are described in clause 6.4.3.7.

The relay_indication in the Direct Communication Request is used to indicate whether UE-to-UE relay can forward the Direct Communication Request message or not. It is also used to limit the number of hops of UE-to-UE relay by removing relay_indication in the Direct Communication Request message from the UE-to-UE Relay.

NOTE:	The data type of relay_indication can be determined in Stage 3.

2.	When receiving Direct Communication Request with relay_indication from UE-1, Relay (here, relay-1 and relay-2) may decide to participate in the procedure and broadcast a Direct Communication Request message in its proximity without relay_indication. The parameters included in the Direct Communication Request message are described in clause 6.4.3.7.

The Direct Communication Request message uses relay's Layer-2 ID as the source Layer-2 ID.

3.	When UE-2 receives a Direct Communication Request from one or multiple UE-to-UE relays, UE-2 select a UE-to-UE Relay which UE-2 will respond. UE-2 may select UE-to-UE relay according to the signal strength, local policy, operator policy per Relay Service Code if any.

4.	security establishment happens between UE-2 and the selected 5G ProSe UE-to-UE Relay (here relay-1), if needed.

Editor's note:	It is FFS how to perform PC5 link modification between UE-2 and relay-1.

5.	UE-2 replies Direct Communication Accept message to relay-1. The parameters included in the Direct Communication Accept message are described in clause 6.4.3.7.

6. For IP traffic, IPv6 prefix or IPv4 address is allocated for the target 5G ProSe Layer-3 End UE as defined in clause 5.5.1.3.

7.	security establishment happens between UE-1 and relay-1, if needed.

Editor's note:	It is FFS how to perform PC5 link modification between UE-1 and relay-1.

8. 	For 5G ProSe UE-to-UE Relay Communication via Layer-3 UE-to-UE Relay, after receiving QoS Info of the end-to-end QoS from UE-1, Relay-1 provides the QoS info of the second hop QoS to UE-2 with Link Modification Request message.

9. 	For 5G ProSe UE-to-UE Relay Communication via Layer-3 UE-to-UE Relay, UE-2 responds with a Link Modification Accept message.

10.	Relay-1 responds with Direct Communication Accept to the UE-1. The parameters included in the Direct Communication Accept message are described in clause 6.4.3.7.

11. For IP traffic, IPv6 prefix or IPv4 address is allocated for the source 5G ProSe Layer-3 End UE as defined in clause 5.5.1.3.

Editor's note:	It is FFS how to support non-IP case.

6.7.3.3	Procedure for 5G ProSe UE-to-UE Relay Communication with integrated Discovery via Layer-2 UE-to-UE Relay

Figure 6.7.3.3-1: 5G ProSe UE-to-UE Relay Communication with integrated Discovery via Layer-2 UE-to-UE Relay

Step 0- step 5 are same as Step 0- step 5 of Figure 6.7.3.2-1.	Step 6 is same as Step 7 of Figure 6.7.3.2-1. 	Step 7 is same as Step 10 of Figure 6.7.3.2-1. The parameters included in the messages are described in clause 6.4.3.7.

8. 	 For 5G ProSe UE-to-UE Relay Communication via Layer-2 UE-to-UE Relay, UE-1 establishes an end-to-end connection for unicast mode communication with UE-2 as described in clause 6.4.3.7.

Editor's note:	Any additional update of procedure via Layer-2 UE-to-UE Relay, e.g. according to RAN's decision, will be included here.

# * * * * End of changes * * * *