3GPP TSG-SA WG2 Meeting #154 AH	S2-2300499

Emeeting, Jan 16 - 20, 2022

Title:	[Draft] Reply LS on PDU Set Handling

Response to:	(R2-2213351) LS on PDU Set Handling

Release:	Rel-18

Work Item:	XRM

Source:	[Tencent, will be] SA2

To:	RAN2

Cc:	SA4, RAN3

Contact Person:

Name:	Lei Yixue

Tel. Number:	86-10-17710237811

E-mail Address:	yixuelei@tencent.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

SA2 thanks RAN2 for the LS on PDU set handling.

Regarding to the question in R2-2213351 on PDU set and QoS flow mapping, SA2 would like to reply RAN2 questions as follows:

Q1: In order to decide how PDU sets could be mapped in radio protocols, RAN2 is wondering if different PDU sets could have different characteristics (for instance importance, PSER, and/or PSDB) and if so, which characteristics can be different and with which granularity (e.g. QoS flow, individual PDU Sets…)

SA2 answer: Yes, different PDU sets could have different characteristics.  PDU Set Importance, PSER and PSDB can be different for different PDU sets in the QoS flow.

Q2: RAN2 would also like to know whether different types of PDU set can be mapped to the same QoS flow and if so whether RAN should have the ability to treat those differently over the air interface.

SA2 answer: SA2 has agreed that different types of PDU set can be mapped into the same QoS flow in 5GC and meanwhile, in GTP-u header, PDU set importance can be included.  SA2 thinks that whether RAN treat these PDU sets differently or not is up to RAN to decide and the PDU set importance information in GTP-u header can be used.  How to use this information is also up to RAN to decide.

Q3: If RAN should have such an ability, RAN2 would like to know based on what information signalled to the gNB this would be based on.

SA2 answer:  SA2 thinks that given 5GC has in GTP-u header provided the PDU set importance, RAN2/3 can discuss and decide whether/how gNB uses such information to perform different treatment over the air-interface.

2. Actions:

To CT3

ACTION: 	SA2 kindly asks RAN2 to take the above into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA WG2#155	Feb 20 – 24, 2023		E-meeting